3GPP TSG-RAN WG2 Meeting #131	R2-2505240

Bangalore, India  Aug 25th – 29th , 2025

Agenda item:	8.1.3

Source:	LG Electronics Inc.

Title:	Remaining issue on NW side Data Collection

Document for:	Discussion and Decision

1. Introduction

During the RAN2 #130 meeting, RAN2 reached the following agreements.

Agreements on 1 bit indication

- (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced.  In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data).   We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it.   Notify RAN3

Agreements on assistant information

- Buffer threshold to trigger data availability indication should be set based on specific size, e.g., KB instead of percentage

- UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

- No additional signaling from the UE is required when the low power issue is resolved

- No additional signaling from the UE is required when the buffer full issue is resolved

Agreements on reporting contents

1	Data is collected on per data logging configuration basis and UE indicates data logging configuration ID.    An indication of the “gap” is needed.  “Gap” is time interval larger than the configured logging periodicity.    FFS if timestamp and relative time stamp for each group is needed per “group”.

2   The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

This contribution aims to specify the configuration and procedure involved in data collection for BM use case.

2. Discussion

Remaining open issues

Open issue RRC-19: Reporting assistance information related to logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated? This issue is related to the RAN2#129bis agreement: Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined. The issue is captured as an editor’s notes in the running CR, in clause 5.3.5.9, 5.7.4.3, 6.2.2, and 6.3.4. Based on the comments in the RRC running CR, some companies have a different view on the solution: - Each of the three indications (full buffer being reached, buffer threshold being reached, and low power) is configured separately; or - Full buffer reporting is enabled by default whenever LoggedDataCollectionAssistanceConfig is configured (no additional configuration bit is needed) and only buffer threshold being reached and low power indications are configured each separately. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

Full buffer trigger can be used to ensure that logged data is retrieved when the UE buffer is full, preventing data loss, whereas threshold-based trigger can allow the network to retrieve data opportunistically, taking into account the UE’s state and network conditions. Therefore, these triggers can be independently configurable, allowing the network to enable only the triggers that are relevant to the specific operational scenario. Also, having these triggers independently configurable enables future extensions without structural changes when new triggering conditions are introduced. Therefore, it is proposed that the network can configure separate bits for low power, buffer full, and buffer threshold reached indications in otherConfig. Accordingly, the UE can also include separate bits for low power, buffer full, and buffer threshold reached indication in UAI.

Proposal 1. [RRC19] The otherConfig includes separate bits for low power, buffer full, and buffer threshold reached indications.

Open issue RRC-24: Where to include the logging configuration from NW to UE

Issue description: It is FFS whether the logging configuration is included in the CSI framework (whether in CSI-ReportConfig or directly under CSI-MeasConfig) or at L3. This issue was discussed in RAN2#129bis and the following outcome was captured: Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.. The issue is captured as an editor’s note in the running CR, in clause 6.3.2. Configuration details of events for event-based logging configuration, e.g. whether we reuse configuration in reportConfig, can be included. Proposed resolution: This open issue is addressed in the email discussion [POST130][031][AI PHY] NW side data collection (Ericsson/ZTE).Companies can also discuss relevant aspects in their contributions.

Logging condition/event

Regarding the method of logging condition configurations, there are two options:

- O1. logging configuration is introduced as a new list of configurations under CSI-MeasConfig, based on TP1

- O2. logging configuration is introduced in a new L3 measurement framework, at the same level as MeasConfig and CellGroupConfig, based on TP2

When considering future extensions for mobility-related use cases, introducing a new event for event-based logging may be a suitable approach. That is, if logging event is well defined, these events could be extensible to other use cases.

Observation 1. [RRC24] When considering future extensions for mobility-related use cases, introducing a new event for event-based logging may be a suitable approach.

Based on the understanding of most companies, defining the condition would typically require L3 signal quality threshold along with TTT and hysteresis, and leveraging existing measurement-reporting event is likely the simplest and most natural approach. Some companies, however, have raised questions about the CU configuring logging-related events in a CU-DU split architecture.

If CU-DU split scenario is considered, the CU has a centralized view of the network, including multiple DUs, and can optimize event parameters to align with network wide strategies. Also, since event triggering is directly tied to UEInformationResponse message at the RRC layer, CU control over these configurations can ensure consistency in signaling procedures and policy enforcement across the network.

Observation 2. [RRC24] When leveraging conventional measurement-reporting events, if CU-DU split scenario is considered, the CU has a centralized view of the network, including multiple DUs, and can optimize event parameters to align with network wide strategies. Also, since event triggering is directly tied to UE information Response message at the RRC layer, CU control over these configurations can ensure consistency in signaling procedures and policy enforcement across the network.

Given that the new event should not trigger an actual report, new event should be separate from the conventional event. Accordingly, a separate report type for logging purposes needs to be considered to not include report related configuration (e.g., report interval, report amount, etc).

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18,

eventTriggeredLogging-r19                   EventTriggerConfigLogging-r19

}

}

EventTriggerConfigLogging-r19 ::=                      SEQUENCE {

logEventId                                     CHOICE {

logEventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

logEeventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

...

},

...

}

Proposal 2. [RRC24] To introduce New reportType (CHO-like) for logging purposes, based on the conventional measurement-reporting configuration (MR config).

Logged data management upon receiving a new logging configuration

In legacy logged MDT operation, the network sets logging configuration within a separate RRC message, i.e., LoggedMeasurementConfiguration message. Upon receiving the LoggedMeasurementConfiguration message, the UE shall discard the logged measurement configuration as well as the logged measurement information.

In contrast, logging configuration for AI/ML data collection is included in the RRCReconfiguration message and typically uses a list-based structure (ToAddModList and ToReleaseList). Since delta configuration is possible, it is inefficient to discard all logged data unconditionally.

Therefore, for AI/ML-oriented data collection, a differentiated approach can be applied to ensure efficient data retention and avoid unnecessary loss of valid training data. For example, the UE can retain any logged data that is not associated with configuration IDs in either the ToAddModList or the ToReleaseList.

Observation 3. [RRC24] Since the logging configuration is set with list-based structure (i.e., ToAddModList and ToReleaseList, delta configuration is possible. A differentiated approach can be applied to ensure efficient data retention and avoid unnecessary loss of valid training data.

Proposal 3. [RRC24] Upon receiving a new configuration, the UE discards only the logged data that corresponds to the configuration IDs included in the ToAddModList or ToReleaseList

- e.g., If a configuration ID in the ToAddModList matches an existing logged data entry, the UE discards the corresponding logged data

- e.g., If a configuration ID in the ToReleaseList matches an existing logged data entry, the UE discards the associated logged data

- e.g., The UE retains any logged data that is not associated with configuration IDs in either the ToAddModList or the ToReleaseList

Logging resumption when memory issue is resolved

When the buffer issue is resolved, e.g., by memory being cleared or space becoming available, it is unclear whether the UE can autonomously resume logging according to the retained configuration, at least for periodic CSI-RS logging.

Similar behavior is observed in legacy MDT operations, where logging can be paused due to IDC issues. In such cases, once the issue is resolved, the UE autonomously resumes logging without requiring explicit instruction or reconfiguration from the network.

When the buffer limit issue is resolved, UE resumes logging at least for periodic CSI-RS according to the retained configuration. This approach avoids unnecessary signaling while maintaining continuity in data collection.

Proposal 4.   [RRC24] When the buffer limit issue is resolved, UE resumes logging at least for periodic CSI-RS according to the retained configuration.

Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Issue description: RAN2#127 made the following agreement regarding NW-side data collection: 1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.. The second FFS on dynamic activation/deactivation has not yet been addressed, as also pointed out in the stage-2 running CR draft. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

Open issue RRC-30: Semi-persistent resources for data collection

Issue description: RAN2 excluded usage of aperiodic CSI resource for data collection, but it is still unclear whether semi-persistent resources are needed for this. Proposed resolution: It is suggested that companies provide contributions to resolve this issue later.

In general, periodic CSI-RS may be cell-specific, while UE-dedicated CSI-RS, particularly in scenarios requiring beam refinement or per-UE adaptation, may rely on semi-persistent configurations. In such cases, supporting logging on semi-persistent CSI-RS may be necessary to enable data collection for training purposes.

Although the activation mechanism differs (i.e., semi-persistent CSI-RS requires explicit activation), once activated, the reporting behavior follows a periodic pattern. As a result, the logging or reporting burden imposed by semi-persistent CSI-RS is not significantly different from that of periodic CSI-RS when used for training data collection. Therefore, semi-persistent CSI-RS resource should be considered for inclusion in data collection frameworks.

Observation 4. [RRC-25/30] In general, periodic CSI-RS may be cell-specific, while UE-dedicated CSI-RS, particularly in scenarios requiring beam refinement or per-UE adaptation, may rely on semi-persistent configurations. In such cases, supporting logging on semi-persistent CSI-RS may be necessary to enable data collection for training purposes

Proposal 5. [RRC-25/30] For measurement/logging for semi-persistent CSI-RS, dynamic activation/deactivation mechanism is supported

Open issue RRC-27: How to set logging periodicity

Issue description: Regardless of the chosen logging framework, further discussion is needed on how to set the logging periodicity for each logging RS. To provide flexibility in the logging timing, two options could be considered for logging periodicity: (i) aligning with the RS transmission periodicity, or (ii) introducing an optional logging interval setting. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In legacy MDT operations, when periodic reporting is configured, the UE logs data at specific logging intervals. Similarly, for CSI-RS-based logging, it is desirable to support a configurable logging interval that allows the network to control how frequently the UE logs the RS measurement results. If the logging interval is not configurable and logging is instead performed at every RS transmission, this could lead to unnecessarily frequent data collection, increasing UE processing burden and memory usage without proportional benefit.

There are two approaches regarding logging interval:

- 1. Logging aligned with the CSI-RS transmission periodicity: This approach has minimal specification impact but limited flexibility

- 2. Logging based on an optionally configured logging interval, which may be longer than the RS transmission period: This introduces additional specification impact but offers greater flexibility, especially in scenarios with stable signal conditions

During the email discussion, opinions vary among companies on these two approaches, and each has its own trade-offs. Therefore, it is feasible to support both approaches. That is, if no logging interval is explicitly configured, the default behavior can fall back to RS transmission periodicity.

Observation 5. [RRC27] There are two approaches regarding logging interval, and each has its own trade-offs. It is feasible to support both approaches.

- O1. Logging aligned with the CSI-RS transmission periodicity: This approach has minimal specification impact but limited flexibility

- O2. Logging based on an optionally configured logging interval, which may be longer than the RS transmission period: This introduces additional specification impact but offers greater flexibility, especially in scenarios with stable signal conditions

Proposal 6. [RRC27] The UE performs logging at the logging interval defined in the logging configuration. If no logging interval is configured, the UE shall perform logging based on the RS resource transmission periodicity. (i.e., Both approaches are supported depending on the network configuration)

Open issue RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc

Issue description: Handling of configuration for UE assistance information, to report data availability and low power state (LoggedDataCollectionAssistanceConfig in RRC running CR) during RRCReestablishment and in transition to RRC_INACTIVE state needs to be specified. In the rapporteur’s view, handling the UAI configuration for reporting data availability and low power state should follow the way that the logging configurations are handled, which depends on the solution that RAN2 will adopt based on the email discussion [POST130][031][AI PHY] NW side data collection (Ericsson/ZTE). Proposed resolution: Adopt the proposal below. - (RRC-28) RAN2 to discuss whether logging configurations for NW-side data collection are discarded upon IDLE/INACTIVE/RLF. - (RRC-28) RAN2 to discuss whether handling the configuration for UE assistance information to report data availability and low power state upon IDLE/INACTIVE/RLF follows the UE behaviour for handling the logging configurations.

Currently, the UE releases the configuration for UE assistance information in the following scenarios:

- When the UE initiates an RRC Release procedure (transitioning to idle or inactive state), it releases the configuration related to assistance information.

- When the UE initiates an RRC Re-establishment procedure, it releases the configuration related to assistance information.

On the other hand, based on the previous agreement, logged data is discarded for transitions to IDLE/INACTIVE and RLF. If the UE follows the legacy operation of releasing the configuration for UE assistance information, there could be a timing difference in when the logged data and the configuration for UE assistance information are discarded.

However, since RRC Re-establishment procedure is eventually performed after RLF, the UE will ultimately release the related configuration, ensuring that UAI is not generated. Therefore, there is no difference in UE behavior in the end. Thus, maintaining the process of discarding the configuration (to report data availability and low power state (LoggedDataCollectionAssistanceConfig in RRC running CR)) for UE assistance information in the same manner as other configurations, as per legacy operations, is no harm.

Observation 6. If the UE releases the configuration for UE assistance information according to the conventional method, there could be a timing difference in when the logged data and the configuration for UE assistance information are discarded in RLF case. However, since RRC Re-establishment procedure is eventually performed after RLF, the UE will ultimately release the related configuration, ensuring that UAI is not generated. Therefore, there is no difference in UE behavior in the end.

Proposal 7. [RRC28] To discard the configuration for UE assistance information to report data availability and low power state in the same manner as other UE assistance information configurations, as per legacy operations (i.e., no spec change)

Open issue RRC-31: The release of logged AIML data in UE’s buffer

Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered: - Power off or deregistration - 48 hrs after the release of logged measurement configuration - Explicit indication from the serving gNB. Proposed resolution: It is suggested that companies provide contributions to resolve this issue later.

If the power-off or deregistration happened or the valid time for the logged data has expired, the UE should consider the logged data as no longer valid and clear it for the next logging operation. The conditions for releasing logged data can be similar with the existing logged MDT operation. UE behavior under the new configuration is described separately in P3.

Proposal 8. [RRC31] UE releases logged data under the following conditions:

- upon power off

- upon deregistration

- upon a certain time later, FFS length of timer (like 48-hour timer in logged MDT)

- upon successful delivery of the UEInformationResponse message (w/ logged data)

Open issue RRC-33: Whether separate user consent for gNB centric training is needed

Issue description: It can be discussed whether a separate user consent is required for gNB-centric training, as it may differ from the conventional MDT user consent. To address the potential impact on gNB and OAM regarding data collection, it may be necessary to send an LS to RAN3 and SA5, including agreements related to logging. Proposed resolution: It is suggested that companies provide contributions to resolve the issue later

Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

Issue description: RAN2 agreed that target gNB can fetch data collected in the source gNB after HO. We need to decide how this data is forwarded to OAM or source gNB, e.g. via inter-node RRC message or in some other way. RAN3 involvement may be needed. Proposed resolution: It is suggested that companies provide contributions to resolve the issue later.

As identified by multiple companies, further coordination with RAN3, SA3 and SA5 may be required to ensure proper alignment of procedures and responsibilities in NW-sided logged data collection for AI/ML purposes.

For example, open issue RRC-33 discusses whether separate user consent is needed for training, distinguishing it from conventional MDT user consent. This has potential impact on gNB and OAM regarding user consent for data collection. Similarly, open issue RRC-36 highlights the need to define how data is forwarded to the OAM or source gNB after a handover. Depending on the chosen solution, this could introduce additional signaling between gNBs or between the gNB and OAM.

Therefore, the agreements reached in RAN2 regarding logged data collection, especially those that may impact UE consent handling, inter-gNB communication, or OAM integration, should be communicated to RAN3, SA3 and SA5 via an LS. This will help ensure that any procedural or architectural impacts are properly considered in the relevant WGs.

Proposal 9. [RRC 33/36] Send LS to RAN3, SA3 and SA5 to inform them of the agreements made regarding NW-sided logged data collection for AI/ML training

Open issue RRC-39: The naming of IEs related to NW-side data collection

Issue description: The naming of IEs related to NW-side data collection can be revisited. We will have more CSI-based and other use cases, each with different measurement logs and we should minimize the risk of having to replicate the NW-side DC procedures and ASN.1 for every new use case. One solution direction is to make the naming of procedures and ASN.1 related to NW-side DC generic and enclose use case specific IEs / data logs within so that our framework can support future use cases. An alternative is to name the IEs based on the use case, which would help to write the procedural text in a simpler way, while being clear. This issue refers to the following field/IE and variable names: - The name of the variable to store logged data. This can be use case specific (i.e. VarCSI-LogMeasReport as proposed by the rapporteur in the draft RRC running CR) and a new variable would be added in the future for e.g. the AIML mobility use case, or it can be changed to have a more generic name and include lists of logged data for different use cases. Note that defining a variable per use case still allows the UE to use a shared memory size for all use cases. - The name of the field to request logged data from the UE and the name of the field where the UE provides the report with logged measurements can be specific per use case (as proposed by the rapporteur in the draft RRC running CR, i.e. csi-LogMeasReportReq in UEInformationRequest and csi-LogMeasReport in UEInformationResponse, respectively) or can be more generic. The rapporteur sees the benefit in having use case specific names, since this way the NW can select which logged data to retrieve from the UE and the UE can select that logged data to report it. - The name of the field to indicate to the NW that the UE has logged data during handover can be use case specific (i.e. csi-LogMeasAvailable in RRCReconfigurationComplete, as proposed by the rapporteur in the draft RRC running CR) or can be more generic. As in the point above, in the rapporteur’s view it is beneficial to have use case specific naming, so that the NW can distinguish which kind of logged data the UE has. Proposed resolution: Adopt the proposals below. - (RRC-39) The UE stores logged data for BM in a variable specific to L1 CSI related measurements. - (RRC-39) RAN2 to discuss whether the name of the fields/IEs to request logged data from the UE and to report logged data or the availability of logged data to the NW are use case specific.

The naming of fields/IEs for logged data can be based on the contents of the logged data rather than the specific use case. It can allow to reuse logged data across multiple use cases and minimizing duplication in ASN.1 definitions. Therefore, it is beneficial to define field names and IE based on the content of the logged data rather than the specific use case. For instance, if a data type is defined based on L1- or L3-related measurements, the same data type can be reused across multiple use cases. This method promotes efficiency and consistency in data logging and usage.

Observation 7. [RRC39] The naming of fields/IEs for logged data can be based on the contents of the logged data rather than the specific use case. It can allow to reuse logged data across multiple use cases and minimizing duplication in ASN.1 definitions.

Proposal 10. [RRC39] To define field names and IE based on the content of the logged data rather than the specific use case

3. Conclusion

Open issue RRC-19: Reporting assistance information related to logged measurements

Proposal 1. [RRC19] The otherConfig includes separate bits for low power, buffer full, and buffer threshold reached indications.

Open issue RRC-24: Where to include the logging configuration from NW to UE

Logging condition/event

Observation 1. [RRC24] When considering future extensions for mobility-related use cases, introducing a new event for event-based logging may be a suitable approach.

Observation 2. [RRC24] When leveraging conventional measurement-reporting events, if CU-DU split scenario is considered, the CU has a centralized view of the network, including multiple DUs, and can optimize event parameters to align with network wide strategies. Also, since event triggering is directly tied to UE information Response message at the RRC layer, CU control over these configurations can ensure consistency in signaling procedures and policy enforcement across the network.

Proposal 2. [RRC24] To introduce New reportType (CHO-like) for logging purposes, based on the conventional measurement-reporting configuration (MR config)

Logged data management upon receiving a new logging configuration

Observation 3. [RRC24] Since the logging configuration is set with list-based structure (i.e., ToAddModList and ToReleaseList, delta configuration is possible. A differentiated approach can be applied to ensure efficient data retention and avoid unnecessary loss of valid training data.

Proposal 3. [RRC24] Upon receiving a new configuration, the UE discards only the logged data that corresponds to the configuration IDs included in the ToAddModList or ToReleaseList

- e.g., If a configuration ID in the ToAddModList matches an existing logged data entry, the UE discards the corresponding logged data

- e.g., If a configuration ID in the ToReleaseList matches an existing logged data entry, the UE discards the associated logged data

- e.g., The UE retains any logged data that is not associated with configuration IDs in either the ToAddModList or the ToReleaseList

Logging resumption when memory issue is resolved

Proposal 4.   [RRC24] When the buffer limit issue is resolved, UE resumes logging at least for periodic CSI-RS according to the retained configuration.

Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Open issue RRC-30: Semi-persistent resources for data collection

Observation 4. [RRC-25/30] In general, periodic CSI-RS may be cell-specific, while UE-dedicated CSI-RS, particularly in scenarios requiring beam refinement or per-UE adaptation, may rely on semi-persistent configurations. In such cases, supporting logging on semi-persistent CSI-RS may be necessary to enable data collection for training purposes

Proposal 5. [RRC-25/30] For measurement/logging for semi-persistent CSI-RS, dynamic activation/deactivation mechanism is supported

Open issue RRC-27: How to set logging periodicity

Observation 5. [RRC27] There are two approaches regarding logging interval, and each has its own trade-offs. It is feasible to support both approaches.

- O1. Logging aligned with the CSI-RS transmission periodicity: This approach has minimal specification impact but limited flexibility

- O2. Logging based on an optionally configured logging interval, which may be longer than the RS transmission period: This introduces additional specification impact but offers greater flexibility, especially in scenarios with stable signal conditions

Proposal 6. [RRC27] The UE performs logging at the logging interval defined in the logging configuration. If no logging interval is configured, the UE shall perform logging based on the RS resource transmission periodicity. (i.e., Both approaches are supported depending on the network configuration)

Open issue RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc

Observation 6. If the UE releases the configuration for UE assistance information according to the conventional method, there could be a timing difference in when the logged data and the configuration for UE assistance information are discarded in RLF case. However, since RRC Re-establishment procedure is eventually performed after RLF, the UE will ultimately release the related configuration, ensuring that UAI is not generated. Therefore, there is no difference in UE behavior in the end.

Proposal 7. [RRC28] To discard the configuration for UE assistance information to report data availability and low power state in the same manner as other UE assistance information configurations, as per legacy operations (i.e., no spec change)

Open issue RRC-31: The release of logged AIML data in UE’s buffer

Proposal 8. [RRC31] UE releases logged data under the following conditions:

- upon power off

- upon deregistration

- upon a certain time later, FFS length of timer (like 48-hour timer in logged MDT)

- upon successful delivery of the UEInformationResponse message (w/ logged data)

Open issue RRC-33: Whether separate user consent for gNB centric training is needed

Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

Proposal 9. [RRC 33/36] Send LS to RAN3, SA3 and SA5 to inform them of the agreements made regarding NW-sided logged data collection for AI/ML training

Open issue RRC-39: The naming of IEs related to NW-side data collection

Observation 7. [RRC39] The naming of fields/IEs for logged data can be based on the contents of the logged data rather than the specific use case. It can allow to reuse logged data across multiple use cases and minimizing duplication in ASN.1 definitions.

Proposal 10. [RRC39] To define field names and IE based on the content of the logged data rather than the specific use case

4. Reference

[1] R2-250xxxx - RRC open issues AIML PHY

[2] 38214-j00