3GPP TSG-RAN WG2 Meeting #131	R2-2505242

Bengaluru, India, Aug. 25th– 29th, 2025

Agenda Item:	8.12.3

Source:	OPPO

Title:	Clarification on the coexistence between LTM and UL-only TRP

Document for:	Discussion, Decision

# Introduction

According to the discussion in the RAN2#130 meeting, RAN2 discussed whether the LTM can be configured together with the UL-only TRP. In this contribution, we provide our analysis on the coexistence issue between LTM and UL-only TRP.

# Discussion

## Coexistence issue of LTM and UL-only TRP

TS 38.321: Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

Email discussion summary for the MAC running CR: Proposal 8 (12/12): sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

According to the email discussion summary of the MAC running CR for the Rel-19 MIMO, all companies consider that if pathloss offset is not configured by RRC, sDCI mTRP 2TA can be configured for the LTM candidate configuration, and no MAC specification change is needed.

WID of Rel-19 MIMO: - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

According to the Rel-19 MIMO WID as quoted above, the asymmetric DL sTRP/UL mTRP is only for intra-band intra-DU case, this means that during the inter-gNB or inter-DU handover, the target node is not able to provide the configuration for the asymmetric DL sTRP/UL mTRP, including the pathloss offset configuration.

For the LTM procedure, we think that for the intra-DU LTM, the pathloss offset configuration can still be provided via the candidate LTM configuration to improve the uplink throughput. As the pathloss offset can be changed when the UE moves around, the UE should be updated with the latest pathloss offset if the UL only TRP is to be used during the cell switching procedure triggered by LTM Cell Switch Command MAC CE. However, as the LTM procedure is used to reduce the handover latency, and the UL-only TRP is used to improve the uplink throughput, the gNB can still indicate the TCI-state of the UL/DL symmetric TRP via LTM Cell Switch Command MAC CE, and the latest pathloss offset for the uplink-only TRP can be indicated after the completion of the handover. There is no need to add the pathloss offset indication in the LTM Cell Switch Command MAC CE, and there is also no need to add a restriction to prohibit the pathloss offset configuration in the LTM candidate configuration.

Observation 1: The gNB by implementation can indicate the TCI-state of the UL/DL symmetric TRP via LTM Cell Switch Command MAC CE without adding the pathloss offset, and the latest pathloss offset can be provided after LTM completion.

Proposal 1: The pathloss offset can be included in the LTM candidate configuration, without adding the pathloss offset in the LTM Cell Switch Command MAC CE.

Running CR of 38.331:

TS 38.331:

Regarding the early RACH procedure for the LTM, RAN2 needs to clarify whether the preamble for the early RACH procedural can be transmitted to the UL-only TRP. According to the running CR of TS 38.331, the configuration (i.e. pathlossOffsetPRACH-DCI-1-0 and prachAssociationDCI-1-0) of allowing the PDCCH-order indicating pathloss offset is included in the BWP-DownlinkDedicated, and the EarlyUL-SyncConfig has no configuration of BWP-DownlinkDedicated. This means that the PDCCH-order indicating pathloss offset cannot be used for the early RACH procedure. Regarding whether the preamble transmission (without extra pathloss offset) can be sent to the UL-only TRP, as the PRACH resource for early RACH is associated to SSB which is not restricted to a specific TRP type and there is no DL feedback (i.e. no Msg2) needed for the early RACH preamble, no extra specification change is needed to clarify whether the preamble transmission (without extra pathloss offset) can be sent to the UL-only TRP. If the target node by implementation is able to receive the preamble of early RACH via the UL-only TRP, it will still be able to indicate the TA in the LTM Cell Switch Command MAC CE for the UL-only TRP.

Proposal 2: The PDCCH-order indicating pathloss offset cannot be used for the early RACH procedure. No extra specification change is needed.

Proposal 3: It is up to gNB implementation whether the UL-only TRP is used to receive the preamble of early RACH. No extra clarification in the specification is needed on whether the preamble transmission (without extra pathloss offset) can be sent to the UL-only TRP.

Proposal 4: It is up to gNB implementation whether to indicate a TA for the TCI-state of the UL-only TRP. No extra clarification in the specification is needed

# Conclusion

According to the analysis give above, we have the following observations and proposals:

Observation 1: The gNB by implementation can indicate the TCI-state of the UL/DL symmetric TRP via LTM Cell Switch Command MAC CE without adding the pathloss offset, and the latest pathloss offset can be provided after LTM completion.

Proposal 1: The pathloss offset can be included in the LTM candidate configuration, without adding the pathloss offset in the LTM Cell Switch Command MAC CE.

Proposal 2: The PDCCH-order indicating pathloss offset cannot be used for the early RACH procedure. No extra specification change is needed.

Proposal 3: It is up to gNB implementation whether the UL-only TRP is used to receive the preamble of early RACH. No extra clarification in the specification is needed on whether the preamble transmission (without extra pathloss offset) can be sent to the UL-only TRP.

Proposal 4: It is up to gNB implementation whether to indicate a TA for the TCI-state of the UL-only TRP. No extra clarification in the specification is needed

# Reference

[1] 3GPP TS38.321, “Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification”.