3GPP TSG-RAN WG2 Meeting #131	R2-2505244

Bengaluru, India, Aug. 25th– 29th, 2025

Agenda Item:	8.11.2

Source:	OPPO

Title:	Clarification on the CFRA for SBFD RO

Document for:	Discussion, Decision

# Introduction

According to the discussion in the RAN#130 meeting, RAN2 made the following agreements on the handling of SBFD RO when the UE switches between CFRA and CBRA:

RAN2#130 meeting agreement [1]: For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion. SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

In this contribution, we discuss the remaining issues regarding the CFRA configuration and procedure for SBFO.

# Discussion

## RO indexing for CSI-RS based CFRA

RAN2#130 meeting agreement [1]: For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion.

Text proposal for the running CR of TS 38.331 [2]: ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered (as specified in TS 38.213 [13], clause 8), first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

TS 38.213: # 8 Random access procedure … ## 8.1 Random access preamble …

According to the latest RAN1 specification of TS 38.213 as quoted above, the legacy RO and the SBFD RO use separate RO indexing (i.e. separate RO indexing pools for legacy RO and SBFD RO). This means that when a RO index is indicated in ra-OccasionList, both the legacy RO and the SBFD RO with the same RO index is used for the CFRA.

According to the RAN2#130 meeting discussion [1], RAN2 agreed to support CSI-RS based CFRA using SBFD RO. According to the email discussion summary of the running CR of TS 38.331 for SBFD WID, the CFRA configuration includes the configuration of the CFRA RO(s) (i.e. ra-OccasionList), which is indicated by the indexing of the RO(s) provided by the “prach-ConfigurationIndex and msg1-FDM in RACH-ConfigGeneric”. In the field description of ra-OccasionList, each RO is numbered only in increasing order of frequency resources and time resources in all RO(s) (i.e. common RO indexing pool for legacy RO and CBFD RO).

To avoid the misunderstandings on the RO indexing, we think that we can add the TS 38.213 as the reference for the RO index as described in the field description of ra-OccasionList.

Observation 1: In the RRC running CR, each RO is numbered only in increasing order of frequency resources and time resources in all RO(s), i.e. common RO indexing pool for legacy RO and CBFD RO. However, TS 38.213 is saying that separate RO indexing pools for legacy RO and SBFD RO is used.

Proposal 1: In the field description of ra-OccasionList, TS 38.213 is added as the reference for the RO indexing.

## RO (type) configuration for CFRA and CBRA

RAN2#130 meeting agreement: - SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

Running CR of TS 38.331: CFRA RO configuration: occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP. ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots. CFRA RO type indication:

Running CR of TS 38.331: CBRA RO configuration: CBRA RO type indication:

According to the running RRC CR as quoted above, when the CBRA RO configuration (i.e. rach-ConfigCommn) includes the SBFD RO, the CFRA RO configuration (i.e. occasions) may/may not include the SBFD RO. When the CBRA configuration indicates that SBFD RO type is prioritized (i.e. sbfd-RO-Type set to “sbfd”), the CFRA configuration may/may not indicate that SBFD RO type is prioritized (i.e. ra-OccasionType is configured).

Running CR of TS 38.321: RA procedure - Select UL carrier between SUL and NUL - BWP operation during RACH - Select RO type between SBFD RO and non-SBFD RO - select the set of Random Access resources applicable to the current Random Access procedure - Select the RA type between 2-step RA and 4-step RA - Initialization of variables specific to Random Access type - Random Access Resource selection - Selection between CFRA and CBRA - Random Access Preamble transmission - Random Access Response reception - Contention Resolution

According to the running CR of TS 38.321, the RO type selection is performed before the selection between CFRA and CBRA, and the selection between CFRA and CBRA is performed for each preamble (re)transmission.

Observation 2: According to the running RRC CR, when the CBRA RO configuration includes the SBFD RO, the CFRA RO configuration may not include the SBFD RO.

Observation 3: According to the running RRC CR, when the CBRA configuration indicates that SBFD RO type is prioritized, the CFRA configuration may not indicate that SBFD RO type is prioritized.

The misalignment on the RO configuration and the RO type indication in CFRA and CBRA could lead to the following issues:

- Issue 1: The SBFD RO type is selected in the green-highlighted step, but the SBFD RO resource is not available in CFRA or CBRA in the red-highlighted step.

- Issue 2: The SBFD RO resource is available in the initial preamble transmission of CBRA (or CFRA), but the SBFD RO resource is not available in the preamble retransmission of CFRA (or CBRA), when CBRA (or CFRA) fallbacks to CFRA (or CBRA).

Considering that RAN2 only has one meeting left for the Rel-19 SBFD WID, to avoid the above issues, we think that the gNB by implementation should ensure the aligned configuration for CFRA and CBRA, regarding the RO type indication and RO resource configuration.

Proposal 2: The gNB by implementation should ensure that when the RO configuration of the CBRA includes SBFD RO, the RO configuration of the CFRA also includes SBFD RO.

Proposal 3: The gNB by implementation should ensure that RO type indication for both CFRA and CBRA is the same.

# Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: In the RRC running CR, each RO is numbered only in increasing order of frequency resources and time resources in all RO(s), i.e. common RO indexing pool for legacy RO and CBFD RO. However, TS 38.213 is saying that separate RO indexing pools for legacy RO and SBFD RO is used.

Observation 2: According to the running RRC CR, when the CBRA RO configuration includes the SBFD RO, the CFRA RO configuration may not include the SBFD RO.

Observation 3: According to the running RRC CR, when the CBRA configuration indicates that SBFD RO type is prioritized, the CFRA configuration may not indicate that SBFD RO type is prioritized.

Proposal 1: In the field description of ra-OccasionList, TS 38.213 is added as the reference for the RO indexing.

Proposal 2: The gNB by implementation should ensure that when the RO configuration of the CBRA includes SBFD RO, the RO configuration of the CFRA also includes SBFD RO.

Proposal 3: The gNB by implementation should ensure that RO type indication for both CFRA and CBRA is the same.

# Reference

[1] RAN2#130 meeting minutes

[2] [Post130][216][SBFD] Running CR for 38.331 (Huawei)

[3] 3GPP TS38.213, “NR; Physical layer procedures for control”