3GPP TSG-RAN WG2 Meeting #131				               R2-2505253

Bengaluru, India, August 25th – 29th , 2025

Agenda item:	8.5.3

Source:	Samsung

Title:	On-demand SIB1

Document for:	Discussion and decision

# Introduction

RAN2 discussed procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode in RAN2#130 and following agreements were made:

- Access information of NES cell

  - Not to include access related information of NES into WUS configuration

- Broadcasting status for OD-SIB1 request

  - Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed.

    - RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- Handling of RRC connected UE

  - The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

  - If there is no common search space configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- UL carrier for OD-SIB1

  - Revisit it next meeting

- Parameters for OD-SIB1

  - Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

In this contribution we discuss remaining details/open issues of on-demand SIB1.

# Discussion

## 2.1. UL carrier for SIB1 request

Cell which supports on demand SIB1 can be configured with both SUL and NUL. So SIB1 request should be supported for both SUL and NUL in a cell supporting SUL. UE should select the UL carrier before transmitting SIB1 request. SIB1 request is based on random access and in legacy random access procedure, UL carrier selection is performed at the initiation of random access procedure. So, both NUL and SUL can easily be supported for SIB1 request without any impact to SIB1 request procedure i.e. no special handling or modification to SIB1 request procedure is needed.

Observation 1: No special handling or modification to SIB1 request procedure is needed for supporting SUL.

Additionally, in legacy, SI request for on demand OSI is supported for both SUL and NUL. There is no reason to not support it for OD-SIB1.

Observation 2: SI request for on demand OSI is supported for both SUL and NUL

In previous meetings some companies suggested that there is no need to support OD-SIB1 for SUL as OD-SIB1 is supported only in small cells. This is not correct. During the scenario discussions, there was no such discussion and agreement.

Observation 3: There was no discussion/agreement on supporting OD-SIB1 only for small cells.

In various releases, several WIs (msg1 repetition, msg3 repetitions, rach partition, early TA, etc) enhanced random access procedure. For all these WIs, general procedure was discussed/agreed and applied for both SUL and NUL.

Observation 4: Enhancements in earlier releases involving random access procedure are applied for both SUL and NUL.

Not supporting SUL for SIB1 request has more specification effort:

- a) there is impact on random access procedure as exception (i.e. no UL carrier selection for the case random access procedure is initiated for SIB1 request) need to be defined in MAC spec.

- b) we need to specify that cell supporting SUL does not provide SIB1 on demand. The reason is that if cell supporting SUL provides SIB1 on demand and UE is not in NUL coverage, UE will not be able to acquire SIB1 in the absence of SIB1 request configuration for SUL.

Proposal 1a: SIB 1 request is supported for both SUL and NUL.

Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration.

2.2 SIB1 request upon connection release

UE is in RRC_CONNECTED in Cell A (PCell). Cell A supports SIB X and UE acquires and maintains a valid version of the Cell A’s SIBX. Upon releasing the RRC connection, UE performs cell selection and selects Cell B.

If Cell A’s SIB X include SIB1 request configuration of Cell B, the issue is whether the UE can use that configuration for requesting SIB1 of Cell B. In our view similar to RLF case, UE should be allowed to use SIB1 request configuration from SIB X of Cell A for SIB1 request to selected Cell B after connection release.

Proposal 2: SIB1 request configuration of a cell (say cell B) from SIB X of PCell can be used for SIB1 request to a cell B selected upon connection release.

2.3 MIB re-acquisition for OD-SIB1

Scenario

- During cell reselection to a cell say Cell X, UE acquires MIB.

  - Kssb based on information in MIB indicates ‘No SIB1’.

- UE checks Kssb. As Kssb indicates ‘No SIB1’ and UE has OD-SIB1 configuration of Cell X, UE performs OD-SIB1 request procedure using OD-SIB1 configuration of Cell X, acquires SIB 1 and camps to Cell X.

- Later, while the UE is camped in Cell X, UE needs to reacquire SIB1 (e.g. to acquire some feature specific SIB)

  - In this case, for the Kssb based checking to decide whether to initiate OD-SIB1 request procedure or not, UE needs to know the latest Kssb. Its possible that Kssb currently being transmitted in MIB is not the same Kssb as acquired in step 1, as network may have switched the mode to periodic broadcast.

There are two options:

Option 1: UE relies on latest acquired Kssb i.e. UE does not reacquire MIB autonomously.  This option works under the assumption that when Kssb is changed, network sends SI change notification and UE acquires MIB upon receiving the SI change notification as in legacy.

Option 2: UE reacquires MIB, if the last MIB was not acquired in current MIB period (i.e. 80ms if SSB periodicity is <=80ms and 160ms if SSB periodicity is >80ms). In this option network may or may not send SI change notification when Kssb is changed.

Option 2 is not good for UE. It also does not follow the current principle wherein in change in SI triggers SI change notification. Note that MIB is also SI and referred as minimum or essential SI in spec.

Proposal 3: For the Kssb based checking to decide whether to initiate OD-SIB1 request procedure or not in the camped cell:

Option 1 (Preferred): UE relies on Kssb in last acquired MIB i.e. UE does not reacquire MIB autonomously.  This option works under the assumption that when Kssb is changed, network sends SI change notification and UE acquires MIB upon receiving the SI change notification as in legacy.

Option 2: UE reacquires MIB, if the last MIB was not acquired in current MIB period (i.e. 80ms if SSB periodicity is <=80ms and 160ms if SSB periodicity is >80ms). In this option network may or may not send SI change notification when Kssb is changed.

2.4 SIBxx handling

##### In RAN2#130, following TP was agreed for RRC CR

“

##### 5.2.2.4.2x             Actions upon reception of SIBxx

Upon receiving SIBxx, the UE shall:

1> if the UE has (re)selected to a cell providing OD-SIB1, the UE stores the configuration for SIB1 request for this cell and considers it valid while camping in this cell:

“

This TP is misleading. It seems to suggest that UE continues to use the SIB1 request of Cell X acquired from Cell A after the cell reselection. The issue (Samsung 002) was raised during the RRC CR review email discussion.

What we have previously agreed is that

- SIB1 request of Cell X acquired from Cell A is used during reselection to Cell X.

- After reselection to Cell X, for SIB1 request to Cell X, UE needs to use SIB1 request configuration from SIBxx valid in Cell X. SIBxx of Cell A may or may not be valid in Cell X depending on area id/value tag of SIBxx of Cell A and SIBxx of CellX

So, the text should be revised as suggested by OPPO during the RRC CR review email discussion as follows

Proposal 4: Revised the text as follows in RRC CR

##### 5.2.2.4.2x             Actions upon reception of SIBxx

1> if the UE has (re)selected to a cell providing OD-SIB1, the UE stores the configuration for SIB1 request for this cell and considers it valid while camping in this cell:

- store the SIBxx;

- apply the SIB1 request configuration for acquiring OD-SIB1 of this cell;

- apply the SIB1 request configuration of another cell in this stored SIBxx for acquiring OD-SIB during reselection to that cell, and after reselection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1;

# Conclusion

Observation 1: No special handling or modification to SIB1 request procedure is needed for supporting SUL.

Observation 2: SI request for on demand OSI is supported for both SUL and NUL

Observation 3: There was no discussion/agreement on supporting OD-SIB1 only for small cells.

Observation 4: Enhancements in earlier releases involving random access procedure are applied for both SUL and NUL.

Proposal 1a: SIB 1 request is supported for both SUL and NUL.

Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration.

Proposal 2: SIB1 request configuration of a cell (say cell B) from SIB X of PCell can be used for SIB1 request to a cell B selected upon connection release.

Proposal 3: For the Kssb based checking to decide whether to initiate OD-SIB1 request procedure or not in the camped cell:

Option 1 (Preferred): UE relies on Kssb in last acquired MIB i.e. UE does not reacquire MIB autonomously.  This option works under the assumption that when Kssb is changed, network sends SI change notification and UE acquires MIB upon receiving the SI change notification as in legacy.

Option 2: UE reacquires MIB, if the last MIB was not acquired in current MIB period (i.e. 80ms if SSB periodicity is <=80ms and 160ms if SSB periodicity is >80ms). In this option network may or may not send SI change notification when Kssb is changed.

Proposal 4: Revised the text as follows in RRC CR

##### 5.2.2.4.2x             Actions upon reception of SIBxx

1> if the UE has (re)selected to a cell providing OD-SIB1, the UE stores the configuration for SIB1 request for this cell and considers it valid while camping in this cell:

- store the SIBxx;

- apply the SIB1 request configuration for acquiring OD-SIB1 of this cell;

- apply the SIB1 request configuration of another cell in this stored SIBxx for acquiring OD-SIB during reselection to that cell, and after reselection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1;

# References