3GPP TSG RAN WG2 Meeting #131                                                            R2-2505263

# Bangalore, India, August 25th - 29th, 2025

Agenda item:	8.2.2

Source:	Ofinno

Title:	Remaining issues on A-IoT paging

ocument for:	Discussion and decision

# Introduction

This document discusses the potential optionality of the Transaction ID field in A-IoT Paging message for CFA and how to capture the termination of an ongoing procedure due to a subsequent/applicable A-IoT Paging message considering the A-IoT MAC running CR reviewed as part of the email discussion [POST130][027][AIoT] [1][2].

# Discussion

## Transaction ID in A-IoT Paging message

Email discussion [POST130][027][AIoT] captures the following proposal “Proposal 3 (Issue 1-5 Transaction ID in CFRA paging): RAN2 confirms the pervious RAN2 baseline that transaction ID is not included in paging message for CFRA. (23/25)”. We understand that there is a large support not to introduce the transaction ID in paging msg for CFA for Rel-19 considering that parallel services are not allowed.

Rel-19 CFA addresses a very simple scenario in which a reader initiates A-IoT CFA procedure for a single device and waits to conclude it (or terminate it) before initiating any other A-IoT procedure (with this or other devices). For future release, it is possible that the reader may perform more than one A-IoT procedures (e.g. inventory and command) to one or multiple devices that are associated with the one or more A-IoT service (e.g., A-IoT inventory service or A-IoT command service) by sending one or more A-IoT Paging message(s). In our understanding, it will be up to the reader’s implementation to decide how many A-IoT Paging messages the reader will send to perform inventory of all the devices associated with one specific A-IoT service. Reader’s decision may vary depending on multiple factors, for example, depending on the availability of the radio resource used for paging and random access, as well as a success or failure rate of inventorying the devices, etc. Moreover, the A-IoT Paging message could be followed by one or more subsequent R2D messages (e.g., subsequent A-IoT Paging messages) associated with the same A-IoT service/transaction. Therefore, it may be unclear how this association (i.e., messages that belong to the same A-IoT service/transaction) could be distinguished. For this, the transaction ID in the A-IoT Paging message could be used and helpful to identify each specific A-IoT procedure, as well as, for scenarios in which different readers send A-IoT Paging messages to the same device (or group of devices) potentially associated to different A-IoT services. For this, the A-IoT Paging message and one or more subsequent R2D messages (e.g., subsequent A-IoT Paging messages) that are associated with the same A-IoT service should include the same transaction ID.

Summarizing, even though there is a large preference not to include transaction ID in A-IoT Paging message for Rel-19, we think this could improve future compatibility and also provide flexibility in the network side. We propose for companies to consider defining the transaction ID in A-IoT paging message at least as optional. For example, a reserved bit of the A-IoT paging message for CFA in the current A-IoT MAC running CR can be used for indicating whether the this A-IoT paging message includes the transaction ID or not. Moreover, the A-IoT paging message size might not be as critical concert to add this optionality after getting SA3 input that requested to include quiet large security related information in each A-IoT Paging message.

For CFA, including transaction ID in A-IoT Paging message could provide future compatibility/flexibility while also allow the network to allow, when required, the usage of CBRA and CFA together (e.g., CFA proc. could be interleaved within a A-IoT CBRA ongoing procedure).

A-IoT Paging message has larger size after including security related information requested by SA3 and might not be as critical to define transaction ID as an optional field in the A-IoT Paging message for CFA (to allow flexibility in future releases, network side and A-IoT deployments).

- To define transaction ID as an optional field in the A-IoT Paging message for CFA.

A-IoT MAC running CR captures that upon triggering the initiation of CFA, the device always releases any stored AS ID and transaction ID. This is a simple behavior as RAN2 agreed not to support parallel services, CFA may not use/include any transaction ID and a device always responds to a new A-IoT Paging message with a different transaction ID. However, if transaction ID were defined as optional in A-IoT Paging message (as captured in Proposal 1), it may be better and simple to also allow the usage of CBRA and CFA together (e.g., CFA proc. could be interleaved within a CBRA proc.), when required by the network. This could be enabled by only releasing and/or replacing the stored AS ID and transaction ID when different value is provided due to the initiation of CFA.

- If Proposal 1 is agreed, to discuss the handling of an stored Transaction ID when initiating CFA procedure: option (1) the device always release the stored AS ID and transaction ID upon initiation of CFA (with this behaviour being the same as in current A-IoT MAC running CR), or option (2) the device releases the stored AS ID and transaction ID only when different transaction ID is provided in A-IoT Paging message due to the initiation of CFA (with minimal update required in A-IoT MAC running CR).

## Termination of a procedure due to a subsequent and applicable A-IoT Paging message

A-IoT MAC running CR [1] captures whether a device is selected to initiate the random procedure (as shown in the green highlighted references below in §5.2 of [1]), as well as, when the current procedure associated with the stored Transaction ID is considered as failed (as shown in the blue highlighted references below in §5.5 of [1]). However, there is no explicit description of when the ongoing AS/A-IoT procedure is terminated due to a reception of a subsequent and applicable A-IoT Paging message.

A-IoT MAC running CR captures in the procedural text when a device is selected to initiate random procedure, as well as, when the current procedure associated with the stored Transaction ID is considered failed; however, there is no explicit description when the ongoing AS/A-IoT procedure is terminated (due to a reception of a subsequent and applicable A-IoT Paging message).

Our suggestion is to add the conditions that determines the termination or completion of the ongoing procedure when a A-IoT Paging message indicates CFA or CBRA as shown in the yellow highlighted references below in §5.2. I.e.,, previous (and ongoing) AS/A-IoT procedure, associated with the stored AS ID and the stored Transaction ID (if available), ends when procedure upon reception of a A-IoT Paging message indicating CFA or CBRA with the corresponding conditions are met:

- Condition (1) If the Access Type field in the A-IoT Paging message indicates CFA and AS ID is stored.

- Condition (2) If the Access Type field in the A-IoT Paging message indicates CBRA and the value of the Transaction ID field in the A-IoT Paging message is different from the stored Transaction ID.

- Upon receiving the A-IoT Paging message, to capture explicitly in the procedural text when a previous (and ongoing) AS/A-IoT procedure, associated with the stored AS ID and the stored Transaction ID (if available) ends (with an example TP shown within this document). For this, based on A-IoT MAC running CR, the conditions when a previous (and ongoing) procedure is terminated are: condition (1) if the Access Type field in the A-IoT Paging message indicates CFA and AS ID is stored, or condition (2) if the Access Type field in the A-IoT Paging message indicates CBRA and the value of the Transaction ID field in the A-IoT Paging message is different from the stored Transaction ID.

## 5.2 A-IoT paging The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader may include the Paging ID field to select a specific device or a group of devices, or may not include Paging ID field to select all devices. The device monitors for the A-IoT Paging message, and determines whether the device is selected to initiate the access procedure. Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall: 1> if the Access Type field in the A-IoT Paging message indicates CFA and AS ID is stored; or 1> if the Access Type field in the A-IoT Paging message indicates CBRA and the value of the Transaction ID field in the A-IoT Paging message is different from the stored Transaction ID: 2> consider that the previous procedure, associated with the stored AS ID and the stored Transaction ID, if available, ends; 1> if the Access Type field in the A-IoT Paging message indicates CBRA: 2> if the device has no stored Transaction ID; or 2> if the value of the Transaction ID field is different from the stored Transaction ID; or 2> if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5: 3> release the stored AS ID if any; 3> store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID; 3> if the Paging ID Presence Indication field indicates Paging ID field is absent: 4> consider the device is selected and indicate to the upper layers; 3> else: 4> forward the value of the Paging ID field to the upper layers; 4> if the upper layers indicate that the Paging ID is matched: 5> consider the device is selected; 3> if the device is selected: 4> initiate Contention-Based Random Access procedure as specified in clause 5.3.1; 1> else (i.e., the Access Type field in the A-IoT Paging message indicates CFA): 2> release the stored AS ID if any; 2> release the stored Transaction ID, if any 2> forward the value of the Paging ID field to the upper layers; 2> if the upper layers indicate that this Paging ID is matched: 3> consider the device is selected; 3> initiate Contention-Free Access procedure as specified in clause 5.3.2. ## 5.5 Failure detection Once the device transmitted the first D2R Upper Layer Data Transfer message after CBRA procedure, the A-IoT MAC entity shall monitor for NACK Feedback message until the device receives a A-IoT Paging message or R2D Upper Layer Data Transfer message addressed to the device (i.e., the device does not process NACK Feedback message after that). Upon reception of NACK Feedback message, the A-IoT MAC entity shall: 1> for each AS ID entry in the NACK Feedback message: 2> if the value indicated by the AS ID field is identical to the stored AS ID: 3> consider that the current procedure associated with the stored Transaction ID failed, upon which this procedure of processing NACK Feedback message ends. Upon reception of A-IoT Paging message as specified in clause 5.2, the A-IoT MAC entity shall: 1> if CBRA procedure has not been considered as successful as specified in clause 5.3.1.3: 2> consider that the current procedure associated with the stored Transaction ID failed.

# Conclusion

The observations captured are the following:

Observation 1.	For CFA, including transaction ID in A-IoT Paging message could provide future compatibility/flexibility while also allow the network to allow, when required, the usage of CBRA and CFA together (e.g., CFA proc. could be interleaved within a A-IoT CBRA ongoing procedure).

Observation 2.	A-IoT Paging message has larger size after including security related information requested by SA3 and might not be as critical to define transaction ID as an optional field in the A-IoT Paging message for CFA (to allow flexibility in future releases, network side and A-IoT deployments).

Observation 3.	A-IoT MAC running CR captures in the procedural text when a device is selected to initiate random procedure, as well as, when the current procedure associated with the stored Transaction ID is considered failed; however, there is no explicit description when the ongoing AS/A-IoT procedure is terminated (due to a reception of a subsequent and applicable A-IoT Paging message).

The proposals captured are the following:

Proposal 1.	To define transaction ID as an optional field in the A-IoT Paging message for CFA.

Proposal 2.	If Proposal 1 is agreed, to discuss the handling of an stored Transaction ID when initiating CFA procedure: option (1) the device always release the stored AS ID and transaction ID upon initiation of CFA (with this behaviour being the same as in current A-IoT MAC running CR), or option (2) the device releases the stored AS ID and transaction ID only when different transaction ID is provided in A-IoT Paging message due to the initiation of CFA (with minimal update required in A-IoT MAC running CR).

Proposal 3.	Upon receiving the A-IoT Paging message, to capture explicitly in the procedural text when a previous (and ongoing) AS/A-IoT procedure, associated with the stored AS ID and the stored Transaction ID (if available) ends (with an example TP shown within this document). For this, based on A-IoT MAC running CR, the conditions when a previous (and ongoing) procedure is terminated are: condition (1) if the Access Type field in the A-IoT Paging message indicates CFA and AS ID is stored, or condition (2) if the Access Type field in the A-IoT Paging message indicates CBRA and the value of the Transaction ID field in the A-IoT Paging message is different from the stored Transaction ID.

# Reference

- [POST130][027][AIoT] MAC Running CR (Huawei), MAC running CR, TS 38.391 V0.0.3 (2025-08).

- [POST130][027][AIoT] MAC Running CR (Huawei), Remaining A-IoT MAC open issues.