3GPP TSG-RAN WG2 Meeting #131											R2-2505272

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.7.4

Source: 	Sharp

Title:  	Leftover issues on scheduling enhancements

Document for: 	Discussion

# Introduction

In RAN2#130 meeting, RAN2 made the following agreements:

[MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization. When PSI-based discarding is activated for a DRB, priority adjustment is applied only if there is at least one high importance packet in the buffer has a remaining time smaller than the threshold. (MAC spec is already aligned with this agreement). (MAC-13) When pdu-SetDiscard is configured, PDU sets should be treated as a whole in the LCP procedure with adjusted priority. Confirm the working assumption “No Bj enhancement is introduced” as an agreement. Both PDCP and RLC consider Control PDU and/or retransmitted data into the shortest configured reporting threshold. The value of the remaining time field in the enhanced DSR MAC CE is set to 0, if there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold of the LCG. An understanding is that there will be no DSR with no data indication (i.e. indicating only volume of C-PDU and/or retransmissions for any LCG) (RRC-7) If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold. New LCID is used for R19 DSR (MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR. (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

This document discusses leftover open issues on scheduling enhancements for XR.

# Discussion

## (UE Capability-1) Rel-18 DSR and Rel-19 DSR

(UE capability-01): Whether a UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18

Rel-19 XR introduces a Multiple-Entry DSR format to support delay information with multiple reporting thresholds. The Rel-19 DSR format is a generalized format which covers all possible cases that Rel-18 DSR format can report. Also, the current CR captures the UE behaviour that if at least one LCG is configured with dsr-ReportingThresList, the Rel-19 DSR MAC CE is used. Hence, there is no case that a Rel-19 UE uses both Rel-19 MAC CE and Rel-18 MAC CE simultaneously. Thus, the Rel-18 DSR format does not need to be a pre-requisite of the Rel-19 DSR capability.

(UE Capability-1) UE can support Rel-19 DSR capability without Rel-18 DSR capability.

## (MAC-1) Rel-19 DSR Cancellation Considering Delay-Critical SDU

Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

For Rel-19 Multiple-Entry DSR procedure, a concept of delay-reporting data volume has been introduced whereas Rel-18 Single-Entry DSR procedure relies on delay-critical data. The issue here is whether a special handling is needed when Rel-18 remainingTimeThreshold is lower than the largest reporting threshold. In our understanding, Rel-19 DSR procedure is independent of Rel-18 threshold. DSR triggering and reporting can be performed based on only Rel-19 thresholds. Thus, we do not see the necessity of the coupling of the Rel-18 threshold and Rel-19 threshold in DSR procedure.

(MAC-1) DSR cancellation does not consider remainingTimeThreshold.

## (MAC-2) BSR Cancellation after DSR Transmission

Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

DSR procedure has been considered as an independent procedure to report the delay-critical data volume and timing information. Rel-18 DSR is also able to report the UE’s buffer status to be included in the BSR, but BSR cancellation in this case is not supported. We can keep the same principle in Rel-19. From the network perspective, frequent buffer status updates based on BSR and DSR can be useful for scheduling at the network.

(MAC-2) DSR procedure does not affect BSR cancellation.

## Joint Configurations of DSR and Additional Priority

- In RAN2#127bis, RAN2 agreed not to introduce any setting restrictions of the new remaining time threshold for additional priority with relation to DSR triggering threshold remainingTimeThreshold. But there was no discussion/conclusion on whether it is possible to independently configure additional priority and remainingTimeThreshold for DSR.

- In RAN2#129 meeting, the impact of additional priority on BSR was discussed, and it was not agreed because we can rely on DSR. That means that a common understanding is that additional priority is configured only if remainingTimeThreshold for DSR is configured. An expected technical issue is that without DSR configuration, gNB does not have any idea whether the UE has delay-critical data or whether additional priority is being used. It will probably give an impact to timely scheduling. We see that DSR should be a pre-requisite of additional priority.

Additional priority is configured only if remainingTimeThreshold for DSR is configured.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- (UE Capability-1) UE can support Rel-19 DSR capability without Rel-18 DSR capability.

- (MAC-1) DSR cancellation does not consider remainingTimeThreshold.

- (MAC-2) DSR procedure does not affect BSR cancellation.

- Additional priority is configured only if remainingTimeThreshold for DSR is configured.