3GPP TSG-RAN WG2 Meeting #131	R2-2505274

Bengaluru, India, 25th - 29th August, 2025

Agenda Item:	8.7.4

Source: 	OPPO

Title:  	Discussion on scheduling enhancements for XR

Document for:	Discussion and Decision

- Introduction

In Rel-19 XR WID [1] revised in RANP#105 meeting, the objectives of scheduling enhancements are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on LCP and DSR enhancements for scheduling.

- SRB impact for LCH priority adjustment

In RAN2#125bis meeting, the following agreement was made on SRB impact due to LCP enhancement, but no further conclusion on this.

RAN2 should consider potential impact on traffic from SRBs.

In our understanding, a proper network implementation would solve this issue easily. Networks can ensure the LCH of SRB always has higher LCH priority than the additional LCH priority used for the LCH with LCH priority-adjusted data.

- Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

- Open issues of DSR enhancements

[UE Cap-1] The pre-requisite of enhanced DSR

One open issue on UE capability is the pre-requisite for the capability of Rel-19 DSR, i.e., whether a UE supporting Rel-19 DSR shall also indicate support of delayStatusReport-r18 or not, as shown below.

Open issue 1: the pre-requisite of enhanced delay status report This was discussed in RAN2#129bis meeting (as shown below) and an FFS was captured: “FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.” - LGE, QCM thinks we do not need to couple R18 and R19 DSR. Vivo thinks reasonable UE will implement R18 if implements R19, but does not want to couple in specs. - Huawei does not think it is possible to support R19 without R18

In our understanding, the UE behavior for Rel-18 DSR is actually a special case of Rel-19 DSR. For example, for a LCG with Rel-19 DSR, if only one DSR-ReportingThreshold equal to remainingTimeThreshold-r18 is configured in dsr-ReportingThresList-r19, at the same time dsr-ReportNonDelayCriticalData-r19 is not enable, the UE behaviors for this LCG are fully same as Rel-18 DSR. Therefore, it is not possible for a UE only support Rel-19 DSR without Rel-18 DSR.

Although a proper UE implementation should support both Rel-18 and Rel-19 DSR together, but we still understand no need to couple them from UE capability signaling perspective in the specification.

- From UE implementation perspective, considering UE behaviour for Rel-18 DSR is actually a special case of Rel-19 DSR, it is not possible for a UE only support Rel-19 DSR without Rel-18 DSR.

- [UE Cap-1] From UE capability signalling perspective, no need to have the pre-requisite for the capability of Rel-19 DSR.

[MAC-1] Pending DSR cancellation

In the current MAC spec, the DSR cancellation mechanism is based on the delay-critical PDCP SDU, as shown below.

DSR cancellation: A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

During the email discussion on the MAC running CR, one open issue raised by companies is to discuss whether a pending DSR should be cancelled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported, as shown below.

[MAC-1] Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

Companies think this issue mainly comes from the mismatch, i.e.,

- for the generation of Rel-19 DSR MAC CE, we introduce new concept of delay-reporting PDCP SDUs,

- while the cancellation of pending Rel-19 DSR MAC CE is still based on legacy concept of delay-critical PDCP SDUs, i.e., no relying on the concept of delay-reporting PDCP SDUs.

If relying on the legacy pending DSR cancellation mechanism, in case that some configured thresholds in dsr-ReportingThresList are greater than remainingTimeThreshold, the pending DSR cancellation only depends on the data whose remaining time is below remainingTimeThreshold, i.e., it is both delay-critical PDCP SDU and delay-reporting PDCP SDU. If no this kind of data, even there is still the data whose remaining time is between remainingTimeThreshold and the largest configured reporting threshold, i.e., it is the delay-reporting PDCP SDUs but not delay-critical PDCP SDUs, the Rel-19 DSR MAC CE would be cancelled.

However, we understand that it is intentional for the current design of cancellation of Rel-19 DSR MAC CE, i.e., relying on the concept of delay-critical PDCP SDUs. For the delay-reporting PDCP SDUs which are not delay-critical PDCP SDUs, the report of delay status information is not so urgent for UE and NW side. The cancellation of pending DSR which only include the delay status information of this kind of data does not hurt anything. On the contrary, we see the benefits on signalling overhead reduction. And it still has chance to be reported in the future DSR, e.g., trigger a new DSR reporting when the remaining time of the data becomes below remainingTimeThreshold.

Besides, we understand it also provides flexibility for NW, considering that NW can fully control to allow/avoid this case based on the proper configuration of dsr-ReportingThresList and remainingTimeThreshold.

- For Rel-19 DSR, no issue is foreseen if relying on legacy pending DSR cancellation mechanism based on the concept of delay-critical PDCP SDUs.

- [MAC-1] RAN2 does not consider any enhancement on pending DSR cancellation for Rel-19 DSR.

[MAC-2] Pending BSR cancellation

During the email discussion on the MAC running CR, companies propose to cancel pending BSR based on DSR MAC CE. One open issue for MAC is captured as following.

[MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

Note that in Rel-18 DSR, the similar issue was also discussed and no any enhancement of pending BSR cancellation based on Rel-18 DSR was introduced. Considering the last meeting before Rel-19 feature freeze, to avoid triggering unnecessary discussion, we prefer not to support this.

- In Rel-18, the enhancement of pending BSR cancellation based on Rel-18 DSR is not supported.

- [MAC-2] RAN2 does not consider to cancel pending BSR based on Rel-19 DSR, as what we did for Rel-18 DSR.

[MAC-6] DSR cancellation in the DC case

In last meeting, the open issue on DSR cancellation in the DC case was discussed, and the following agreements were made.

(MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR. (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

During the email discussion on the MAC running CR, companies have concern on whether any spec change is needed to capture these agreements, and the issue is still open as shown below.

[MAC-6] Whether any spec change is needed to capture the agreement on DSR cancellation in the DC case.

From UE point of view, it is reasonable to cancel DSR if a MAC PDU contains all SDUs associated with the DSR is transmitted from MAC entity itself or other MAC entity.

Meanwhile, we understand the chair’s note with the agreements made in the last meeting is sufficient to clarify that there is no restriction on DSR cancellation no matter whether the MAC PDU is transmitted by the MAC entity itself or other MAC entity.

Therefore, regarding the MAC specification, we prefer to keep the current MAC specification on DSR cancellation as it is.

- [MAC-6] For DSR cancellation in case of DC in Rel-19, no MAC specification change is foreseen.

4. Conclusion

Based on the discussion above, we made the following observations and proposals:

SRB impact for LCH priority adjustment

- Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

[UE Cap-1] The pre-requisite of enhanced DSR

- From UE implementation perspective, considering UE behaviour for Rel-18 DSR is actually a special case of Rel-19 DSR, it is not possible for a UE only support Rel-19 DSR without Rel-18 DSR.

- [UE Cap-1] From UE capability signalling perspective, no need to have the pre-requisite for the capability of Rel-19 DSR.

[MAC-1] Pending DSR cancellation

- For Rel-19 DSR, no issue is foreseen if relying on legacy pending DSR cancellation mechanism based on the concept of delay-critical PDCP SDUs.

- [MAC-1] RAN2 does not consider any enhancement on pending DSR cancellation for Rel-19 DSR.

[MAC-2] Pending BSR cancellation

- In Rel-18, the enhancement of pending BSR cancellation based on Rel-18 DSR is not supported.

- [MAC-2] RAN2 does not consider to cancel pending BSR based on Rel-19 DSR, as what we did for Rel-18 DSR.

[MAC-6] DSR cancellation in the DC case

- [MAC-6] For DSR cancellation in case of DC in Rel-19, no MAC specification change is foreseen.

4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia