3GPP TSG-RAN WG2 Meeting #131								R2-2505275

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.5.4

Source: 	OPPO

Title:  	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and Decision

- Introduction

In Rel-19 NES WID [1], the objective on adaptation of common signal/channel transmissions is captured as follows.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution would like to provide our considerations on the objective of Rel-19 NES.

- Discussion

  - RACH adaptation

For RACH adaptation, one of remaining open issues is down-selection of solutions on how to disable RACH adaptation for MSG1 repetition, based on the following agreements made in last meeting.

RAN2 will select one of the following options. (CE here means MSG1 repetition). - Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon. - Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17)

For Option-1c, although it relies on network implementation, we understand it still need to clarify network restriction in spec to keep common understanding, so that the spec impact cannot be avoided. Considering that there are spec impacts for both options, but Option-1b puts less restriction to the configuration, we propose to select Option-1b as the solution on this issue.

- R2 selects Option-1b as the solution of RACH adaptation for MSG1 repetition.

  - Paging adaptation

2.2.1 Issue 3: Whether to introduce any other Rel-19 PEI configuration (Q15)

During the email discussion of the current RRC running CR, Q15 was discussed as shown below.

Q15a: Please comment on whether the following parameters should be introduced for Rel-19 PEI configuration: - po-NumPerPEI-r19 - payloadSizeDCI-2-7-r19 - pei-FrameOffset-r19 - subgroupsNumPerPO-r19 - subgroupsNumForUEID-r19

We understand that there is no clearer benefit of introducing these additional Rel-19 PEI configurations, since

1) for i, extending the value would lead to smaller number of subgroups for each PO, and will increase false alarm rate and thus hurt the original intention of introducing PEI, i.e., for UE power saving.

2) for ii, iv, v, it is restricted by R1 design, so we should not touch it.

3) for iii, it is not clear what is the reason for extending the value range.

Therefore, we propose not to introduce these parameters for Rel-19 PEI configurations.

- R2 not pursue introducing the parameters of: po-NumPerPEI-r19, payloadSizeDCI-2-7-r19, pei-FrameOffset-r19, subgroupsNumPerPO-r19, subgroupsNumForUEID-r19, for Rel-19 PEI configurations.

2.2.2 Issue 4: Whether to group paging adaptation configuration (Samsung004)

Considering for paging adaptation, more than one parameter is introduced in PCCH-Config, including

[[

pagingAdaptation-NS-r19                                  ENUMERATED {eight,four, two, one}                 OPTIONAL,  -- Need R

pagingAdaptationNAndPagingFrameOffset-r19               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15),

oneThirtySecondT                     INTEGER (0..31)

}                                                                                                          OPTIONAL,  -- Need R

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZoneThirtySecondT                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-SCS60KHZoneThirtySecondT                                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneEighthT-sCS120KHZoneThirtySecondT                                                   SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneSixteenthT                                                      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

}                                                                                                              OPTIONAL           -- Need R

]]

One open issue proposed by companies is whether to group these parameters into one umbrella IE, e.g., pagingAdaptation-r19, and pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset is set to be mandatory within the umbrella IE.

The only point to consider is whether we need to mandate these parameters for paging adaptation or not. In order to obtain the network power saving gain from paging adaptation, we understand N value (i.e., pagingAdaptationNAndPagingFrameOffset) and Ns value (i.e., pagingAdaptation-NS-r19) should be both mandatory present. And the PO-location (i.e., firstPDCCH-MonitoringOccasionOfPO-r19) can be optional, from signalling overhead reduction perspective.

- R2 agrees to group the parameters for paging adaptation into one optional umbrella IE, and, within the umbrella IE, set pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset to be mandatory present, while keep firstPDCCH-MonitoringOccasionOfPO-r19 to be optional.

2.2.3 Issue 5: Whether support (e)RedCap + paging adaptation (OPPO005)

In the current RRC running CR, pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 is implemented in PDCCH-ConfigCommon for PO locations with the condition of “Cond OtherBWP”, while R2 has already made agreement to exclude paging clustering/bundling/adaptation in RRC_CONNECTED.

During the email discussion, companies clarify that it is used for BWP other than InitialDownlinkBWP (e.g., initialDownlinkBWP-RedCap), i.e., to configure PO locations in PDCCH-ConfigCommon for (e)RedCap. Therefore, one general issue is that whether to support both (e)RedCap and paging adaption or not.

We understand that it is doubtful whether we can easily extend the applicable scenario for paging adaptation to (e)RedCap without detailed analysis. For example, the capability for supporting PO bundling within initialDownlinkBWP-RedCap-r17 may not be covered by legacy (e)RedCap capability or the new NES capability for non-redcap UE. If R2 goes for a single capability bit, i.e., pagingAdaptation-r19, an example of the change on capability CR is shown as below, which would lead to different interpretation of the bits for (e)RedCap UE and non-(e)RedCap UE.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pagingAdaptation-r19 Indicates whether the UE supports paging adaption, in which the value range for parameter N and Ns as defined in TS 38.331[9] are extended to make it possible to have increased interval between Paging Frames and compensate the decrease in the number of Paging Frames. If (e)redcap UE reports its supporting for paging adaptation, it shall support paging adaptation in initialDownlinkBWP-RedCap-r17, while for other UE reports its supporting for paging adaptation, it does not support it. | UE | No | No | No |

- If (e)RedCap UE reports its supporting for paging adaptation, it shall support paging adaptation in initialDownlinkBWP-RedCap-r17,

- while for other non-(e)RedCap UE reports supporting for paging adaptation, it shall support paging adaptation configured for initialDownlinkBWP,

i.e., the supported function for (e)RedCap UE and non-(e)RedCap UE is different relying on POs configured for normal initial BWP and for RedCap-specific initial BWP, and thus motivates separate capability bits.

- The requirement of support of paging adaptation for (e)RedCap UE and non-(e)RedCap UE is different, where the former needs to consider for RedCap-specific initial BWP, while the latter is only for normal initial BWP. It would thus lead to different capability requirement.

Considering the last meeting before Rel-19 feature freeze, to avoid leading additional discussion in R2 or even R1 impact caused by this issue, we prefer not to support this extension, and leave all the cross-WI issue(s) to maintenance phase.

Besides, the similar issue can also be considered for every objective of Rel-19 NES. For example,

- For (e)RedCap UE supporting RACH adaptation, one can consider whether/how to enable additional RACH in RedCap-specific initial UL BWP.

- For (e)RedCap UE supporting OD-SIB1, one can consider whether/how to configure separate RACH resource used for OD-SIB1 request for (e)RedCap UE in RedCap-specific initial UL BWP.

i.e., paging adaptation should not be handled differently.

- For co-existence with (e)RedCap, besides paging bundling, similar issues can also be considered for like RACH adaptation and OD-SIB1.

- R2 not work on paging adaptation co-existence with (e)RedCap at Q3, i.e., remove the IE pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 implemented in PDCCH-ConfigCommon in the current RRC running CR for now, and discuss the cross-WI issues (if any) in the maintenance phase.

- If P4 is not agreeable, introduce separate capabilities for the support of paging bundling for non-redcap UE (which does not need to support pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 in PDCCH-ConfigCommon) and redcap UE (which needs to support pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 in PDCCH-ConfigCommon).

  - SSB adaptation

In R2#129bis and R2#130 meetings, the following agreements for L3 measurement were made.

Agreements on common channel adaptation 5. For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. 6. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO. No RAN2 spec impact is foreseen.

In the last meeting, R2 has agreed that relying on a single MO configuration for SSB supporting SSB burst periodicity switching to perform a combined L3 measurement and MR report.

If R2 would like to progress more, the remaining issue on servingCellMO configuration granularity could be considered, i.e., whether to apply this to per-cell MO (i.e., servingCellMO in ServingCellConfig) only or per-BWP MO (i.e., servingCellMO in BWP-DownlinkDedicated) as well. Considering that the similar issue also exists for OD-SSB, we propose to align with the conclusion for OD-SSB.

- For SSB adaptation, R2 keeps the same decision as for OD-SSB, i.e., not pursue adaptation of NCD-SSB configured in BWP-DownlinkDedicated.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

RACH adaptation

- R2 selects Option-1b as the solution of RACH adaptation for MSG1 repetition.

Paging adaptation

- R2 not pursue introducing the parameters of: po-NumPerPEI-r19, payloadSizeDCI-2-7-r19, pei-FrameOffset-r19, subgroupsNumPerPO-r19, subgroupsNumForUEID-r19, for Rel-19 PEI configurations.

- R2 agrees to group the parameters for paging adaptation into one optional umbrella IE, and, within the umbrella IE, set pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset to be mandatory present, while keep firstPDCCH-MonitoringOccasionOfPO-r19 to be optional.

- The requirement of support of paging adaptation for (e)RedCap UE and non-(e)RedCap UE is different, where the former needs to consider for RedCap-specific initial BWP, while the latter is only for normal initial BWP. It would thus lead to different capability requirement.

- For co-existence with (e)RedCap, besides paging bundling, similar issues can also be considered for like RACH adaptation and OD-SIB1.

- R2 not work on paging adaptation co-existence with (e)RedCap at Q3, i.e., remove the IE pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 implemented in PDCCH-ConfigCommon in the current RRC running CR for now, and discuss the cross-WI issues (if any) in the maintenance phase.

- If P4 is not agreeable, introduce separate capabilities for the support of paging bundling for non-redcap UE (which does not need to support pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 in PDCCH-ConfigCommon) and redcap UE (which needs to support pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 in PDCCH-ConfigCommon).

SSB adaptation

- For SSB adaptation, R2 keeps the same decision as for OD-SSB, i.e., not pursue adaptation of NCD-SSB configured in BWP-DownlinkDedicated.

4. Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson