3GPP TSG-RAN WG2 Meeting #131	R2-2505277

Bangalore, India, 25th – 29th Aug. 2025

Source: 	Xiaomi

Title:  	Remaining issues of inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

# Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM), as shown below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In this contribution, we will discuss remaining issues of inter-CU LTM.

# Discussion

  - (MAC-25) Handling of the activated SP CSI-RS for target cell after LTM cell switch

In email discussion “[POST130][111][MOB] (Vivo)” [2], open issue MAC-25 is as follows:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-25 | Handling of the activated SP CSI-RS for target cell after LTM cell switch Editor’s NOTE: After reconfiguration with sync that is triggered by LTM, it is FFS how to handle the configured Semi-persistent CSI-RS/CSI-IM resource sets for target cell. Based on the following agreement: UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell | Issue Type: Essential How to address it: based on companies’ contribution |

The issue is related to the following RAN1 agreements on CSI-RS measurement for CSI acquisition:

RAN1#120 - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration RAN1#120bis Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured RAN1#121 For PUSCH to convey the early CSI report, - For RACH-less LTM, the first CG or DG PUSCH after CSC is used. - For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A. - For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B. For the reporting LTM early CSI reporting, Table 6.3.1.1.2-7 in TS 38.212 is used as a UCI report format.

It can be seen that from RAN1 agreements, UE can perform CSI-RS measurement of target cell for early CSI report purpose, and such report is sent once after CSC. Afterwards, the SP CSI-RS resource is deactivated. Current MAC CR [3] captures the deactivation of SP CSI-RS resource as follows:

After reconfiguration with sync that is triggered by LTM, the configured Semi-persistent CSI-RS/CSI-IM resource sets for all candidate cell(s), except the target cell, are deactivated.

Since early CSI report can be considered as part of reconfiguration with sync, SP CSI-RS resource for target cell is deactivated after cell switch.

Proposal 1: (MAC-25) SP CSI-RS resource for target cell is deactivated after cell switch.

  - LTM based failure recovery

Failure recovery when the selected cell is the failed target cell

For inter-CU LTM failure recovery, RAN2 reached the following agreement [4]:

If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell's PCI and frequency.

Based on the agreement, if the selected cell is the target cell, the UE performs LTM fast recovery to the target cell with the security key update using the NCC in LTM CSC.

In the current RRC running CR [5], this case includes the following steps, as shown in figure 1:

- During inter-CU LTM execution, the UE performs AS key update to generate the new AS key KgNB* based on Target PCI +ARFCN and the NCC value in LTM CSC, and performs PDCP re-establishment. The COUNT value of SRB(s) is the initial value (e.g., 0).

- Upon the inter-CU LTM failure, the UE reverts back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG.

- When the selected cell is the target cell, the UE performs LTM execution to access the target cell again, and the UE performs AS key update, and performs PDCP re-establishment again. The security key is the same key in step1 and the COUNT value is also the initial value.

Figure 1

Based on the above analysis, although both COUNT value and AS key are reused for LTM based failure recovery when the selected cell is the target cell, the SRB messages in Step 1 and Step 3 are the same message, which are the RRCReconfigurationComplete message associated with the RRCReconfiguration of the target cell (i.e., the selected cell).  For the same SRB message, even if both COUNT value and AS key are reused, there is no key stream reuse issue.

Proposal 2: For inter-CU LTM failure recovery, even if the selected cell is the target cell, there is no security issue, and no specification change is needed in RRC CR for the case.

The UE capability for inter-CU LTM based failure recovery

In the post email discussion [6], whether to define a capability for inter-CU LTM recovery has been discussed.

Compared with Rel-18 intra-CU LTM, in Rel-19 inter-CU LTM, to execute LTM based failure recovery, more UE behaviours are needed and the UE needs to compare the Rel-19 ID, perform key update and PDCP re-establishment. Hence, RAN2 can define a capability for inter-CU LTM recovery.

Proposal 3: Define a per UE capability for inter-CU LTM recovery.

  - SCG LTM Configuration

How to configure the coexistence of intra-CU and inter-CU SCG LTM.

The coexistence case of intra-CU SCG LTM and inter-CU SCG LTM has been supported and RAN2 to support intra-CU SCG LTM in MN RRC message, in addition to SN RRC message. Based on current progress, one open issue is whether SCG LTM in MN RRC message (i.e., LTM-ConfigNRDC-r19) and SCG LTM in SN RRC message (i.e., LTM-Config-r18) can be configured to UE simultaneously.

If SCG LTM in MN RRC message and SCG LTM in SN RRC message are configured to UE at the same time, there might be two different LTM candidate ID pools, which can cause ambiguity regarding LTM configuration ID. Because SCG LTM can only be triggered by the Cell Switch Command MAC CE from the serving SN, the UE cannot determine whether the LTM configuration ID indicated in the SN Cell Switch Command MAC CE is from the MN RRC message ID pool or the SN RRC message ID pool. Therefore, the common LTM candidate ID pool is needed for all SCG LTM configurations. In Rel-18 SCPAC, for one UE, only either MN format or SN format (only intra-SN case is possible) is used. The similar restriction, that the mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN RRC message (i.e., LTM-ConfigNRDC-r19), can address the common LTM candidate ID pool issue for all SCG LTM configuration.

Proposal 4: The mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN RRC message (i.e., LTM-ConfigNRDC-r19).

# Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1	(MAC-25) Handling of the activated SP CSI-RS for target cell after LTM cell switch

Proposal 1: (MAC-25) SP CSI-RS resource for target cell is deactivated after cell switch.

2.2	LTM based failure recovery

Proposal 2: For inter-CU LTM failure recovery, even if the selected cell is the target cell, there is no security issue, and no specification change is needed in RRC CR for the case.

Proposal 3: Define a per UE capability for inter-CU LTM recovery.

2.3 SCG LTM Configuration

Proposal 4: The mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN RRC message (i.e., LTM-ConfigNRDC-r19).

# Reference

[1] RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

[2] R2-2505398, vivo, "Discussion summary and list of MAC open issue for Mob Ph4"

[3] R2-2505397, vivo (Rapporteur), "MAC CR for Mob Ph4"

[4] R2-xxxxxxx, “Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta”

[5] R2-xxxxxxx, “Running RRC CR mobility enhancements phase 4”, Ericsson, Huawei

[6] R2-xxxxxxx, “Report of [POST130][120][MOB] (CATT)”, CATT