3GPP TSG-RAN WG2 Meeting#131      					           R2-2505288

Bangalore, India  Aug 25th – 29th, 2025

Source: 			ZTE corporation, Sanechips

Title: 	Discussion on tracking area code information in SON reports

Agenda item:		5.1.3.1

Document for: 	Discussion and Decision

# Introduction

This contribution intends to discuss the issues on tracking area code information in RA, SHR and SPR report. Proposed solution is also included in conclusion part.

# Discussion

According to RAN2#108 CR R2-1915672, the presence and absence of Tracking Area Code (TAC) in NR cell is defined as follows:

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN).

For the highlighted content, the absence of TAC can be deduced to indicate that the cell supports only PSCell/SCell functionality. While the TAC is always present for a PCell, otherwise UE will consider the cell as barred according to current specs. Only for PSCell/SCell only cell, TAC can be optional.

Observation 1: If a cell is to support PCell functionality, then TAC must be present. While, for a PSCell or SCell, TAC may or may not be available.

TAC information logging have been supported in several SON reports. However, there are some misalignment between availability of TAC and UE's logging behavior, which are discussed in below section.

TAC information in the RA report

----------------------------------------------------------- From 38331 V16.20.0------------------------------------------------------

#### 5.7.10.4	Actions upon successful completion of random-access procedure

Upon successfully performing random-access procedure initialized with 4-step RA type, the UE shall:

************omitted text***********************

3>	append the following contents associated to the successfully completed random-access procedure as a new entry in the VarRA-Report:

4>	if the list of EPLMNs has been stored by the UE:

5>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN) without exceeding the limit of maxPLMN;

4>	else:

5>	set the plmn-Identity, in plmn-IdentityList, to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

4>	set the cellId to the global cell identity and the tracking area code, if available, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

----------------------------------------------------------- From 38331 V16.20.0------------------------------------------------------

According to the procedural text as cited above, when UE logs the CellId information in the RA report UE prioritize logging of CGI and TAC or the physical cell identity and carrier frequency information of the cell in which the corresponding random-access preamble was transmitted.

Based on Observation 1, since the cell logged in RA report can be a PCell or a PSCell or a SCell, the TAC information may or may not be available. While current procedure text mandates UE to log TAC when CGI is logged, which is incorrect. Therefore, it is proposed to update the procedure text that when including CGI as part of cellId in an RA report, the tracking area information shall remain optional. The text shall be updated since R16.

Observation 2: RA report contains RA information occurred in SpCells or SCells where the TAC information might not be available while current procedure text mandates UE to log TAC when CGI is logged as part of cellId.

Proposal 1: Correct since Rel-16, that when cellId in RA report is set to CGI, the TAC is logged only when available.

TAC information for the PCell included in SON reports

In the current specs, there are different handling of recording the PCell ID and TAC information, which are cited as follows:

----------------------------------------------------------- From 38331 V17.13.0------------------------------------------------------

#### 5.3.10.5	RLF report content determination

The UE shall determine the content in the VarRLF-Report as follows:

************omitted text***********************

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

************omitted text***********************

2>	if last MobilityFromNRCommand concerned a failed inter-RAT handover from NR to E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA (NR to EUTRA):

3>	set the eutraFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last MobilityFromNRCommand message was received;

************omitted text***********************

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

************omitted text***********************

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the PCell where the last executed RRCReconfiguration message including reconfigurationWithSync was received;

************omitted text***********************

4>	include the eutraPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the E-UTRA PCell where the last RRCReconfiguration message including reconfigurationWithSync was received embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

#### 5.7.10.4	Actions upon successful completion of a random-access procedure or on completion of a request of on-demand system information

Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5, the UE shall:

************omitted text***********************

4>	if the UE supports spCell ID indication:

5>	if the corresponding random-access procedure was performed on an SCell of MCG:

6>	set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on an SCell of SCG:

6>	set the spCellId to the global cell identity of the PSCell, if available, otherwise, set the spCellId to the global cell identity of the PCell;

#### 5.7.10.6	Actions for the successful handover report determination

The UE shall for the PCell:

************omitted text***********************

3>	for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

4>	set the sourcePCellID in sourceCellInfo to the global cell identity and tracking area code, if available, of the source PCell;

************omitted text***********************

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for the target PCell indicated in the last applied RRCReconfiguration message including reconfigurationWithSync:

4>	set the targetPCellID in targetCellInfo to the global cell identity and tracking area code, if available, of the target PCell; otherwise, set the targetCell-PCI-ARFCN to the physical cell identity and carrier frequency of the target PCell;

----------------------------------------------------------- From 38331 V17.13.0------------------------------------------------------

----------------------------------------------------------- From 38331 V18.6.0------------------------------------------------------

#### 5.7.10.7	Actions for the successful PSCell change or addition report determination

The UE shall for the PSCell:

************omitted text***********************

3>	set the pCellId to the global cell identity and, if available, tracking area code of the PCell;

----------------------------------------------------------- From 38331 V18.6.0------------------------------------------------------

Based on the clarification regarding the presence scenarios of TAC in the aforementioned CR and the current specs description (green highlighted content), we have sorted out the types of PCell ID settings, as shown in the following table:

|  | UE behavior when sets the PCell ID | Relevant reports | Analysis |
| --- | --- | --- | --- |
| Case 1 | UE only logs global cell identity, no need to log TAC | spCellId in RA report | TAC is not needed for this scenario, since spCellId is not used for RA report forwarding. |
| Case 2 | Both the global cell identity and tracking area code is logged | nrPreviousCell in RLF- report eutraPreviousCell in RLF- report | In this scenario, the UE has already connected to the PCell for a period of time, which guarantees that TAC information is always available. |
| Case 3 | UE logs either ( the global cell identity and tracking area code), if available, otherwise, (the physical cell identity and carrier frequency) | nrFailedPCellId in RLF- report eutraFailedPCellId in RLF- report targetPCellID in SHR- report | For HO case, HO can be based on PCI and ARFCN of target PCell, therefore TAC and CGI might not be available. For RLF case, the RLF can happen shortly after a successful HO where UE might not be able to acquire SIB1 when RLF happens. Therefore the TAC and CGI may not be available for this case. |

Further, for the yellow-highlighted content above (sourcePCellID in SHR and pCellId in SPR), it should fall into Case 2 because for both cases the UE has already been connected to the PCell for a sufficient time, long enough for UE to acquire CGI.In this case TAC shall also be available for the PCell, but the specified UE behavior is not as intended.

For both sourcePCellID in SHR and pcellId in SPR, the logging of PCell information is mandatory and UE logs CGI as part of the cell identity. In such case the TAC information shall be also available since it is for a PCell. There is no case TAC will be not available for PCell, therefore, the ‘if available’ in the procedure text shall be removed. .

Observation 3: For sourcePCellID logged in SHR and pcellId in SPR, UE always logs CGI information, in such case TAC must be available for the PCell logged based on the TAC presence condition specified in standards.

Since current procedure text specifies a condition that doesn’t exist, it is proposed to update the procedure text to remove the invalid scenarios. For the sourcePCellID logging in SHR, the text shall be updated since R17. For the setting of pCellId in SPR report, the text shall be updated since R18.

Proposal 2a: Correct since Rel-17, that if the UE logs sourcePCellID in SHR, it shall be set to the global cell identity and tracking area code of the source PCell. i.e., remove the ‘if available’ from the procedure text when setting sourcePCellID in SHR..

Proposal 2b: Correct since Rel-18, that if the UE logs pCellId in SRP, it shall be set to the global cell identity and tracking area code of the PCell, i.e., remove the ‘if available’ from the procedure text when setting pCellId in SPR.

TAC information in the PSCell included in SPR report

------------------------------------------------------------- From 38331 V18.6.0------------------------------------------------------

#### 5.7.10.7	Actions for the successful PSCell change or addition report determination

The UE shall for the PSCell:

************omitted text***********************

3>	for the source PSCell (in case of PSCell change procedure) in which the last RRCReconfiguration message for the SCG including reconfigurationWithSync was applied:

4>	set the sourcePSCellId in sourcePSCellInfo to the global cell identity and tracking area code, and otherwise to the physical cell identity and carrier frequency of the source PSCell;

4>	set the sourcePSCellMeas in sourcePSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE successfully completed the random access procedure for the SCG;

4>	set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE successfully completed the random access procedure for the SCG;

3>	for the target PSCell indicated in the last applied RRCReconfiguration message for the SCG including reconfigurationWithSync:

4>	set the targetPSCellID in targetPSCellInfo to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PSCell;

------------------------------------------------------------- From 38331 V18.6.0------------------------------------------------------

Currently, when logging sourcePSCellId and targetPSCellID in SPR as shown in above text. Similar issue as analyzed in subclause 2.1 exists, that UE always log CGI together with tracking area code for a PSCell, which is incorrect since TAC can be optional for a cell supports only PSCell functionality. Therefore, it is proposed to update the procedure text that when including CGI as part of sourcePSCellId and targetPSCellId in a SPR report, the tracking area information shall remain optional. The text shall be updated since R18.

Proposal 3: Correct since Rel-18, that when sourcePSCellId and targetPSCellId in SPR report are set to CGI, the TAC is logged only when available.

Since the proposals are targeted for different release, TPs for corrections targeting for different release are provided om Annex. It is proposed that RAN2 discusses and agrees on the TPs.

Proposal 4: If P1-P3 are agreed, RAN2 discusses and agrees on the TPs provided in Annex.

# Conclusion and proposals

Based on above analysis, we have the following proposals:

Observation 1: If a cell is to support PCell functionality, then TAC must be present. While, for a PSCell or SCell, TAC may or may not be available.

Observation 2: RA report contains RA information occurred in SpCells or SCells where the TAC information might not be available while current procedure text mandates UE to log TAC when CGI is logged as part of cellId.

Observation 3: For sourcePCellID logged in SHR and pcellId in SPR, UE always logs CGI information, in such case TAC must be available for the PCell logged based on the TAC presence condition specified in standards.

Proposal 1: Correct since Rel-16, that when cellId in RA report is set to CGI, the TAC is logged only when available and agrees on the TP included in Annex for P1.

Proposal 2a: Correct since Rel-17, that if the UE logs sourcePCellID in SHR, it shall be set to the global cell identity and tracking area code of the source PCell. i.e., remove the ‘if available’ from the procedure text when setting sourcePCellID in SHR.

Proposal 2b: Correct since Rel-18, that if the UE logs pCellId in SRP, it shall be set to the global cell identity and tracking area code of the PCell, i.e., remove the ‘if available’ from the procedure text when setting pCellId in SPR.

Proposal 3: Correct since Rel-18, that when sourcePSCellId and targetPSCellId in SPR report are set to CGI, the TAC is logged only when available.

Proposal 4: If P1-P3 are agreed, RAN2 discusses and agrees on the TPs provided in Annex.

# Reference

- R2-1915672 Presence and absence of TAC in NR cell, RAN2#108, Ericsson

- Annex 1 – TP for R16 38.331

--------------------------------------- 1st change to TS 38331 ---------------------------------------

#### 5.7.10.4	Actions upon successful completion of random-access procedure

Upon successfully performing random-access procedure initialized with 4-step RA type, the UE shall:

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report:

2>	clear the information included in VarRA-Report;

1>	if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of PLMN entries in plmn-IdentityList stored in VarRA-Report is less than maxPLMN; or

2>	if the number of PLMN entries in plmn-IdentityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the successfully completed random-access procedure as a new entry in the VarRA-Report:

4>	if the list of EPLMNs has been stored by the UE:

5>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN) without exceeding the limit of maxPLMN;

4>	else:

5>	set the plmn-Identity, in plmn-IdentityList, to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

4>	if global cell identity is available, set the cellId to the global cell identity and, if available, the tracking area code, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

4>	set the raPurpose to include the purpose of triggering the random-access procedure;

4>	set the ra-InformationCommon as specified in clause 5.7.10.5.

--------------------------------------- 1st change to TS 38331 ---------------------------------------

- Annex 2 – TP for R17 38.331

- --------------------------------------- 1st change to TS 38331 -----------------------------------------

#### 5.7.10.4	Actions upon successful completion of a random-access procedure or on completion of a request of on-demand system information

Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, the UE shall:

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report:

2>	clear the information included in VarRA-Report;

1>	if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of PLMN entries in plmn-IdentityList stored in VarRA-Report is less than maxPLMN; or

2>	if the number of PLMN entries in plmn-IdentityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the successfully completed random-access procedure or the failed or successfully completed on-demand system information acquisition procedure as a new entry in the VarRA-Report:

4>	if the list of EPLMNs has been stored by the UE:

5>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN) without exceeding the limit of maxPLMN;

4>	else:

5>	set the plmn-Identity, in plmn-IdentityList, to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

4>	if global cell identity is available, set the cellId to the global cell identity and, if available, the tracking area code, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

- --------------------------------------- 1st change to TS 38331 -----------------------------------------

- --------------------------------------- 2nd change to TS 38331 -----------------------------------------

#### 5.7.10.6	Actions for the successful handover report determination

The UE shall for the PCell:

1>	if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync; or

1>	if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or

1>	if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or

1>	if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

3>	clear the information included in VarSuccessHO-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e., includes the RPLMN);

3>	set the c-RNTI to the C-RNTI assigned by the target PCell of the handover;

3>	for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

4>	set the sourcePCellID in sourceCellInfo to the global cell identity and tracking area code of the source PCell;

4>	set the sourceCellMeas in sourceCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	set the rsIndexResults in sourceCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

- --------------------------------------- 2nd change to TS 38331 -----------------------------------------

- Annex 3 – TP for R18 38.331

- --------------------------------------- 1st change to TS 38331 -----------------------------------------

#### 5.7.10.4	Actions for the Random Access report determination

Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5, the UE shall:

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report; or

1>	if the registered SNPN identity or the SNPN identity selected by upper layers (see TS24.501 [23]) from the list of SNPN(s) included in the NPN-IdentityInfoList in SIB1is not included in snpn-IdentityList stored in a non-empty VarRA-Report:

2>	clear the information included in VarRA-Report;

1>	if the UE is not in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is less than maxPLMN; or

2>	if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the random-access procedure as a new entry in the VarRA-Report:

4>	if the list of EPLMNs has been stored by the UE:

5>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN) without exceeding the limit of maxPLMN;

4>	else:

5>	set the plmn-Identity, in plmn-IdentityList, to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

4>	if global cell identity is available, set the cellId to the global cell identity and, if available, the tracking area code, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

4>	if the UE supports spCell ID indication:

5>	if the corresponding random-access procedure was performed on an SCell of MCG:

6>	set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on an SCell of SCG:

6>	set the spCellId to the global cell identity of the PSCell, if available, otherwise, set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on PSCell:

6>	if the cellId is not set to the global cell identity of the PSCell, set the spCellId to the global cell identity of the PCell;

4>	if the random-access procedure is initiated for SDT and the SDT transmission was failed as defined in TS 38.300 [2]:

5>	include the sdt-Failed;

4>	set the raPurpose to include the purpose of triggering the random-access procedure;

4>	set the ra-InformationCommon as specified in clause 5.7.10.5.

1>	if the UE is in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of SNPN identity entries in snpn-IdentityList in identityList stored in VarRA-Report is less than maxNPN; or

2>	if the number of SNPN identity entries in snpn-IdentityList in identityList stored in VarRA-Report is equal to maxNPN and the list of equivalent SNPN(s) is subset of or equal to the snpn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the random-access procedure as a new entry in the VarRA-Report:

4>	if the list of equivalent SNPN(s) has been stored by the UE:

5>	set the snpn-IdentityList to include the list of equivalent SNPN(s) stored by the UE (i.e. includes the registered SNPN identity) without exceeding the limit of maxNPN;

4>	else:

5>	set the snpn-Identity, in snpn-IdentityList, to the SNPN identity selected by upper layers (see TS 24.501 [23]) from the SNPN identities included in the NPN-IdentityInfoList in SIB1;

4>	if global cell identity is available, set the cellId to the global cell identity and, if available, the tracking area code, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

- --------------------------------------- 1st change to TS 38331 -----------------------------------------

- --------------------------------------- 2nd change to TS 38331 -----------------------------------------

#### 5.7.10.6	Actions for the successful handover report determination

The UE shall for the PCell:

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running; or:

1>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA; or

1>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the Mobility from NR to E-UTRA and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA:

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

3>	clear the information included in VarSuccessHO-Report, if any;

3>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e., includes the RPLMN);

3>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity), if available;

3>	for intra-NR handover, set the c-RNTI to the C-RNTI assigned by the target PCell of the handover;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; or

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for the source PCell in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied:

4>	set the sourcePCellID in sourceCellInfo to the global cell identity and tracking area code of the source PCell;

- --------------------------------------- 2nd change to TS 38331 -----------------------------------------

- --------------------------------------- 3nd change to TS 38331 -----------------------------------------

#### 5.7.10.7	Actions for the successful PSCell change or addition report determination

The UE shall for the PSCell:

1>	if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message for the SCG including the reconfigurationWithSync, is greater than thresholdPercentageT304-SCG if included in the successPSCell-Config received before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync for the SCG, is greater than thresholdPercentageT310-SCG included in the successPSCell-Config if configured by the source PSCell before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured and if the T312 associated to the measurement identity of the target PSCell included in a measConfig associated with the SCG was running at the time of initiating the execution of the reconfiguration with sync procedure for the SCG and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312-SCG included in the successPSCell-Config if configured by the source PSCell before executing the last reconfiguration with sync for the SCG:

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is not configured and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync for the SCG, is greater than thresholdPercentageT310-SCG included in the successPSCell-Config if configured by the PCell before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is not configured and if the T312 associated to the measurement identity of the target PSCell included in a measConfig associated with the SCG was running at the time of initiating the execution of the reconfiguration with sync procedure for the SCG and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312-SCG included in the successPSCell-Config if configured by the PCell before executing the last reconfiguration with sync for the SCG:

2>	clear the information included in VarSuccessPSCell-Report, if any;

2>	store the successful PSCell change or addition information in VarSuccessPSCell-Report and determine the content in VarSuccessPSCell-Report as follows:

3>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs (including the RPLMN) stored by the UE;

3>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPN identities (including the registered SNPN identity) stored by the UE, if available;

3>	set the pCellId to the global cell identity and tracking area code of the PCell;

3>	for the source PSCell (in case of PSCell change procedure) in which the last RRCReconfiguration message for the SCG including reconfigurationWithSync was applied:

4>	if global cell identity is available, set the sourcePSCellId in sourcePSCellInfo to the global cell identity and, if available, tracking area code, and otherwise to the physical cell identity and carrier frequency of the source PSCell;

4>	set the sourcePSCellMeas in sourcePSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE successfully completed the random access procedure for the SCG;

4>	set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE successfully completed the random access procedure for the SCG;

3>	for the target PSCell indicated in the last applied RRCReconfiguration message for the SCG including reconfigurationWithSync:

4>	if global cell identity is available, set the targetPSCellID in targetPSCellInfo to the global cell identity and, if available, tracking area code, and otherwise to the physical cell identity and carrier frequency of the target PSCell;

- --------------------------------------- 3nd change to TS 38331 -----------------------------------------