3GPP TSG RAN WG2 Meeting #131         	                          R2-2505301

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.1.2.2

Source:	Xiaomi

Title:	Discussion on open issues of AI/ML air LCM

Document for:	Discussion and Decision

# Introduction

Based on the conclusions and running CR in [POST130][026][AI PHY] 38.331 CR (Ericsson), in this contribution, we further discuss the remaining open issues of AI/ML air LCM.

# Discussion

## RRC-14: Repeated reports of applicability for configurations which consistently perform poorly

During post email discussion, there’s some concerns that UE may repeatedly report a configuration as applicable due to poor performance. It was proposed in [POST130][026][AI PHY] 38.331 CR (Ericsson) that the network should release the configurations of functionalities that continuously perform poor. This operation can be up to network implementation.

In our understanding, 5GA system shall ensure that the 5GA System can also operate without AI/ML. Even network releases inference configuration of applicable functionalities, network can provide non-AI/ML configurations in the same RRCReconfiguration message while releasing the inference configuration. Alternatively, network can also provide inference configuration for other applicable functionalities, if previously not provided to UE. With new configuration, either for non-AI operation or AI operation, UE does not need to go to IDLE when inference configuration of the poor performed functionality is released by network.

Proposal 1: (RRC-14) When network releases inference configurations of poor-performed applicable functionalities, network should also provide either non-AI/ML configuration in CSI-ReportConfig or provides full inference configuration of other applicable functionalities, if previously not configured to UE.

## RRC-15/RRC-17/RRC-45: Activation Time and Processing Delay

Regarding activation of AI/ML functionality, RAN1 and RAN2 have made the following agreements:

R1-2410898 in RAN2 #129 meeting: - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. RAN2 agreements: If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Additionally, RAN2 receives an LS from RAN4 (R4-2508085 [1]) asking what is the correct interpretation of the above RAN2 agreement:

- Interpretation 1: the applicable functionalities are already activated (i.e., the applicable functionalities activation is completed) before reporting applicable functionalities via RRCReconfigurationComplete in step 4 - Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

In our understanding, RAN4 LS is discussing the same issue as RRC-15.

Looking at the above agreements from both RAN1 and RAN2, it is clear that, as long as one functionality is reported as ‘applicable’ in RRCReconfigurationComplete message, the reported applicable AI/ML functionality is in ‘activated’ state, as yellow-highlighted in the above RAN1/2 agreements.

Moreover, in our understanding, ‘activated’ state of the applicable functionality means an AI/ML functionality is ready to perform inference anytime. Additional processing time to activate the applicable functionality after reporting is not needed.

Proposal 2: (RRC-15) Applicable functionalities are in ‘activated’ state (i.e., ready to perform inference) upon reporting via RRCReconfigurationComplete message. Additional processing time after applicability reporting is not needed.

However, how UE activates AI/ML functionalities and how long does the UE need to make AI/ML functionality become available before reporting RRCReconfigurationComplete message depends on UE capability. This further lead to different requirements on RRC processing delay.

As captured in TS 38.331, RRC procedure delay requirement is defined between the time when UE received RRC message from network till UE message on UE physical layer is ready for transmission (i.e., ready for the reception of uplink grant for the UE).

12 Processing delay requirements for RRC procedures The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3. Figure 12.1-1: Illustration of RRC procedure delay

As discussed earlier, UE should complete applicability checking and functionality activation before applicability reporting via RRCReconfigurationComplete. It was proposed in [POST130][026][AI PHY] 38.331 CR (Ericsson) that 16ms can be considered as the processing delay requirement.

Proposal 7 (RRC-17) RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms with respect to the reception of RRCReconfiguration. FFS whether RAN4 input is needed. FFS whether this solves open issue RRC-15.

However, for some cases, 16ms may not be enough for UE to activate a certain functionality, especially considering different UE implementations on how the model is stored in the modem. Therefore, RRC processing delay requirements should depend on UE capabilities on how long time does the UE needs to activate the functionality. Of course, the detailed values of different processing delay requirements can be further checked and defined by RAN4.

Proposal 3: (RRC-17) Introduce multiple RRC processing delay requirements for applicability reporting based on UE capability. Values of RRC processing delay requirement is up to RAN4.

Based on this, activation of functionality can be up to UE implementation, i.e., there’s no specification impact on how UE activates the functionality before applicability reporting.

Proposal 4: (RRC-45) There’s no specification impact on how UE activates the functionality before applicability reporting.

## RRC-16: UE behaviour when the associated ID is not provided by the network

It was proposed in [POST130][026][AI PHY] 38.331 CR (Ericsson) to further discuss UE behaviours while associated ID is not provided by the network:

“Proposal 6 (RRC-16) RAN2 to discuss the following options: (a) If the network does not provide the associated ID, the UE reports the applicability (applicable/inapplicable) based on UE-side additional conditions (internally known by UE), model availability in device and the other provided information in the configuration. (b) If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.”

Recalling that associated ID is mainly introduced to keep consistency between training and inference.

Network may or may not provide associated ID in training data collection and inference in the following four scenarios:

- Case 1: NW provides associated ID for both data collection and applicability report.

- Case 2: NW doesn’t provide associated ID for both data collection and applicability report.

- Case 3: NW provide associated ID for data collection, but doesn’t provide associated ID for applicability report.

- Case 4: NW doesn’t provide associated ID for data collection, but provide associated ID for applicability report.

From UE perspective, without consistently knowing associated ID in both training data collection and inference, it is impossible for UE to select a model to perform inference that can be consistent with its training condition.

Proposal 5: (RRC-16) NW shall ensure to provide associated ID consistently in training data collection configuration and inference configuration.

For the case where associated ID is not provided, the only possible UE behaviour is UE decides functionality applicability up to UE implementation. It is most likely the inapplicable functionality may perform poor under current network condition and configuration. Network may implicitly know the functionality is not applicable based on performance monitoring (if performance is continuously poor).

Observation 1: (RRC-16) Network can implicitly know the functionality is not applicable based on performance monitoring.

As discussed in RRC-14, network can then release current inference configuration, then either provides new inference configuration or fallback to non-AI/ML.

Proposal 6: (RRC-16) If associated ID is not provided by network, it is up to UE implementation how to determine the applicability.

## RRC-41/RRC-46: Associated ID

Before discussing the value range of associated ID, we first need to align the coverage of associated ID, i.e., whether it is cell-specific or multi-cell specific.

According to RAN1 previous discussion, associated ID is used to distinguish different NW-side additional condition. When associated ID of the AI/ML functionality is changed, it is possible that the AI/ML model at UE side may not be suitable for the new NW-side additional condition (i.e., an AI/ML functionality with a new associated ID). We understand it is true that associated ID is configured by network up to network implementation, as network vendors are clear about NW-side additional conditions which are not visible to UE. It was previously agreed by RAN1 that associated ID is at least per cell-specific. However, in our understanding, cell-specific associated ID may heavily increase UE implementation complexity. A wider-coverage granularity (i.e., multi-cell specific) is preferred to make UE-side model commercially successfully deployed. Therefore, in below section, we try to address companies concern on defining associated ID per multi-cell.

Observation 2: (RRC-46) Defining multi-cell specific associated ID can significantly simply UE-side model implementation complexity, which is critical for successful commercialization.

One may think that applicability checking is done based on associated ID regardless the knowledge of which cell. However, even the cell-specific associated ID and inference configuration provided via CSI-ReportConfig and/or OtherConfig are the same, it is still possible that two cells have different NW-side additional conditions, which makes the same associated ID in different cells have different

Observation 3: (RRC-46) Even UE receives the same cell-specific associated ID and inference configuration from the network, it is still possible that NW-side additional condition is different among cells.

One may also think that UE can train a single model by collecting data from different cells with different associated IDs. However, based on TR 38.843, the generalization performance could be very poor due to combining datasets from different conditions/assumptions. To make sure the proper performance of UE-side model, UE cannot train a single model by using multiple inputs from different associated IDs.

Observation 4: (RRC-46) Generalization performance of UE-side model can be poor if a single model is trained based on dataset from different associated IDs.

Based on above observations, we think it is important to consider associated ID per multi-cell basis. More specifically, for Rel-19, as long as the associated ID with same/similar NW-side additional condition (e.g., beam deployment) is unique within PLMN, UE can combine the dataset from the same associated ID and

Proposal 7: (RRC-46) Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment. Whether it’s cell-specific or multi-cell specific is up to network implementation.

## RRC-44: RRCReconfigurationComplete for applicability updates

It was agreed in RAN2 #129 meeting with following agreements:

3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

In our understanding, it is clear from the agreement that RRCReconfiguartionComplete message is used to send initial applicability repot, while reporting applicability update for functionalities that provided by earlier CSI-ReportConfig uses UAI. This is applicable for both Option A and Option B. Using RRCReconfigurationComplete message for delta applicability reporting is not aligned with RAN2 previous enhancement. That is, RRCReconfigurationComplete message only reports applicability status of functionalities configured in its corresponding RRCReconfiguration message.

For example, if RRCReconfiguration contains inference configuration of CSI-ReportConfigId 1/2/3/4 in 1st message, UE immediately reports RRCReconfigurationComplete with applicability status of CSI-ReportConfigId 1/2/3/4. If UE receives a follow-up RRCReconfiguration message updating inference configurations, e.g., CSI-ReportConfigId 1, UE will only include applicability status of CSI-ReportConfigId 1 in the corresponding RRCReconfigurationComplete message.

However, if configured, UE is allowed to report updates of applicability via UAI, after each RRCReconfigurationComplete contains applicability reporting.

Proposal 8: (RRC-44) RRCReconfigurationComplete message only reports applicability status of functionalities configured in its corresponding RRCReconfiguration message. UAI is used to report updates of applicability functionalities after RRCReconfigurationComplete.

## RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

Based on current agreements, the UE reports the applicability of both full inference configurations and corresponding inference related parameter sets, even if some of them may correspond to each other.

To support coexistence between Option A and Option B, the following agreement on configuration of two options were agreed in RAN2 #130 meeting:

3 (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

Considering the configuration of Option A and Option B can be provided in the same RRC message, it is straightforward that UE can report ID of both Option A and Option B in the same applicability reporting (i.e., either RRCReconfigurationComplete or UAI), or ID from either Option A or Option B.

Proposal 9: (RRC-48) It is up to UE implementation which ID (either ID of Option A/B or both IDs of Option A/B) is reported when both Option A and Option B are configured.

# Conclusion

Proposal 1: (RRC-14) When network releases inference configurations of poor-performed applicable functionalities, network should also provide either non-AI/ML configuration in CSI-ReportConfig or provides full inference configuration of other applicable functionalities, if previously not configured to UE.

Proposal 2: (RRC-15) Applicable functionalities are in ‘activated’ state (i.e., ready to perform inference) upon reporting via RRCReconfigurationComplete message. Additional processing time after applicability reporting is not needed.

Proposal 3: (RRC-17) Introduce multiple RRC processing delay requirements for applicability reporting based on UE capability. Values of RRC processing delay requirement is up to RAN4.

Proposal 4: (RRC-45) There’s no specification impact on how UE activates the functionality before applicability reporting.

Proposal 5: (RRC-16) NW shall ensure to provide associated ID consistently in training data collection configuration and inference configuration.

Observation 1: (RRC-16) Network can implicitly know the functionality is not applicable based on performance monitoring.

Proposal 6: (RRC-16) If associated ID is not provided by network, it is up to UE implementation how to determine the applicability.

Observation 2: (RRC-46) Defining multi-cell specific associated ID can significantly simply UE-side model implementation complexity, which is critical for successful commercialization.

Observation 3: (RRC-46) Even UE receives the same cell-specific associated ID and inference configuration from the network, it is still possible that NW-side additional condition is different among cells.

Observation 4: (RRC-46) Generalization performance of UE-side model can be poor if a single model is trained based on dataset from different associated IDs.

Proposal 7: (RRC-46) Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment. Whether it’s cell-specific or multi-cell specific is up to network implementation.

Proposal 8: (RRC-44) RRCReconfigurationComplete message only reports applicability status of functionalities configured in its corresponding RRCReconfiguration message. UAI is used to report updates of applicability functionalities after RRCReconfigurationComplete.

Proposal 9: (RRC-48) It is up to UE implementation which ID (either ID of Option A/B or both IDs of Option A/B) is reported when both Option A and Option B are configured.

# Reference

[1] R4-2508085	LS on AI/ML functionality activation