3GPP TSG-RAN WG2 Meeting #131	R2-2505328

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.7.5

Source:	Nokia, Nokia Shanghai Bell

Title:	RLC enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#130:

Timely retransmissions Excessive polling and retransmissions due to polling (RLC-8, RLC-9) - (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this - (RLC-9) No additional conditions are needed for the polling enhancement. Autonomous Retransmission coexistence with ARQ (RLC-7) - RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. PDU set impact on timely retransmissions (PDCP-2) - (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. - Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImportance Configuration clarifications (RRC-8) - The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity. Unnecessary retransmission avoidance Impact on RLF (RLC-6) - (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. No SDU to retransmit the poll with (RLC-11) - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work Capability or capabilities (Capability-2) - Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

Moreover, the following open issues are listed based on R2-2503767, and will be discussed in this contribution.

| Category | Index | Issue description |
| --- | --- | --- |
| Avoid unnecessary RLC retransmissions | RLC-6 | Editor’s Note: FFS whether there are any RLF detection impacts when avoiding unnecessary retransmissions is introduced. |
|  | RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. |
|  | RLC-11 | FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” |
| Rel-18 leftovers | RLC-12 | FFS how to handle PDCP SN gap report handling during UE mobility - As raised by Qualcomm below. The corresponding Rel-18 agreement is: - RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives. |

# 2	Discussions on earlier open issues

## 2.1	RLC-6: RLF detection issue

In RAN2 #130 the agreement regarding RLC-6 was:

“There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission.”

Reading and considering this rigorously, we do not think this warrants the closing of the above open issue RLC-6 yet.

Previous RAN2 meetings have seen a number of proposals for the Rel-19 RLC AM to report failure to RRC when SDUs are consistently discarded. Indeed, for an SDU the RETX_COUNT may be incremented by a timer-based retransmission and the first NACK received (whose transmission likely did not account for the timer-based retransmission), but the SDU may be discarded before value 3 is reached.

However, while consistent discarding of SDUs does indicate a failure to meet the strict QoS requirements, it does not necessarily indicate failure of the radio link as such. The sustainable bitrate may simply have dropped enough to prevent timely delivery of SDUs.

Observation 1:	Consistent discarding of SDUs by Rel-19 RLC AM may be due to a drop in the sustainable bitrate, but not due to failure of the radio link.

Proposal 1 (RLC-6):	Rel-19 RLC AM does not indicate failure to RRC based on consistent discarding of SDUs.

In fact, if the link is operational enough for UE to keep receiving the NACKs needed to increment RETX_COUNT, one might argue that the network can equally well count the NACKs it sends and eventually send an RRC Reconfiguration instead of RLC Status PDU.

Nevertheless, it still seems important for the UE to trigger RRC Re-establishment when it is unreachable in the downlink and its only active radio bearer is a DRB carrying XR traffic. For this purpose, we think the most reliable quantity for the UE to measure is unresponded polls.

Observation 2:	It seems important for the UE to trigger RRC Re-establishment when it is unreachable in the downlink and its only active radio bearer is a DRB carrying XR traffic.

Proposal 2 (RLC-6):	t-PollRetransmit expiry increments a newly introduced counter that keeps track of the number of consecutive expiries of t-PollRetransmit without t-PollRetransmit being stopped. Once the new counter reaches its own configured threshold value, RLC provides to RRC the same indication as it currently does upon RETX_COUNT reaching its threshold value.

In Annex we provide an RLC text proposal for implementing this proposal.

## 2.2	RLC-10: configuration of stopReTxObsoleteSDU

This issue was raised by ZTE in R2-2503767 where they suggested: “… , it should be specified that if DL stopReTx ObsoleteSDU is supported, the DL t-RxDiscard should be configured; and if UL stopReTxObsoleteSDU is configured, UL timer based Rx discard should be activated in NW.” This is proposed in order to avoid TX window stalling due to RX-side SDU discarding. However, based on analysis done by Futurewei in R2-2502218, such issues can be avoided with smart gNB implementation where it can advance its TX window based on awareness of the configured t-RXDiscard.

Proposal 3 (RLC-10):	No Spec impact for constraining the mutual configuration of t-RxDiscard and stopReTxObsoleteSDU is needed.

## 2.3	RLC-11: Polling with no new SDUs

RAN2 agreement in #130 meeting was:

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

- Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

Regarding the latter point, if it was indeed valid to assume that a Status PDU will always arrive, RAN2 would never have needed to introduce polling in the first place. Yet polling was introduced to cover the case where the last SDU transmitted, or a number of them, is lost at MAC and therefore the receiver cannot detect the loss, such as by starting a timer like t-Reassembly or the new t-RxDiscard.

Proposal 4 (RLC-11):	RAN2 agree that status reporting alone cannot be relied on to prevent window stalling, considering the case where the last SDU transmitted, or a number of them, is lost at MAC and therefore the receiver cannot detect the loss.

Furthermore, as indicated by Apple in R2-2503700, this issue is not a corner case and can happen often due to PDU Set discarding functionality discarding many packets at once.Thus, depending on the status of the TX window, two actions can be taken:

- If the transmitting window is stalled, to un-stall the window ASAP there seems to be no alternative to re-sending the poll in a retransmitted SDU when t-PollRetransmit expires.

- If the transmitting window is not stalled, there seems to be no reason to keep t-PollRetransmit running if the transmissions of all transmitted but non-ACKed SDUs have been stopped due to discard indication from PDCP.

Proposal 5 (RLC-11):	If the transmitting window is stalled when t-PollRetransmit expires, the poll is re-sent in a retransmitted SDU, even if only SDUs discarded by PDCP are buffered.

Proposal 6 (RLC-11):	When an RLC ACK or a PDCP-discard indication for an SDU is received, if the transmitting window is not stalled, t-PollRetransmit is stopped if, after the reception, all SDUs previously submitted to lower layer have been either ACKed or their transmissions have been stopped due to discard indication from PDCP.

## 2.4	RLC-12: SN gap report issue

Based on the current Rel-18 specifications, the following alternative courses of events may have happened with a DRB mapped on RLC AM, just before the UE receives an RRC command for an event involving network-side change of the DRB’s PDCP-termination point (handover, or bearer-type change between MN- and SN-terminated bearer):

A.	The source RAN node successfully received a PDCP SN Gap report from the UE:

-	Based on the recent RAN3 agreement, Rel-19 XnAP SN Status Transfer will provide means to transfer this information from the source RAN node to the target RAN node;

B.	The UE transmitted a PDCP SN Gap report, but its RLC transmissions did not successfully complete before the layer was flushed by the RRC command:

-	In this case, there is no information to be transferred from the source RAN node to the target RAN node.

In the latter case B, a more detailed sequence can be as follows:

0.	All PDCP SDUs before SDU #X have been successfully delivered from UE to the network;

1.	SDU #X is discarded, and PDCP SN Gap report is triggered and transmitted by RLC, together with some SDUs from #(X+1) onwards;

2.	UE receives and executes an RRC command for handover, including RLC re-establishment for the DRB;

-	The RLC re-establishment takes place before successful delivery of any PDCP PDU transmitted in step 1;

3.	In line with the above, XnAP SN Status transfer from source RAN node to target RAN node indicates #X as the First missing UL SDU (and no bitmap indicating higher-numbered received SDUs);

4.	After handover completion by the UE (and the related delay), as part of PDCP Re-establishment, UE PDCP retransmits the SDUs from #(X+1) onwards. At this point, reception of these SDUs while SDU #X remains missing starts t-Reordering at the target RAN node, which will run until expiry before the received SDUs are delivered to upper layers.

It can be observed that t-Reordering runs unnecessarily in step 4, only because the RAN is missing the PDCP-SN-gap information.

Observation 3: Handover-related retransmissions by UE PDCP, of SDUs following a discarding gap, can unnecessarily start t-Reordering at the handover-target RAN node if the RAN is missing the related PDCP-SN-gap information.

Proposal 7: RAN2 agree that unnecessary starting of t-Reordering at PDCP-mobility target RAN node because of PDCP-SN-gap information missing in the RAN is a problem to be solved in Rel.19.

The only solution direction that also solves case B seems to involve the UE retransmitting PDCP SN Gap information to the target RAN node.

Observation 4: The UE PDCP may have transmitted a PDCP SN Gap report just before a PDCP-mobility event that interrupts the report’s RLC transmission. The only way to recover the report’s information seems to be retransmission by the UE PDCP, to the target RAN node.

A simple fix seems to be that, as part of PDCP Re-establishment (which is always invoked at any PDCP termination-point change), the UE re-transmit any previously transmitted PDCP SN Gap report(s) for which no lower-layer confirmation of successful delivery has been received.

Proposal 8: as part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

# 3	Discussions on other open issues

In this section we discuss other identified issues that require handling within the RLC related enhancements in Release 19. The list of the issues are as follows:

- (RLC-A) FFS how to minimize timer-based retransmissions by enabling more frequent provisioning of ACKs without the cost of premature NACKs.

- (RLC-B) FFS whether/how timer-based retransmissions are reported in BSR.

- (RLC-C) FFS how the transmitting PDCP reliably knows how high-numbered PDCP SDUs it can proceed to transmit, when RLC ACK no longer necessarily indicates successful delivery

## 3.1	RLC-A: enabling more frequent ACKs

The main concerns expressed with timer-based retransmissions had to do with capacity implications. However, these retransmissions can be kept to a minimum provided that RLC ACKs are provided rapidly. What makes the current RLC Status PDU difficult for rapid acknowledgment is that, among the SDUs that are acknowledged, all the other SDUs are negatively acknowledged. The t-Reassembly timer avoids such premature NACKs but also holds back the ACKs.

Observation 5: among the SDUs that are acknowledged in a RLC Status PDU, all the other SDUs are always negatively acknowledged.

Since RAN2 agreed to focus specifically on timely uplink retransmissions, a complete solution to this problem would be to make it configurable whether the UE considers SDUs for retransmission when NACKed. A UE that would not, would allow the network to send RLC ACKs rapidly without any concerns about premature NACKs.

Proposal 9: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Another mechanism that may hold back RLC ACKs is LCP. Currently in LCP, there is no consideration regarding prioritizing control PDUs such as RLC Status PDU. The RLC Status PDU is prioritized over data PDU only within the same bearer/LCH when the LCH is treated in LCP. As a result, during LCP, the LCHs with RLC Status PDU awaiting transmission, may end up not getting enough allocation to put such PDUs in the MAC PDU being generated for transmission, delaying transmission of these PDUs.

Proposal 10: RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

## 3.2	RLC-B: timer-based retransmissions and BSR

With autonomous RLC retransmissions in place comes the question whether they should be reflected in BSR. If they are, the gNB is not able to distinguish the data volume for initial transmissions from the data volume for retransmissions and as a result, may prioritise the wrong UE. To avoid that, either autonomous retransmission data is not included in data volumes, or it is reported separately.

Proposal 11: autonomous retransmission data is either not included in data volumes, or is reported separately.

## 3.3	RLC-C: PDCP’s transmitting window

By previous RAN2 agreements and the current running CR agreement #4 in RAN2 #128, the new RLC timer t-RxDiscard can be started by a received SDU segment not yet delivered to PDCP. It follows that for an unreceived RLC SDU, t-RxDiscard the new RLC timer may start and expire earlier than PDCP’s t-Reordering, and consequently that the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU. This means that RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

Observation 6: accounting for possible RLC SDU segments, for an unreceived RLC SDU, the new RLC timer t-RxDiscard may expire earlier than PDCP’s t-Reordering, and consequently the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU.

Observation 6a: RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

As a replacement, regular Status reporting would seem to be needed at PDCP.

Proposal 12: to allow the transmitting PDCP to reliably know how high-numbered PDCP SDUs it can proceed to transmit, the agreed RLC ACK no longer indicating successful delivery is combined with regular PDCP status reporting.

# 4	Conclusion

This document has made the following observations:

Observation 1: Consistent discarding of SDUs by Rel-19 RLC AM may be due to a drop in the sustainable bitrate, but not due to failure of the radio link.

Observation 2:	It seems important for the UE to trigger RRC Re-establishment when it is unreachable in the downlink and its only active radio bearer is a DRB carrying XR traffic.

Observation 3: Handover-related retransmissions by UE PDCP, of SDUs following a discarding gap, can unnecessarily start t-Reordering at the handover-target RAN node if the RAN is missing the related PDCP-SN-gap information.

Observation 4: The UE PDCP may have transmitted a PDCP SN Gap report just before a PDCP-mobility event that interrupts the report’s RLC transmission. The only way to recover the report’s information seems to be retransmission by the UE PDCP, to the target RAN node.

Observation 5: among the SDUs that are acknowledged in a RLC Status PDU, all the other SDUs are always negatively acknowledged.

Observation 6: accounting for possible RLC SDU segments, for an unreceived RLC SDU, the new RLC timer t-RxDiscard may expire earlier than PDCP’s t-Reordering, and consequently the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU.

Observation 6a: RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

And proposed the following:

Proposal 1 (RLC-6): Rel-19 RLC AM does not indicate failure to RRC based on consistent discarding of SDUs.

Proposal 2 (RLC-6): t-PollRetransmit expiry increments a newly introduced counter that keeps track of the number of consecutive expiries of t-PollRetransmit without t-PollRetransmit being stopped. Once the new counter reaches its own configured threshold value, RLC provides to RRC the same indication as it currently does upon RETX_COUNT reaching its threshold value. An RLC text proposal for implementing this proposal has been provided in the annex

Proposal 3 (RLC-10): No Spec impact for constraining the mutual configuration of t-RxDiscard and stopReTxObsoleteSDU is needed.

Proposal 4 (RLC-11):	RAN2 agree that status reporting alone cannot be relied on to prevent window stalling, considering the case where the last SDU transmitted, or a number of them, is lost at MAC and therefore the receiver cannot detect the loss.

Proposal 5 (RLC-11):	If the transmitting window is stalled when t-PollRetransmit expires, the poll is re-sent in a retransmitted SDU, even if only SDUs discarded by PDCP are buffered.

Proposal 6 (RLC-11):	When an RLC ACK or a PDCP-discard indication for an SDU is received, if the transmitting window is not stalled, t-PollRetransmit is stopped if, after the reception, all SDUs previously submitted to lower layer have been either ACKed or their transmissions have been stopped due to discard indication from PDCP.

Proposal 7: RAN2 agree that unnecessary starting of t-Reordering at PDCP-mobility target RAN node because of PDCP-SN-gap information missing in the RAN is a problem to be solved in Rel.19.

Proposal 8: as part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

Proposal 9: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Proposal 10: RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

Proposal 11: autonomous retransmission data is either not included in data volumes, or is reported separately.

Proposal 12: to allow the transmitting PDCP to reliably know how high-numbered PDCP SDUs it can proceed to transmit, the agreed RLC ACK no longer indicating successful delivery is combined with regular PDCP status reporting.

# Annex: Text proposal for Proposal 2 (RLC-6)

#### 5.3.3.3	Reception of a STATUS report

Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall:

-	if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN:

-	set POLL_RETX_COUNT to 0;

-	if t-PollRetransmit is running:

-	stop and reset t-PollRetransmit.

#### 5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	increment the POLL_RETX_COUNT;

-	if POLL_RETX_COUNT = maxPollRetxThreshold:

-	indicate to upper layers that max retransmission has been reached;

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in clause 5.3.3.2.

[…]

## 7.1	State variables

This sub clause describes the state variables used in AM and UM entities in order to specify the RLC protocol. The state variables defined in this clause are normative.

All state variables and all counters are non-negative integers.

All state variables related to AM data transfer can take values from 0 to 4095 for 12 bit SN or from 0 to 262143 for 18 bit SN. All arithmetic operations contained in the present document on state variables related to AM data transfer are affected by the AM modulus (i.e. final value = [value from arithmetic operation] modulo 4096 for 12 bit SN and 262144 for 18 bit SN).

All state variables related to UM data transfer can take values from 0 to 63 for 6 bit SN or from 0 to 4095 for 12 bit SN. All arithmetic operations contained in the present document on state variables related to UM data transfer are affected by the UM modulus (i.e. final value = [value from arithmetic operation] modulo 64 for 6 bit SN and 4096 for 12 bit SN).

When performing arithmetic comparisons of state variables or SN values, a modulus base shall be used.

TX_Next_Ack and RX_Next shall be assumed as the modulus base at the transmitting side and receiving side of an AM RLC entity, respectively. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. RX_Next <= SN < RX_Next + AM_Window_Size is evaluated as [RX_Next – RX_Next] modulo 2[sn-FieldLength] <= [SN – RX_Next] modulo 2[sn-FieldLength] < [RX_Next + AM_Window_Size – RX_Next] modulo 2[sn-FieldLength]), where sn-FieldLength is 12 or 18 for 12 bit SN and 18 bit SN, respectively.

RX_Next_Highest– UM_Window_Size shall be assumed as the modulus base at the receiving UM RLC entity. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. (RX_Next_Highest– UM_Window_Size) <= SN < RX_Next_Highest is evaluated as [(RX_Next_Highest– UM_Window_Size) – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] <= [SN – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] < [RX_Next_Highest– (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength]), where sn-FieldLength is 6 or 12 for 6 bit SN and 12 bit SN, respectively.

The transmitting side of each AM RLC entity shall maintain the following state variables:

a) TX_Next_Ack – Acknowledgement state variable

This state variable holds the value of the SN of the next RLC SDU for which a positive acknowledgment is to be received in-sequence, and it serves as the lower edge of the transmitting window. It is initially set to 0, and is updated whenever the AM RLC entity receives a positive acknowledgment for an RLC SDU with SN = TX_Next_Ack.

b) TX_Next – Send state variable

This state variable holds the value of the SN to be assigned for the next newly generated AMD PDU. It is initially set to 0, and is updated whenever the AM RLC entity constructs an AMD PDU with SN = TX_Next and contains an RLC SDU or the last segment of a RLC SDU.

c) POLL_SN – Poll send state variable

This state variable holds the value of the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer when POLL_SN is set according to sub clause 5.3.3.2. It is initially set to 0.

The transmitting side of each AM RLC entity shall maintain the following counters:

a) PDU_WITHOUT_POLL – Counter

This counter is initially set to 0. It counts the number of AMD PDUs sent since the most recent poll bit was transmitted.

b) BYTE_WITHOUT_POLL – Counter

This counter is initially set to 0. It counts the number of data bytes sent since the most recent poll bit was transmitted.

c) RETX_COUNT – Counter

This counter counts the number of retransmissions of an RLC SDU or RLC SDU segment (see clause 5.3.2). There is one RETX_COUNT counter maintained per RLC SDU.

d) POLL_RETX_COUNT – Counter

This counter is initially set to 0. It counts the number of retransmissions of a poll (see clause 5.3.3).

The receiving side of each AM RLC entity shall maintain the following state variables:

a) RX_Next – Receive state variable

This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

b) RX_Next_Status_Trigger – t-Reassembly state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Reassembly.

c) RX_Highest_Status – Maximum STATUS transmit state variable

This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

d) RX_Next_Highest – Highest received state variable

This state variable holds the value of the SN following the SN of the RLC SDU with the highest SN among received RLC SDUs. It is initially set to 0.

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next – UM send state variable

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables:

a) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0. For groupcast and broadcast of NR sidelink communication, it is initially set to the SN of the first received UMD PDU containing an SN.

b) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

c) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0. For groupcast and broadcast of NR sidelink communication, it is initially set to the SN of the first received UMD PDU containing an SN.

[…]

## 7.4	Configurable parameters

The following parameters are configured by TS 38.331 [5]:

a) maxRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions corresponding to an RLC SDU, including its segments (see clause 5.3.2).

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see clause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see clause 5.3.3).

d) maxPollRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions of a poll (see clause 5.3.3).