3GPP TSG-RAN WG2 Meeting #131	R2-2505336

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.4.2

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_IDLE INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2 meeting, we had the following agreement:

RAN2 aim at supporting enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, if the solution can be concluded in the August meeting.

And during post email discussion [Post130][222][LP-WUS], we discussed potential solution and made the following proposals:

Observation (9/15): Many prefer to specify either NAS or RRC signalling to support enabling/disabling LP-WUS per UE.

Proposal: RAN2 to down-select from NAS signalling or RRC signalling to support enabling/disabling LP-WUS per UE.

In this contribution, we further discuss this open issue (RRC-12) dedicated signalling to enable/disable LP-WUS.

- 2 Discussion

- 2.1 Dedicated signalling to enable/disable LP-WUS (RRC-12)

Why we need dedicated signalling?

Enable/disable of LP-WUS monitoring means the NW can indicate LP-WUS capable UE whether needs to apply LP-WUS feature or not when the UE is related to RRC_IDLE/INACTIVE. If without dedicated enable/disable indication, all LP-WUS supporting UEs need to apply LP-WUS feature (i.e., monitoring LP-WUS when entry condition is fulfilled) as long as the NW is broadcasting LP-WUS configuration in system information, this is not ideal and there are following reasons to support dedicated enable/disable indication:

1. Paging false alarm

Some companies think LP-WUS has already supported 32 group information, but noted that RAN1 agreed that one LO can be mapped up to 4 POs in one paging DRX cycle, which means for each PO, only 8 group information (including one common group) is available, therefore, paging false alarm issue still can be considered.

When large amount of UEs are monitoring LP-WUS, once one UE is paged, unnecessary LP-WUS reception to other UEs is increased. Therefore if NW can control part of UEs (instead of all LP-WUS supporting UEs) monitor LP-WUS and other part of UEs do not monitor LP-WUS, it can provide flexibility and avoid potential serious paging false alarm issue.

2. Latency requirement

For LP-WUS monitoring UE, the paging delay is increased compared with normal paging monitoring UE when the paging is initiated after LO and before PO. If NW want one specific LP-WUS capable UE not to enter LP-WUS monitoring (e.g., considering latency), the NW can send this dedicated enable/disable indication to this UE, which provides flexibility and avoids latency issue.

3. Power consumption

The purpose of introducing LP-WUS is for power saving, but if the transition between MR and LR is frequent for some specific UEs, the power saving could be overkilled. For example, if the paging probability is relatively high, the UE better not to monitor LP-WUS even though this UE supports LP-WUS feature, otherwise, every time when this UE is monitoring LP-WUS, it will wake up its MR soon, which cannot be acceptable. For such UEs, it could just directly monitor PEI/PO. Therefore, NW can, based on implementation (e.g., information from paging probability), to determine whether such kind of UEs need to apply LP-WUS or not.

Observation-1: considering the issue on false alarm, latency and power consumption, it is promising to support dedicated signalling for enable/disable of LP-WUS when the UE is released to RRC_IDLE/INACTIVE.

What potential solution we can have?

In [Post130][222][LP-WUS], RAN2 identified the following solutions as way forward:

Solution-1: NAS solution

Solution-2: RRC solution

Solution-3: based on the comparison between I-DRX cycle and LP-WUS offset (CATT solution)

Solution-4: based on registration for CN-based and lp-SubgroupsNumForUEID for UE-ID-based (Apple solution)

For solution-3 (CATT solution), latency requirement is reflected by paging DRX cycle, if the UE “used” paging cycle is larger than LP-WUS offset, the UE is not allowed to apply LP-WUS. In fact this kind of comparison is not clear, in RAN1, the UE wake-up capability is defined as 70ms, 500ms, 900ms, to fit LP-WUS offset, the NW broadcasted LP-WUS offset should be no less than the UE capability. The “70ms” implies the UE can not enter ultra-deep-sleep which is not a normal case, in order to achieve power saving, normally the UE wake-up should be 500ms. On the other hand, according to RRC running CR, the range of lpwus-LoFrameOffsetList is [80ms – 2000ms], it is clear that basically the LO offset should be larger than paging DRX cycle (e.g., 320ms, 640ms) for most of cases. Therefore we don’t think this solution is appropriate.

For solution-4 (Apple solution), two methods are used to disable CN-based LP-WUS and UE-ID-based LP-WUS respectively, for CN-based LP-WUS, CN can indicate the absence of CN-based subgroup ID to disable CN-based LP-WUS while for UE-ID-based LP-WUS, gNB may not broadcast lp-SubgroupsNumForUEID to let all UEs in the cell know UE-ID-based LP-WUS is disabled. In our understanding, what we need is to provide a dedicated signalling to enable/disable one specific UE instead of all UEs in the cell, therefore, this solution is not efficient.

For RRC solution, the procedure is shown as fig.1 and enhancement part is highlighted by red color:

Fig.1 RRC solution

In this solution, if gNB make the decision that the LP-WUS feature should be disabled for one specific UE, RRCRelease can be used to inform the UE of LP-WUS status, besides, gNB also need to indicate this LP-WUS status to CN so that the CN is aware of LP-WUS status when initiating CN paging. For paging procedure/exchange, if LP-WUS is disabled, the NW (including both gNB and CN) could exclude LP-WUS capability within UE Radio Capability for Paging of NR when CN/RAN paging is initiated. In this way, misalignment between UE and NW is avoided and dedicated enable/disable is achieved.

For NAS solution, the procedure is shown as Fig.2 and enhancement is highlighted by red color

Fig.2 NAS solution

In this solution, the UE should first indicate its support of LP-WUS within registration request, if CN make decision that the LP-WUS feature should be disabled for one specific UE, the disable indication can be included within registration accept, besides, the CN should also indicate this LP-WUS status to gNB (normally, Core Network Assistance Information for RRC INACTIVE is reused) so that the gNB is aware of LP-WUS status when initiating RAN paging. For paging procedure/exchange, if LP-WUS is disabled, the NW (including both gNB and CN) could exclude LP-WUS capability within UE Radio Capability for Paging of NR when CN/RAN paging is initiated. In this way, misalignment between UE and NW is avoided and dedicated enable/disable is achieved.

In summary, both RRC and NAS solution can provide dedicated signalling to enable/disable LP-WUS feature and in our understanding, both can work with minor spec impact even though other WGs are involved. But on the other hand, adopting one solution is sufficient to solve this problem, supporting both solution is not necessary and make this procedure a little complicated/redundant. Therefore, RAN2 should make down-selection between RRC and NAS.

Currently companies’ view are diverse as both solution can work, but if we need to prioritize one solution, NAS solution is more promising. The reason is that CN has more information in term of paging procedure to make decision. For example, CN is aware of UE paging probability which is corresponding to false alarm and frequent transition issue, furthermore, CN also provides UE with UE-specific paging cycle, and this is related to latency requirement. Therefore, NAS solution can be prioritized.

Proposal-1a (RRC-12): for enable/disable of LP-WUS feature, RAN2 to make down-selection between RRC solution and NAS solution.

Proposal-1b (RRC-12): NAS solution to enable/disable LP-WUS feature can be prioritized.

To make it clearer, enable/disable of LP-WUS feature here does not mean the UE is indicated to monitor LP-WUS or not monitor LP-WUS, in fact, it enables whether the UE can apply this feature or not, but whether the UE can monitor LP-WUS or not in the end should be based on entry/exit condition of LP-WUS monitoring. For example, if LP-WUS is enabled, then the UE is allowed to evaluate entry condition when released to RRC_IDLE/INACTIVE.

- 3 Conclusions

Observation-1: considering the issue on false alarm, latency and power consumption, it is promising to support dedicated signalling for enable/disable of LP-WUS when the UE is released to RRC_IDLE/INACTIVE.

Proposal-1a (RRC-12): for enable/disable of LP-WUS feature, RAN2 to make down-selection between RRC solution and NAS solution.

Proposal-1b (RRC-12): NAS solution to enable/disable LP-WUS feature can be prioritized.

- 4 References