3GPP TSG-RAN WG2 Meeting #131	R2-2505337

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.3.2.1

Source: 	NEC

Title:  	Discussion on UE-side AIML model for RRM measurement prediction

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#129b meeting, the following agreements are made:

- The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

In RAN2#130 meeting, the following agreements are made :

- The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain

  - PW length (for temporal case A)

  - Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional).  MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI

  - Measured and predicted beam pattern (optional).   MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI

  - Measured and predicted frequency (For inter-freq domain prediction)

- Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In this contribution, we further discuss the inference configuration, report content and condition to perform inference.

- 2 Discussion

- 2.1 Inference configuration

Even though this is a UE-side model for RRM measurement prediction, the model related actions, e.g., model inference should be under NW control. As the model inference output, e.g., future measurement results for temporal domain case A, skipped real measurement results for temporal domain case B, neighbour cell measurement results for frequency domain, should be used to evaluate configured event or periodically report which is configured by NW. Therefore how often to perform inference should be related to event evaluation and periodical report, one way is that when to perform inference is up to UE implementation, but UE should make sure it is efficient for event evaluation and periodical report, e.g., it is not necessary to perform inference after every measurement instance for event evaluation, or the UE should perform inference at least once for each periodical report. Another way is that the NW could explicitly configure inference periodicity to UE, to instruct how often the UE should perform inference, so that the NW could expect how often that the UE should evaluate event, as for periodical report type, for temporal domain case A, it is reasonable to configure the same value for both inference and report periodicity, but for temporal domain case B, inference periodicity and report periodicity may not be same, so that NW could collect multiple of inference outputs at one time. Therefore inference periodicity should also be considered within inference configuration, to instruct UE how to perform inference.

Proposal-1: inference configuration should include inference periodicity.

In RAN2#129b meeting, RAN2 agreed that “The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework”, therefore, we further discuss how to enhance the legacy RRM measurement framework (measID / measObject / reportConfig) to fit for AIML inference.

For temporal domain case A and B, it has been agreed this is only for intra-cell prediction, i.e., serving cell-1 as input to predict serving-cell-1 while neighbour cell-1 as input to predict neighbour cell-1, so that the AIML model should clearly identify which cell should be used for AIML inference. Based on current framework, measObject is used to instruct how the UE performs measurement. However, within measObject, NW only provides frequency information (i.e., ssbFrequency / refFreqCSI-RS), which means the UE only performs measurement based on frequency information instead of specific cell, the UE could derive multiple cell measurement results under this frequency, it is obvious that not all derived cell measurement results are necessary for AIML inference. Therefore, for temporal domain case A and B, cell information list which is used for AIML inference should be included into measObject.

Proposal-2: for temporal domain case A and B, Cell information list is needed in measObject.

For frequency domain, the UE, based on e.g., intra-frequency cell measurement result (as inference input) to predict e.g., inter-frequency cell measurement results, so that measurement reduction is achieved and measurement gap is avoided. Based on current framework, one set of measConfig can only include one measObject, one reportConfig and one measID, which means for this measurement configuration, the UE only performs measurement for one specific frequency, if AIML inference is used to predict another frequency, the current frequency information is not enough, we need to introduce predicted frequency information for the same measConfig. In our understanding, there are two solutions, one is that introduce another measObject which is to provide predicted frequency information, then two measObject will be linked to the same measConfig simultaneously. Another solution is that within the same measObject, introduce another IE which is to indicate predicted frequency information. Furthermore, for predicted frequency information, if the NW/UE wants to predict a specific cell instead of frequency, cell information can also be added, of which details can be FFS during normative work.

Proposal-3: for frequency domain, beside legacy frequency information within measObject, another frequency information which is used for prediction should also be linked to the same measConfig.

- 2.2 Report content

For temporal domain case A, multiple future measurement results can be predicted by measuring historical measurement results, when it comes to periodical measurement report type, in legacy, the UE only include the latest measurement results into MR for the report. Here, to achieve handover improvement, the NW better to know the future measurement results predicted by UE, MR should be enhanced to report multiple measurement results with time information for one cell.

Proposal-4: for temporal domain case A, multiple future measurement results with time information can be reported in one MR.

For temporal domain case B, the intention is not to improve handover performance, instead, the UE can still report MR as legacy while some measurement reduction is achieved (but keep the similar handover performance). In this case, when the UE initiates MR, the latest measurement result could be real measurement or predicted measurement depends on the time instance to trigger MR. Therefore, to let NW better know the property of measurement result reported by the UE, it is better to introduce an indication within MR to indicate whether the measurement result is predicted or actually measured.

Proposal-5: for temporal domain case B, an indication to indicate whether the measurement result within MR is predicted or actually measured.

- 2.3 Condition to perform inference

In last RAN2#129b meeting, RAN2 has agreed the UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference.

Based on online discussion, we understand that dynamic signalling will not be supported to activate/deactivate inference configuration, instead the UE just follows the inference configuration, but applying inference configuration does not mean the UE immediately performs inference. Here we discuss FFS part, i.e., when the UE can perform inference, in our view, the AIML mobility prediction is mainly used to serve handover, but in some cases, the UE does not have to always perform AIML mobility prediction, for example, if one UE is staying in cell centre (e.g., signal quality is good enough) or the UE is in low mobility/stationary mode, it is not likely to perform handover, there is no need to perform AIML inference for mobility. Therefore, when NW configures UE with inference configuration, some conditions (e.g., cell centre and/or low mobility) can be included in this inference configuration. And the UE applying the inference configuration means once the configured conditions within configuration are met, the UE can start to perform inference.

Fig.1 condition to perform inference

Proposal-6: The UE can be configured to perform inference only when it is at the cell edge and/or high mobility.

In addition to mobility and signal condition, UE resource constraint like battery status, overheating status can also impact running of the AI/ML inference efficiently. Performing inference under low battery or overheating can lead to further draining of power unnecessarily. Thus, it is also beneficial to allow UE to stop inference based on the battery and overheating state.

Proposal-7: The UE can stop the AI/ML inference based on the low battery state, overheating state.

- 3 Conclusions

Proposal-1: inference configuration should include inference periodicity.

Proposal-2: for temporal domain case A and B, Cell information list is needed in measObject.

Proposal-3: for frequency domain, beside legacy frequency information within measObject, another frequency information which is used for prediction should also be linked to the same measConfig.

Proposal-4: for temporal domain case A, multiple future measurement results with time information can be reported in one MR.

Proposal-5: for temporal domain case B, an indication to indicate whether the measurement result within MR is predicted or actually measured.

Proposal-6: The UE can be configured to perform inference only when it is at the cell edge and/or high mobility.

Proposal-7: The UE can stop the AI/ML inference based on the low battery state, overheating state.

- 4 References