3GPP TSG-RAN WG2 Meeting #131	R2-2505340

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.5.4

Source:	vivo

Title:	Remaining issues of common signal adaptation

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issues of common signal adaptation.

Discussion

Paging adaptation in the time domain

In this section, we will discuss the following open issues:

- Signaling overhead optimization of configuring Rel-19 firstPDCCH-MonitoringOccasionOfPO;

- Signaling overhead optimization of configuring Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O;

In the legacy firstPDCCH-MonitoringOccasionOfPO configuration, each PO (first PDCCH MO of the PO) can be configured on any symbol between two adjacent PFs. Thus, the maximum signaling overhead of legacy firstPDCCH-MonitoringOccasionOfPO is 4*17 = 68 bits, given by the configuration shown below.

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

In Rel-19 NES, N = T/32, Ns = 8 are supported. Follow the legacy logical, the following table 1 analyzes the signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO, assuming N = T/32, Ns = 8.

Table 1: signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO

| SCS | Total OFDM symbols per radio frame | Total OFDM symbols across T (T = 32) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (Ns = 8) |
| --- | --- | --- | --- |
| 15KHz | 140 | 4480 (13 bits) | 104 bits |
| 30KHz | 280 | 8960 (14 bits) | 112 bits |
| 60KHz | 560 | 17920 (15 bits) | 120 bits |
| 120KHz | 1120 | 35840 (16 bits) | 128 bits |
| 480KHz | 4480 | 143360 (18 bits) | 144 bits |

It can be observed that the maximum signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO configuration exceeds 2 times that of the legacy. Additionally, it should be noted that both the legacy configuration and the Rel-19 configuration can be configured at the same time, meaning only firstPDCCH-MonitoringOccasionOfPO configuration would occupy 68 + 144 =  212 bits, and the maximum SIB1 size is only 2976 bits. We think such signaling overhead is unacceptable and should be optimized.

TS 38.331 NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

In fact, the value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO corresponding to N = 1/32T does not need to span the whole 32 radio frames, considering the target scenario is for NES. Otherwise, the network can simply configure N = 1/16T or 1/8T etc. Therefore, a solution is to restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO. Table 2 summarized the signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO if different maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO is used.

Practically speaking, the maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO does not need exceed more than 16 radio frames. If the NW intends to configure some POs to exceed more than 16 radio frames, the NW can simply rely on the legacy paging configuration.

Table 2: signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO for different maximum value range

| SCS | Total OFDM symbols per radio frame | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 32 frames) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 16 frames) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 2 frames) |
| --- | --- | --- | --- | --- |
| 15KHz | 140 | 104 bits | 96 bits | 72 bits |
| 30KHz | 280 | 112 bits | 104 bits | 80 bits |
| 60KHz | 560 | 120 bits | 112 bits | 88 bits |
| 120KHz | 1120 | 128 bits | 120 bits | 96 bits |
| 480KHz | 4480 | 144 bits | 136 bits | 112 bits |

Moreover, similar signaling overhead issue exists also for firstPDCCH-MonitoringOccasionOfPEI-O-r17, thus the similar handling of Rel-19 firstPDCCH-MonitoringOccasionOfPO can be considered for firstPDCCH-MonitoringOccasionOfPEI-O.

Proposal 1: For signaling overhead optimization of Rel-19 firstPDCCH-MonitoringOccasionOfPO, restrict the maximum value range of the following parameters not to exceed symbol amount corresponding to 16rf (radio frames) for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N):

- Rel-19 firstPDCCH-MonitoringOccasionOfPO;

- Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O;

RACH-ConfigCommon associating with CE

During last meeting, RAN2 discussed on how to avoid the additional RO applicability to Msg1 repetition feature [1], considering support of CE in RACH adaptation may lead to much RAN1 work and the remaining R19 TU is limited, and determine to down-select from the following options [2]:

1. RAN2 will select one of the following options. (CE here means MSG1 repetition). Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon. Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17).

From our perspective, both options can work and at least need a note to restrict the NW behavior. In comparision, Option-1b allows co-existence with Msg1 repetition and RACH adaptation in one RACH-ConfigCommon, while Option-1c only allows either Msg1 repetition or RACH adaptation in one RACH-ConfigCommon. However, considering a NES cell enabling Msg1 repetition may only have limited resources for additional RO, we do not observe much problem with option-1c and thus it is preferred.

Proposal 2: Capture a note in the specification to clarify featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and it is up to the network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: For signaling overhead optimization of Rel-19 firstPDCCH-MonitoringOccasionOfPO, restrict the maximum value range of the following parameters not to exceed symbol amount corresponding to 16rf (radio frames) for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N):

- Rel-19 firstPDCCH-MonitoringOccasionOfPO;

- Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O;

Proposal 2: Capture a note in the specification to clarify featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and it is up to the network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon.

References

- R2-2504718	[AT130][112][NES] (OPPO)	OPPO

- R2-2504671	Report from session on R18 SL, R18/19 MOB, and R19 NES	Vice Chairman (Samsung)