3GPP TSG-RAN WG2 Meeting #131	R2-2505347

Bangalore, India, Aug. 25th – 29th, 2025

Agenda item:	8.3.2.3

Source:	Samsung

Title:	Discussion on data collection for AI/ML mobility

Document for:	Discussion & Decision

Introduction

This document discusses on data collection for UE-sided model for Rel-19 AI/ML mobility SI.

Discussion

Recap of AI/ML for air interface

In Rel-19 AI/ML for air interface, RAN2 has studied data transfer options (so called Option 1a, 2, and 3) for data collection for UE-sided model (a.k.a. UE side data collection). However, the only option (i.e., Option 1a) where data transfer is performed transparently to 3GPP network (i.e., without specification impact) is assumed in Rel-19 WI. The other standardized transfer options (i.e., Option 2 and 3) are to be further studied and/or specified in Rel-20.

<Rel-20 WID for AI/ML for air interface> Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection

Therefore, in Rel-19, there is no specification impact on data transfer part for UE-side data collection. However, before transferring collected data, UE first needs configuration provided by NW to measure the resource and acquire relevant data for training. So, RAN2 made the following agreements (for beam management use-case, and possibly for CSI prediction use-case):

<RAN2#129bis> - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: - An indication on start/stop of data collection - Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request. <RAN2#130> - The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. - For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling

The procedure for UE to request the configuration could be represented as three steps:

- Step 1) NW configures a list of candidate configurations (via OtherConfig) each of which indicates resources for Set A and B for data collection (i.e., CSI-ResourceConfigId of Set A and CSI-ResourceConfigId of Set B). However, UE does not measure the resources as they are merely candidate configurations yet.

- Step 2) UE indicates its preferred configuration(s) (via UEAssisntaceInformation message) among the received candidates for data collection.

- Step 3) NW configures the configuration(s) for measurement (via CSI-ReportConfig). Then, UE starts measuring according to the configuration and acquiring data for UE-side model training (e.g., L1-RSRP).

Way-forward for AI/ML for mobility

Our discussion for AI/ML for mobility focuses on data collection request and configuration, rather than data transfer as clarified in the agenda.

<Agenda for RAN2#131> ## 8.3 AI/ML for Mobility (FS_NR_AIML_Mob; leading WG: RAN2; REL-19; SID: RP-242393) Time budget: 2 TUs Tdoc Limitation: 4 tdocs #### <…> #### 8.3.2.3 Data collection Aspects related to data collection for UE sided model for RRM measurement prediction and RRM measurement event prediction (excluding those aspects that are under study in AI/ML PHY)

While AI/ML for air interface (i.e., beam management or CSI prediction) follows legacy CSI measurement/reporting framework, AI/ML for mobility works based on legacy RRM measurement framework where the radio resources to measure are given in form of measurement object (MO). Therefore, we think the radio resources to measure for UE-side data collection for AI/ML mobility can be configured in form of MO.

Observation 1. In legacy RRM measurement framework, the radio resources to measure are given in form of measurement object (MO).

Proposal 1.  Radio resources to measure for UE-side data collection can be configured in form of MO.

Then, we would like to discuss on whether UE can re-use MO configured for legacy RRM measurement. In our understanding, it is highly beneficial in that UE can collect data even from legacy NW which does not support AI/ML mobility but provides configuration for legacy RRM measurement. In other words, UE may have a chance to collect data from (almost) all gNBs.

Proposal 2. For UE-side data collection, UE can perform measurement by re-using MOs configured for legacy RRM measurement.

However, relying only on legacy MOs may not be enough to acquire sufficient data, because UE or UE server may want to collect data for different MOs or frequencies or cells other than ones configured for legacy RRM measurement. As NW has no idea on what MOs or frequencies or cells UEs are interested in for UE-side data collection, it is necessary for UE to provide its preference on them. To this end, we propose two options:

As Option 1, we first could adopt a similar three-step approach used in AI/ML for air interface as specified below:

- Step 1) NW configures a list of candidate MOs or frequencies or cells for data collection (e.g., via OtherConfig)

- Step 2) UE indicates its preferred MO(s) or frequencies or cells (e.g., via UEAssisntaceInformation message) among the received candidates for data collection.

- Step 3) NW configures MO(s) for measurement of UE-side data collection. Then, UE starts measuring according to the configuration and acquiring data for model training (e.g., cell level RSRP).

Figure 1. Option 1 for UE-side data collection

However, we do not think UE always should rely on provision of the candidate configurations from NW. i.e., UE itself can provide its preferred frequencies or cells to NW, even if they are not provided from NW as candidate configurations. Nevertheless, we still assume the transmission of such UE’s preference without candidate configurations should be allowed by NW. i.e., only if NW permits, UE is allowed to send the UE’s preference without candidate configurations. So, we propose Option 2:

- Step 1) NW allows transmission of UE’s preference without candidate configuration for data collection (e.g., via OtherConfig)

- Step 2) UE indicates its preferred frequencies or cells (e.g., via UEAssisntaceInformation message)

- Step 3) NW configures MO(s) for measurement of UE-side data collection. Then, UE starts measuring according to the configuration and acquiring data for model training (e.g., cell level RSRP).

Figure 2.  Option 2 for UE-side data collection

Proposal 3. RAN2 to consider two options for UE-side data collection:

In Option 1,

- Step 1) NW configures a list of candidate MOs or frequencies or cells for data collection (e.g., via OtherConfig)

- Step 2) UE indicates its preferred MO(s) or frequencies or cells (e.g., via UEAssisntaceInformation message) among the received candidates for data collection.

- Step 3) NW configures MO(s) for measurement of UE-side data collection. Then, UE starts measuring according to the configuration and acquiring data for model training (e.g., cell level RSRP).

In Option 2,

- Step 1) NW allows transmission of UE’s preference without candidate configuration for data collection (e.g., via OtherConfig)

- Step 2) UE indicates its preferred frequencies or cells (e.g., via UEAssisntaceInformation message)

- Step 3) NW configures MO(s) for measurement of UE-side data collection. Then, UE starts measuring according to the configuration and acquiring data for model training (e.g., cell level RSRP).

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Observation 1. In legacy RRM measurement framework, the radio resources to measure are given in form of measurement object (MO).

Proposal 1.  Radio resources to measure for UE-side data collection can be configured in form of MO.

Proposal 2. For UE-side data collection, UE can perform measurement by re-using MOs configured for legacy RRM measurement.

Proposal 3. RAN2 to consider two options for UE-side data collection:

In Option 1,

- Step 1) NW configures a list of candidate MOs or frequencies or cells for data collection (e.g., via OtherConfig)

- Step 2) UE indicates its preferred MO(s) or frequencies or cells (e.g., via UEAssisntaceInformation message) among the received candidates for data collection.

- Step 3) NW configures MO(s) for measurement of UE-side data collection. Then, UE starts measuring according to the configuration and acquiring data for model training (e.g., cell level RSRP).

In Option 2,

- Step 1) NW allows transmission of UE’s preference without candidate configuration for data collection (e.g., via OtherConfig)

- Step 2) UE indicates its preferred frequencies or cells (e.g., via UEAssisntaceInformation message)

- Step 3) NW configures MO(s) for measurement of UE-side data collection. Then, UE starts measuring according to the configuration and acquiring data for model training (e.g., cell level RSRP).

References