3GPP TSG-RAN WG2 Meeting #131	R2-2505349

Bengaluru, India Aug. 25th – 29th, 2025

Agenda Item:	8.6.4

Source: 	Fujitsu

Title:	Discussion on MAC open issues for Conditional intra-CU LTM

Document for:	Discussion and Decision

- Introduction

At previous RAN2 meetings, some agreements on conditional Intra-CU LTM are reached. However, there are still some essential or important remaining open issues [1].

This contribution will discuss the following issues:

- Coexistence with network triggered LTM cell switch (MAC-14)

- Power ramping considering TA group (MAC-24)

- How to obtain MAC PDU in case of a fallback RACH-based CLTM (MAC-30)

- Discussions

2.1 (MAC-14) Coexistence with network triggered LTM cell switch

RAN2 has agreed that the network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration [2]. So, TA value for a CLTM candidate cell can be provided to the UE via early TAC MAC CE, the LTM Cell Switch Command MAC CE and measured by the UE.

Regarding the TA value, the following cases need to be considered:

- Case 1: early TAC MAC CE is received before LTM Cell Switch Command MAC CE

  - Case 1-1: TA provided by the early TAC MAC CE is still valid while valid TA is included in the LTM Cell Switch Command MAC CE

  - Case 1-2: TA provided by the early TAC MAC CE is still valid, but invalid TA (TA = FFF) is included in the LTM Cell Switch Command MAC CE

  - Case 1-3: TA provided by the early TAC MAC CE is invalid while valid TA is included in the LTM Cell Switch Command MAC CE

  - Case 1-4: TA provided by the early TAC MAC CE is invalid while invalid TA (TA = FFF) is included in the LTM Cell Switch Command MAC CE

- Case 2: UE has no stored TA, and preamble is transmitted but no early TAC MAC CE is received before LTM Cell Switch Command MAC CE

  - For this case, there is no impact to the UE, because the network will not transmit the early TAC MAC CE. For example, source DU can wait for the TA value from target DU and include the valid TA in the LTM Cell Switch Command MAC CE rather than early TAC MAC CE. If invalid TA is included in the LTM Cell Switch Command MAC CE, it is not expected for the UE to receive TA value by early TAC MAC CE.

For Case 1, more discussions are needed. We think that valid TA included in the LTM Cell Switch Command MAC CE should be applied for the target cell considering the network may include the updated TA value. This means that TA in the LTM Cell Switch Command MAC CE is applied for Case 1-1 and Case 1-3. For Case 1-4, both TAs provided by the network are invalid and thus UE measured TA is used or RACH-based LTM is performed.

Case 1-2 is possible. For example, the UE received the TA included in the early TAC MAC CE before receiving LTM Cell Switch Command MAC CE. Considering the TA timer maintained at the network for the target cell, the network may not include the same TA value in the LTM Cell Switch Command MAC CE to avoid potential UL interference. Because the TA timer will be started using the configured value rather than the remaining value of TA timer for the LTM candidate cell when the LTM CSC MAC CE is received. In this case, the UL is in-sync even if the PTAT is running. According to Rel-18 mechanism, if invalid TA is provided in the LTM Cell Switch Command MAC CE, UE measured TA is used or RACH-based LTM is performed. However, if UE measured TA is used, then it means that UE measured TA is prioritized over the TA provided by the network, which is not aligned with Rel-18 principles. If RACH-based LTM is performed, larger interruption is incurred.

Based on this, we propose to use valid TA provided in the early TAC MAC CE if invalid TA is indicated in the LTM Cell Switch Command MAC CE. Whether valid TA in the early TAC MAC CE is prioritized over UE measured TA needs more discussion.

Proposal 1 (MAC-14): When the LTM CSC MAC CE indicates a CLTM candidate and its TA to FFF, the stored TA provided by the early TAC MAC CE can be used if the corresponding early TAT is running.

RAN2 has agreed that the UE maintains the TA value and keeps the early TA timer running for other candidate cells when CLTM cell switch is executed and CLTM TAT for the target cell is not stopped if it is running. The UE starts the PTAT using the remaining time for the CLTM timer. If valid TA provided in the early TAC MAC CE is used for LTM cell switch execution triggered by the network, same UE behaviours will apply.

Proposal 2 (MAC-14): Same UE behaviours will apply to the PTAT, early TA timers for the target cell and candidate cells if the TA provided by the early TAC MAC CE is used for the LTM cell switch triggered by the network.

2.2 (MAC-26) Power ramping considering TA group

An LTM candidate cell can be configured with two TAGs. So, the PDCCH order can trigger the random access procedure for one of TAGs for the LTM candidate cell. It means that a different random access procedure will be indicated if the TAG changes for the LTM candidate cell.  Based on this, it is reasonable to maintain a power ramping counter considering the TAG ID.

According to MAC specification [3], the MAC entity will set the PREAMBLE_POWER_RAMPING_COUNTER to 1 when the PDCCH order indicates preamble initial transmission. There is no issue if the PDCCH order indicating preamble initial transmission is received successfully.

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall: 1> flush the Msg3 buffer; 1> flush the MSGA buffer; 1> set the PREAMBLE_TRANSMISSION_COUNTER to 1; 1> if the Random Access procedure is initiated on a Serving Cell; or 1> if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell and the PDCCH order indicates preamble initial transmission; or 1> if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell, which is different from the cell to which the UE performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission: 2> set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

However, as shown in the figure below, a problem is observed if the PDCCH order indicating preamble initial transmission is not received by the UE. In this case, the UE will consider the latest PDCCH to trigger the random access procedure for preamble re-transmission of the last random access preamble transmission and thus the power ramping counter will not be reset.

Figure: PDCCH order missing

To support early TA acquisition via RACH towards the LTM candidate, we propose to reset the power ramping counter if the TAG is different from the last random access preamble transmission. A straightforward mechanism is that the PDCCH order indicates the TAG information, e.g. TAG ID. Alternatively, an implicit mechanism is used. For example, the UE determines whether the TAG has changed according to the indicated SSB in the PDCCH order. If the SSB is configured as the pathloss reference or the QCL RS for TCI state, then TAG information corresponding to the SSB can be considered as the TAG where the random access procedure is initiated.

Proposal 3 (MAC-26): To support RACH towards a CLTM candidate cell configured with two TAGs, REAMBLE_POWER_RAMPING_COUNTER is set to 1 when TAG changes. FFS on explicit TAG ID in the PDCCH order.

It is obvious that modifications to MAC spec are required if TAG information is considered to handle the power ramping counter, which is not expected for Rel-18 specifications.  So, we propose that these modifications are only applied to Rel-19 LTM.

Proposal 3bis (MAC-26): TAG change will not be considered when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1 for RACH towards an intra-CU LTM candidate cell configured with two TAGs (i.e. no modification to Rel-18 MAC spec).

2.3 (MAC-30) How to obtain MAC PDU in case of a fallback RACH-based CLTM

To complete the RACH-less LTM cell switch, RRC provides RRC reconfiguration complete message to MAC as the first UL data to target cell. After CG occasion is determined, MAC PDU including the RRC message is obtained from multiplexing and assembly entity for a HARQ process. When the random access procedure is initiated for the LTM cell switch, the HARQ entity for MSG3 transmission cannot obtain the MAC PDU including RRC reconfiguration complete message since it has been obtained by the HARQ entity for the HARQ process during the RACH-less LTM cell switch. In other words, in case of a fallback RACH-based LTM cell switch, there is no MAC PDU including RRC reconfiguration complete message in multiplexing and assembly entity.

Observation: There is no MAC PDU including RRC reconfiguration complete message in multiplexing and assembly entity in case of a fallback RACH-based LTM cell switch, since it has been obtained by the HARQ entity for the HARQ process during the RACH-less LTM cell switch.

A possible way to solve the issue is that the obtained MAC PDU will be included in multiplexing and assembly entity first and then obtained for subsequent transmission. This mechanism is already used to solve grant size mismatch issue, as captured below [3].

3> else if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a MAC RAR; or 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 4> if the uplink grant size does not match with size of the obtained MAC PDU; and 4> if the Random Access procedure was successfully completed upon receiving the uplink grant: 5> indicate to the Multiplexing and assembly entity to include MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in the subsequent uplink transmission; 5> obtain the MAC PDU to transmit from the Multiplexing and assembly entity.

Alternatively, MAC indicates this to RRC and thus RRC provides RRC reconfiguration complete message to MAC for RACH-based LTM. However, it requires further discussions on potential RRC impacts.

We propose to apply a same mechanism as in case of UL grant size mismatch to solve the abovementioned issue in a fallback RACH-based CLTM cell switch.

Proposal 4 (MAC-30): MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in RACH-less CLTM cell switch procedure will be included in the Multiplexing and assembly entity for the uplink transmission in a fallback RACH-based CLTM cell switch procedure.

- Conclusion

This document discussed the remaining MAC open issues for Intra-CU CLTM and made the following proposals:

MAC-14: Coexistence with network triggered LTM cell switch

Proposal 1 (MAC-14): When the LTM CSC MAC CE indicates a CLTM candidate and its TA to FFF, the stored TA provided by the early TAC MAC CE can be used if the corresponding early TAT is running.

Proposal 2 (MAC-14): Same UE behaviours will apply to the PTAT, early TA timers for the target cell and candidate cells if the TA provided by the early TAC MAC CE is used for the LTM cell switch triggered by the network.

MAC-26: Power ramping considering TA group

Proposal 3 (MAC-26): To support RACH towards a CLTM candidate cell configured with two TAGs, REAMBLE_POWER_RAMPING_COUNTER is set to 1 when TAG changes. FFS on explicit TAG ID in the PDCCH order.

Proposal 3bis (MAC-26): TAG change will not be considered when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1 for RACH towards an intra-CU LTM candidate cell configured with two TAGs (i.e. no modification to Rel-18 MAC spec).

MAC-30: How to obtain MAC PDU in case of a fallback RACH-based CLTM

Proposal 4 (MAC-30): MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in RACH-less CLTM cell switch procedure will be included in the Multiplexing and assembly entity for the uplink transmission in a fallback RACH-based CLTM cell switch procedure.

# Reference

- R2-2505398, “Discussion summary and list of MAC open issue for Mob Ph4”, vivo, RAN2#131

- R2-2502981, “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung), RAN2#129bis

- 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"