3GPP TSG-RAN WG2 Meeting #131                                   R2-2505355

Bangalore, India,  Aug 25th -29th, 2025

Agenda item:		8.1.2.3

Source:				Samsung

Title:					Remaining issues on LCM for POS use case

Document for:		Discussion & Decision

1	Introduction

This contribution provides our view on the remaining open issues on LCM for POS according to open issue list from the rapporteurs.

2	Discussion

LPP-6a: Number of PRUs in NR-PRU-DL-Info

In RAN2#130, RAN2 agreed that IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1 as below.

[LPP-6] The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed. FFS if more PRUs are needed to be included for training purposes.

From our understanding, the background of having IE NR-PRU-DL-Info also for NR AI/ML positioning Case 1 is to use the information (i.e., measurement information and location information from PRU) as label data for training. More specifically, the measurement information (i.e., nr-PRU-DL-XXX-MeasInfo) can be the input part and the location information (i.e., nr-PRU-LocationInfo) can be the output part for the label data.

Observation. 1: IE NR-PRU-DL-Info can be used by UE as training data for Case 1, as it includes both measurement information and location information from PRU, which can be used as training data sample.

In the current LPP CR implementation, LMF can include only one nr-PRU-DL-Info (i.e., a single training data sample) in IE NR-DL-AIML-ProvideAssistanceData for NR DL AI/ML positioning method. However, for model training, we usually need a large number of training data samples with variations (e.g., from different PRU or at different times). Even though the LMF can provide multiple training data sample by sending multiple LPP ProvideAssistanceData messages under the current implementation, this approach appears highly inefficient.

Observation. 2: For model training, a large number of training data samples with variations (e.g., from different PRUs or at different timing) are required.

Observation. 3: With the current LPP CR implementation, LMF can include only one nr-PRU-DL-Info (i.e., a single training data sample) in IE NR-DL-AIML-ProvideAssistanceData for NR DL AI/ML positioning method

To enable efficient delivery of training data from the LMF to the UE, we can consider providing training data samples in two distinct dimensions:

- Multiple samples from different PRUs: To include multiple samples collected from various PRUs, the LMF can incorporate more than one (list of) NR-PRU-DL-Info IEs within a single LPP ProvideAssistanceData message.

- Multiple samples from the same PRU at different times: To include multiple samples collected at different times from the same PRU, each NR-PRU-DL-Info IE can contain multiple sets of measurement and location information.

Proposal. 1: To provide training data samples from different PRUs, LMF can include more than one NR-PRU-DL-Info IEs in NR-DL-AIML-ProvideAssistanceData IE (in LPP ProvideAssistanceData message).

Proposal. 2: To provide training data samples at different times from the same PRU, multiple sets of measurement and location information can be included within each NR-PRU-DL-Info IE.

From LMF’s perspective, without any explicit indication from UE, it’s hard to determine whether the target UE requires the training data samples. Given the substantial size of training data, an explicit request from the UE appears essential for efficient delivery of such data from the LMF to the UE.

Observation. 4: For efficient training data delivery, an explicit indication for UE to request the training data samples from LMF is needed.

Proposal. 3: UE can request training data samples for NR DL AIML positioning (e.g., multiple NR-PRU-DL-Info IEs) via LPP RequestAssistanceData message.

LPP-10a: Applicability of "batch reporting" for AI/ML positioning.

At RAN2#130, several companies provide their view on whether "Batch reporting"--reporting of up to 32 location results in a single report--is applicable to "NR AI/ML Positioning Case 1". However, due to the lack of time, the issue was not discussed.

As outlined in the discussion on “Proposal. 3” above, the training data samples collected at different times from the same UE may be required for model training. To facilitate this, LMF may need to collect the multiple training data samples collected at various time instances from UE. For that, we can reuse the existing batch reporting mechanism for AI/ML positioning.

Observation. 5: For training data collection, the existing batch reporting mechanism can be reused to allow UE to report multiple training data samples collected at different time instances to LMF.

Proposal. 4: Support batch reporting for NR DL AI/ML positioning.

LPP-21: "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19)

At the last RAN1 meeting, RAN1 made the following agreement on the use of associated ID.

Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

According to the rapporteur’s summary on [POST130][025], the following issues have been discovered regarding the use of “Associated ID”.

(1) Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell?

The associated ID is introduced to indicate the consistency of TRP location between training and inference. Considering this motivation, even though the related RAN1 agreement says “an associated ID is configured per-cell”, it seems reasonable to extend its applicability to PRS-only TPs. Since a TP not associated with a cell can be identified by its dl-PRS-ID, the associated ID for the PRS-only TP coordinates can be linked to the dl-PRS-ID for that TP.

Proposal. 5: "Associated ID" for PRS-only TP coordinates can be associated with dl-PRS-ID of that TP.

According to RAN1 agreement, the restriction that TRPs belonging to the same cell should have the same value of associated ID applies only to TRPs associated with a cell. Thus, we can just capture that restriction in the field description of nr-AIML-AssociatedID.

Proposal. 6: Capture the restriction on the value of nr-AIML-AssociatedID as a note in the field description as below.

nr-AIML-AssociatedID This field provides an identity associated with the coordinates of the indicated TRP and coordinates of all the antenna reference points of this TRP. The value of the nr-AIML-AssociatedID is changed if/when the coordinates of the TRP and/or coordinates of any of the antenna reference points of this TRP is changed. NOTE : UE is not expected to receive different values of associated ID for TRPs belonging to the same cell.

(2) Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs?

The associated ID is introduced to implicitly indicate the consistency of Info #7 by RAN1. According to the definition of Info #7 from RAN1, it includes locations for transmitting antennas of the TRPs as below.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

Proposal. 7: RAN2 confirms that "Associated ID" is not only for the TRP location but also for the ARP location of the associated DL-PRS Resource Set/DL-PRS Resource.

(3) Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"?

It is understood that the associated ID for a certain cell can be updated when the information corresponding to Info #7 (e.g., TRP/ARP coordinate) changes for that cell. Thus, the associated ID can be used as a kind of version number of a certain cell (e.g., version of TRP location or antenna setting). Given that the information corresponding to Info #7 rarely changes, the value range of 0..255 appears sufficient for this purpose.

Proposal. 8: RAN2 confirms that the value range of 0..255 for the "Associated ID" is sufficient.

(4) Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both?

In our view, there are scenarios where LMF can provide the explicit TRP coordinate and the implicit TRP coordinates simultaneously. For instance, if UE-side AI model is trained to estimate UE location as like the legacy DL-TDOA method, the explicit TRP coordinate information can serve as input data for the model training and inference. In such case, the LMF can include both the explicit TRP location and the associated ID for that TRP together in the initial ProvideAssistanceData message. Subsequently, in the following ProvideAssistanceData messages, the LMF can provide only the associated ID to indicate the consistency of TRP location.

Proposal. 9: RAN2 confirms that LMF can provide both the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) together.

(5) If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (nr-TRP-ImplicitLocationInfo-r19) (and vice versa)?

The implicit location information (i.e., associated ID) is a new information introduced in Rel-19 to support AI/ML positioning. Meanwhile, the explicit location information is old information introduced from Rel-16 to support UE-based NR RAT-dependent positioning methods (e.g., NR DL-TDOA, NR DL-AoD). Therefore, the UE supporting the explicit location information may not necessarily support implicit location information. For AI/ML positioning Case 1, the explicit location may not be required if the model is trained without explicit TRP location, similar to methods like “footpriting localization”. Therefore, a UE supporting AI/ML positioning can support only implicit location info.

Proposal. 10: UE can support explicit TRP location info (NR-TRP-LocationInfo-r16) and implicit location info (nr-TRP-ImplicitLocationInfo-r19) independently of each other.

LPP-18: Consistency between training and inference

For Case 1 positioning, the UE can have the model trained within a certain area A for AI positioning. Specifically, the model can be trained based on the measurement results of DL-PRS from the set of TRPs (e.g., TRP 1, TRP 2, TRP 3, TRP 4, TRP 5) within the area A. Now let’s assume that the UE receives the assistance data for inference in step 3, but the assistance data (i.e., DL-PRS information) is for the TRPs (e.g., TRP 1’, TRP 2’ TRP 3’ TRP 4’ TRP 5’) different from the ones used in training phase. In this case, we can consider two different scenarios for the reason of different TRPs in the assistance data.

In the first scenario, we can assume that the UE moves to a new area B and thus LMF provides the assistance data for the new TRPs within that area. In this case, it is reasonable for the UE to train/download a new model trained within the new area for the consistency between training and inference.

For the second case, the LMF may dynamically turn on/off the DL-PRS transmission from TRPs while the UE is still located within the area A. For example, the LMF turns off the DL-PRS Tx from the TRPs used in training phase and turn on the DL-PRS Tx from the new TRPs with some reason and provide the assistance data for the new TRPs. In this case, the UE can request LMF to provide the assistance data for a certain set of TRPs used in training phase and the existing On-demand DL-PRS request can be reused for this.

At RAN2#130, the following agreement was made to ensure the consistency between training and inference assuming the second case above.

To ensure the consistency between training and inference, the UE should be able to request assistance data associated with a specific group of TRPs. FFS the request associated information in on demand prs request.

To request the assistance data (e.g., DL-PRS configuration) for a specific group of TRPs, UE can report a list of TRP information to identify a specific group of TRPs via LPP RequestAssistanceData. For the TRP information, we can use a combination of dl-PRS-ID, NCGI, PCI, ARFCN to identify a certain TRP as in the current LPP.

Proposal. 11: To ensure consistency between training and inference, UE can request the assistance data (e.g., DL-PRS configuration) for a specific group of TRPs. I.e., UE can report a list of TRP information (i.e., combination of dl-PRS-ID, NCGI, PCI, ARFCN) to identify the group of TRPs for which the assistance data is requested.

Meanwhile, there is another aspect to consider for ensuring consistency between training and inference. As discussed above in LPP-21, the associated ID is introduced to implicitly indicate the consistency of Info #7 (i.e., TRP coordinate, ARP coordinate). Even when the UE receives the assistance data for the same group of TRPs used during training phase, consistency cannot be guaranteed if the TRP location and/or the ARP location (DL-PRS Resource Set/DL-PRS Resource ARPs) of those TRP has changed. it’s important to note that the ARP location may change frequently, as it’s a kind of beam configuration of DL-PRS. Therefore, to ensure the consistency, UE should be able to report the associated ID used during training phase for each TRP when requesting assistance data.

Proposal. 12: To ensure consistency between training and inference, UE can report the associated ID value used during training phase for each TRP.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

LPP-6a: Number of PRUs in NR-PRU-DL-Info

Observation. 1: IE NR-PRU-DL-Info can be used by UE as training data for Case 1, as it includes both measurement information and location information from PRU, which can be used as training data sample.

Observation. 2: For model training, a large number of training data samples with variations (e.g., from different PRUs or at different timing) are required.

Observation. 3: With the current LPP CR implementation, LMF can include only one nr-PRU-DL-Info (i.e., a single training data sample) in IE NR-DL-AIML-ProvideAssistanceData for NR DL AI/ML positioning method

Proposal. 1: To provide training data samples from different PRUs, LMF can include more than one NR-PRU-DL-Info IEs in NR-DL-AIML-ProvideAssistanceData IE (in LPP ProvideAssistanceData message).

Proposal. 2: To provide training data samples at different times from the same PRU, multiple sets of measurement and location information can be included within each NR-PRU-DL-Info IE.

Observation. 4: For efficient training data delivery, an explicit indication for UE to request the training data samples from LMF is needed.

Proposal. 3: UE can request training data samples for NR DL AIML positioning (e.g., multiple NR-PRU-DL-Info IEs) via LPP RequestAssistanceData message.

LPP-10a: Applicability of "batch reporting" for AI/ML positioning.

Observation. 5: For training data collection, the existing batch reporting mechanism can be reused to allow UE to report multiple training data samples collected at different time instances to LMF.

Proposal. 4: Support batch reporting for NR DL AI/ML positioning.

LPP-21: "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19)

Proposal. 5: "Associated ID" for PRS-only TP coordinates can be associated with dl-PRS-ID of that TP.

Proposal. 6: Capture the restriction on the value of nr-AIML-AssociatedID as a note in the field description as below.

nr-AIML-AssociatedID This field provides an identity associated with the coordinates of the indicated TRP and coordinates of all the antenna reference points of this TRP. The value of the nr-AIML-AssociatedID is changed if/when the coordinates of the TRP and/or coordinates of any of the antenna reference points of this TRP is changed. NOTE : UE is not expected to receive different values of associated ID for TRPs belonging to the same cell.

Proposal. 7: RAN2 confirms that "Associated ID" is not only for the TRP location but also for the ARP location of the associated DL-PRS Resource Set/DL-PRS Resource.

Proposal. 8: RAN2 confirms that the value range of 0..255 for the "Associated ID" is sufficient.

Proposal. 9: RAN2 confirms that LMF can provide both the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) together.

Proposal. 10: UE can support explicit TRP location info (NR-TRP-LocationInfo-r16) and implicit location info (nr-TRP-ImplicitLocationInfo-r19) independently of each other.

LPP-18: Consistency between training and inference

Proposal. 11: To ensure consistency between training and inference, UE can request the assistance data (e.g., DL-PRS configuration) for a specific group of TRPs. I.e., UE can report a list of TRP information (i.e., combination of dl-PRS-ID, NCGI, PCI, ARFCN) to identify the group of TRPs for which the assistance data is requested.

Proposal. 12: To ensure consistency between training and inference, UE can report the associated ID value used during training phase for each TRP.