3GPP TSG-RAN WG2 Meeting #131	R2-2505360

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.6.4

Source: 	Fujitsu, NTT DOCOMO, InterDigital

Title:	Issue in Fast LTM Recovery after Conditional LTM Failure

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to provide our considerations about fast LTM recovery after CLTM failure.

- Discussions

  - Additional latency imposed by SN gap during fast LTM recovery

In Rel-18, to avoid keystream reuse issue, PDCP COUNT for SRB is continued when UE reverts back to source PCell configuration for fast LTM recovery. However, COUNT continuation in some cases leads to the SN gap issue when RRCReconfigurationComplete message is received for fast LTM recovery. When UE sends an RRCReconfigurationComplete message for LTM with COUNT = N and the message cannot reach the gNB and then the UE fails the LTM cell switch, the UE performs fast LTM recovery without re-establishing PDCP if attemptLTM-Switch is set and if the selected cell is one of the LTM candidate cells. During the fast LTM recovery, the UE sends another RRCReconfigurationComplete message with COUNT = N+1. From the NW side, PDCP PDU with COUNT = N is missing, so RRCReconfigurationComplete message with COUNT = N+1 is suspended to be delivered to upper layers until t-Reordering in PDCP expires.

Until RRC layer receives RRCReconfigurationComplete message, data transmission will not resume, so interruption time will be longer than the duration of t-Reordering on network side.

Figure 1: Sequence diagram of fast LTM recovery with SN gap

Observation 1: When the UE performs fast LTM recovery, the interruption time will be prolonged by the duration of PDCP t-Reordering on NW-side in some cases.

  - Distinguish between CLTM and fast LTM recovery after CLTM failure

To cope with the above issue, in Rel-18 RAN2 agreed to leave it to NW implementation as captured in the following NOTE in TS 38.300.

NOTE:	PDCP SN gap for SRB may exist upon LTM attempt toward the selected cell after LTM fails. It is up to network implementation to avoid the latency caused by the PDCP SN gap.

The gNB can know normal LTM is ongoing if the cell towards which LTM cell switch is being performed is the cell the gNB indicated in LTM Cell Switch Command MAC CE and otherwise the gNB can know fast LTM recovery is ongoing. That’s why the gNB can handle the SN gap accordingly in Rel-18.

Observation 2: In Rel-18, the SN gap issue is left up to NW implementation.

In Rel-19, Conditional LTM (CLTM) is introduced and CLTM and LTM can be configured together. Also, fast LTM recovery after CLTM failure is supported. Therefore, the gNB needs to distinguish between LTM, CLTM and fast LTM recovery. However, the gNB cannot distinguish between CLTM and fast LTM recovery after CLTM failure when CBRA-based LTM is being performed under the current specification.

Observation 3: The gNB cannot distinguish between CLTM and fast LTM recovery.

This means that the issue mentioned in chapter 2.1 cannot be resolved by NW implementation when the UE is performing fast LTM recovery after CLTM failure.

Observation 4: Additional latency caused by the PDCP SN gap cannot be avoided by NW implementation in Rel-19 if CLTM is configured.

This compromises the advantage of fast LTM recovery. Configuring PDCP t-Reordering on NW side for SRB as shorter value is one way to solve this issue, however, it is difficult to do that because too short PDCP t-Reordering for SRB would disable some NW features (e.g., RLC re-transmission). In addition, it should be noted that the parameter is not only for LTM but also for other control signaling.

Proposal 1: RAN2 to address the issue about the SN gap in fast LTM recovery after CLTM failure.

To cope with this issue without changing the duration of t-Reordering, we propose the following three options.

- Place a bit in PDCP header of RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

- Attach a new MAC CE to RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

- Define a new RRC parameter which configures whether the UE performs fast LTM recovery after CLTM failure.

In the first two options, the UE indicates to the NW whether CLTM or fast recovery is ongoing. First option is to place a bit in the PDCP header of RRCReconfigurationComplete message indicating whether CLTM or fast LTM recovery is being performed. The principle of this solution is the same as proposed for SN gap issue in Rel-18 in [1]. Second option is to attach a new MAC CE to RRCReconfigurationComplete message indicating whether CLTM or fast LTM recovery is being performed. The NW needs to know whether the ongoing LTM is CLTM or fast LTM recovery before it reorders the RRCReconfigurationComplete message in PDCP layer to handle the SN gap based on that information. Both the PDCP header and the MAC CE are processed before the gNB starts reordering of the PDCP SDU, so the gNB can handle the SN gap according to which LTM is being performed. Regarding the PDCP header solution, the gNB can easily implement this because the indication is given in the same layer, PDCP, in which the gNB reorders the PDCP PDUs, although spec impact will be large. Regarding the MAC CE solution, the gNB can easily implement this although it needs coordination between DU and CU.

Third option is to define a new RRC parameter which configures whether the UE performs fast LTM recovery after CLTM failure. In the current specification, attemptLTM-Switch is contained in LTM-Config, and if attemptLTM-Switch is configured, UE performs fast LTM recovery. Our proposal is to have another attemptLTM-Swtich for CLTM, e.g., attemptLTM-Switch-CLTM, and if the new parameter is not configured, UE does not perform fast LTM recovery after CLTM failure. If this proposal is agreed, the NW can decide whether the UE performs fast LTM recovery after LTM failure/RLF and after CLTM failure separately in operator’s preference. Although this solution cannot solve the SN gap issue directly, it can eliminate the interruption time caused by the SN gap issue by directly going to legacy RRC re-establishment procedure, and it has less spec impact.

Proposal 2: RAN2 to choose the solution from the following three options.

- Place a bit in PDCP header of RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

- Attach a new MAC CE to RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

- Define a new RRC parameter which configures whether the UE performs fast LTM recovery after CLTM failure.

- Conclusion

We have the following observations and proposals.

Observation 1: When the UE performs fast LTM recovery, the interruption time will be prolonged by the duration of PDCP t-Reordering on NW-side in some cases.

Observation 2: In Rel-18, the SN gap issue is left up to NW implementation.

Observation 3: The gNB cannot distinguish between CLTM and fast LTM recovery.

Observation 4: Additional latency caused by the PDCP SN gap cannot be avoided by NW implementation in Rel-19 if CLTM is configured.

Proposal 1: RAN2 to address the issue about the SN gap in fast LTM recovery after CLTM failure.

Proposal 2: RAN2 to choose the solution from the following three options.

- Place a bit in PDCP header of RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

- Attach a new MAC CE to RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

- Define a new RRC parameter which configures whether the UE performs fast LTM recovery after CLTM failure.

# References

[1] R2-2404781, “Discussion on PDCP SN gap at LTM fast recovery”, NTT DOCOMO, INC.