3GPP TSG-RAN WG2 Meeting #131	R2-2505366

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.11.3

Source:	Huawei, HiSilicon

Title:	Discussion on other aspects of SBFD

Document for: Discussion and Decision

# 1	Introduction

In the RAN2#130 meeting, RAN2 achieved the following agreements about other aspects:

- RAN2 confirm that the legacy Aperiodic CSI Trigger State Subselection MAC CE is also used to indicate the selection status of the Aperiodic Trigger States configured for aperiodic CLI measurement report. No spec change is needed. - No enhancement is needed for CSI-RS based RLM/BFD/CBD measurements. - Keep the legacy RLM, BFD and BFR procedure, i.e., no need to trigger RLF, BFD and BFR for non SBFD symbols and SBFD symbols separately.

In this contribution, we will further analyse the remaining RAN2 impacts based on the RRC open issue list.

# 2	Discussion

During the discussion of open issue of RRC, the issue of how to indicate the RO type for initial selection was raised, some companies proposed to use multiple bits considering the load balancing of using SBFD resources in a cell.

On this issue, we think it is better to keep the 1bit signalling (in the current RRC running CR). Since the RSRP method can also be used for the load balancing purpose if the network wants to achieve it. The 1bit signalling can be used by the gNB when the load balancing is not a conern. On the UE side, the associated random number drawing based solution due to such multiple bits signalling may lead to more complexity for UE implementation.

Based on those considerations, we propose to keep the current way of signalling RO type in sbfd-RO-Type-r19.

Proposal 1 for RRC-1: For the network indicating RO type, use 1 bit signalling (as in the current RRC running CR).

There was another extra issue raised that whether there is need to mandate that the legacy RA resource on SBFD RO is always configured if feature specific RA resource on SBFD RO is configured.

For this issue, we think it is reasonable to assume that the UE with one feature or feature combination can also use both SBFD RA resource and legacy RA resource under generic RACH configuration. The normal/reasonable gNB can ensure it by gNB implementation and we don't think there is need to explicitly capture this gNB behaviour in specification.

Proposal 2: RAN2 understands (yet no need to capture in spec) that the generic RA resource on SBFD RO is always configured if feature specific RA resource on SBFD RO is configured.

# 3	Conclusion

Based on the above discussion we have the following proposals for open issues:

Proposal 1 for RRC-1: For the network indicating RO type, use 1 bit signalling (as in the current RRC running CR).

Proposal 2: RAN2 understands (yet no need to capture in spec) that the generic RA resource on SBFD RO is always configured if feature specific RA resource on SBFD RO is configured.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)