# 3GPP TSG-RAN WG2 Meeting #131		                  							R2-2505378

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.2.3

Source:	Robert Bosch GmbH

Title:	Discussions on ambient IoT random access

Document for:	Discussion and Decision

# Introduction

In the RAN2#130 meeting, it was agreed that:

Agreements

A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below.  A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary.

•	Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

•	Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

•	Option A (the boundary being the subsequent paging only) is excluded.

# Ambient IoT Random Access

In ambient IoT random access procedure, a device may fail to receive msg2 from a reader. In this case, the device needs to identify the failure of msg2 and start the random access procedure by transmission of a new msg1 using upcoming access occasions. To identify the failure of msg2, which could be due to the following reasons:

- Failure of msg1 for e.g., collision with msg1 of other devices or

- Not selection of the device by the reader to transmit msg2 for the device or

- Failure of msg2 itself

a device should wait for a limited time window which can be based on the number of R2D trigger messages, and it is based on a K value. The K value defines the length of time which a device needs to wait for msg2 before re-starting random access procedure.

Observation 1: K value is needed to be decided by a reader and informed to the devices or pre-configured in devices.

Proposal 1: Reader may choose K value based on set of criteria such as number of msg1 receptions and inform it to the devices.

Proposal 2: Paging message or any other R2D message can carry the K value to the devices.

Proposal 3: In case if the K value is pre-configured in devices, a device may adapt K value based on set of conditions such as number of failures of msg2 or etc.

After reception of msg2 by a device, the device will transmit msg3 of random access procedure. Msg3 may also fail, and reader may need to repeat msg2 or send a NACK feedback to the device. Other possibility to identify the failure of msg3 is the lack of ACK reception for msg3 in a limited time window. In this case, a device identifies the failure of msg3 and can perform one of the following options:

- Device may wait until the next paging and then may start new random access procedure

- Device may re-transmit msg3 on the next access occasion

Observation 2: Reader need to configure the behavior of a device in case of msg3 failure.

Observation 3: A device may identify the failure of msg2 by reception of msg2 re-transmission or (lack of/) reception of feedback i.e., (ACK/) NACK.

Proposal 4: NACK message from a reader to a device for a msg3 failure can contain configuration information about the behavior of the device in case of msg3 failure.

Proposal 5: The re-transmission of msg2 can contain the required configuration for the device behavior in case msg3 failure.

Proposal 6: A device may decide independently about its behavior in case of msg3 failure based on set of criteria such as energy status/harvesting or service type.

# Conclusions

Based on the discussion in this contribution, we have the following observation and proposals:

Observation 1: K value is needed to be decided by a reader and informed to the devices or pre-configured in devices.

Observation 2: Reader need to configure the behavior of a device in case of msg3 failure.

Observation 3: A device may identify the failure of msg2 by reception of msg2 re-transmission or (lack of/) reception of feedback i.e., (ACK/) NACK.

Proposal 1: Reader may choose K value based on set of criteria such as number of msg1 receptions and inform it to the devices.

Proposal 2: Paging message or any other R2D message can carry the K value to the devices.

Proposal 3: In case if the K value is pre-configured in devices, a device may adapt K value based on set of conditions such as number of failures of msg2 or etc.

Proposal 4: NACK message from a reader to a device for a msg3 failure can contain configuration information about the behavior of the device in case of msg3 failure.

Proposal 5: The re-transmission of msg2 can contain the required configuration for the device behavior in case msg3 failure.

Proposal 6: A device may decide independently about its behavior in case of msg3 failure based on set of criteria such as energy status/harvesting or service type.