3GPP TSG-RAN WG2 #131	R2-2505383

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.3.2.3 Data collection

Source:	Ericsson

Title:	Data collection for UE sided models

Document for:	Discussion

# 1	Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. One of the objectives included in the SID is the Life Cycle Management (LCM) of the AI/ML model:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

This contribution discusses the aspects related to the data collection for UE sided AI/ML model, as part of the LCM framework. The following has been agreed in previous RAN2 meetings.

Agreements: - RAN2#129bis: - Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20. - RAN2#130: - For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

As agreed in RAN2#129bis and RAN2#130, RAN2 will consider data collection for UE-side model of AI ML PHY as a baseline. In the following we state relevant UE-side model data collection agreements for AI/ML PHY:

[RAN2#127bis]: - Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server) [RAN2#128/129]: - For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection. - FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration - The network can provide the data collection configuration (at any point in time), with or without UE requests. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

# 2	Discussion

## 2.1 Priorities

An aspect not addressed during the previous meeting is the potential for data collection instances to conflict with user plane data transmissions/receptions or RRM measurement occurrences. In such scenarios, data collection should be deprioritized to avoid interrupting normal network functionalities such as ongoing traffics and/or scheduled UE actions.

- UE side data collection shall not impair normal network functionality or performance including UL/DL data transmission or normal RRM measurements for mobility purposes.

### 2.2.1 Data collection in IDLE/INACTIVE state

In order to avoid any impairment of data collection on the normal network functionality, we believe UE sided data collection for model training for the AI/ML for Mobility use case should be done mainly by collecting the RRM measurement quantities such as RSRP, RSRQ, and SINR for the SSB based coverage of the serving and neighboring cells. Given that SSB resources are broadcast resources, the UE should be capable of data collection in RRC_IDLE/RRC_INACTIVE states.

Unlike AI/ML for air interface, data collection for UE sided model training is feasible in RRC_IDLE/RRC_INACTIVE states.

Data collection for UEs in RRC_IDLE/INACTIVE states can be done without any specific configuration required from the network, since the UE actions in RRC_IDLE/INACTIVE are, to a large extent, up to UE implementation. The data collection in RRC_IDLE/INACTIVE states can most likely also be up to UE implementation to a large extent. However, it needs to be checked whether the measurements in RRC_IDLE/RRC_INACTIVE fulfill the requirements that are needed for collection of data for training. Also, RAN2 needs to discuss the configuration in the WI phase.

- Data collection for UE sided models in RRC_IDLE/RRC_INACTIVE state is mainly up to UE implementation.

- RAN2 will discuss configuration of data collection in RRC_IDLE/RRC_INACTIVE in the work item phase.

- The measurement requirements for RRC_IDLE/INACTIVE for data collection purposes need to be verified by RAN4.

### 2.2.2 Data collection in RRC_CONNECTED state

When a UE is in RRC_CONNECTED state and collects measurements from its serving frequency, the data collection for model training should not impact ongoing procedures or measurements. However, if a lot of data needs to be collected, it could be cumbersome for the UE to perform the data collection during ongoing transmissions/receptions and other measurements. A possibility for the UE could be to perform the data collection during inactive periods in RRC_CONNECTED when there are no ongoing data transmissions/receptions, such as DRX. There should be no additional configuration required for such data collection, to avoid different solutions for RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED.

The UE can perform data collection in RRC_CONNECTED when there are no ongoing data transmissions/receptions, if no additional configuration is required.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Unlike AI/ML for air interface, data collection for UE sided model training is feasible in RRC_IDLE/RRC_INACTIVE states.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE side data collection shall not impair normal network functionality or performance including UL/DL data transmission or normal RRM measurements for mobility purposes.

Proposal 2	Data collection for UE sided models in RRC_IDLE/RRC_INACTIVE state is mainly up to UE implementation.

Proposal 3	RAN2 will discuss configuration of data collection in RRC_IDLE/RRC_INACTIVE in the work item phase.

Proposal 4	The measurement requirements for RRC_IDLE/INACTIVE for data collection purposes need to be verified by RAN4.

Proposal 5	The UE can perform data collection in RRC_CONNECTED when there are no ongoing data transmissions/receptions, if no additional configuration is required.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

R2-2504131 - Inference and report configuration of UE sided model