3GPP TSG-RAN WG2 Meeting #131		R2-2505394

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In the email discussion on [Post130][210][LP-WUS] Remaining RRC open issue for LP-WUS/WUR (vivo), one open issue is identified as below:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-12 | whether/how to enable/disable LP-WUS, e.g. by RRC/NAS As raised by Ericsson below. | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |

In the email discussion on [Post130][211] [LP-WUS] Running CR for 38.304 (CATT), three open issues are identified as below:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| 38304-8 | Whether LP-WUS is only used in the last used cell or in any cell | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| 38304-9 | FFS the SubgroupID for LP-WUS used outside CN PTW in RRC_INACTIVE state with CN configured PTW Note: the open issue was discussed online in RAN2#130 (R2-2504677) without conclusion. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| 38304-10 | FFS whether/how LP-WUS with SDT is supported Note: the open issue was discussed online in RAN2#130 (R2-2504264) without conclusion. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

In this contribution, we will further discuss the above issues for RRC_IDLE/ INACTIVE procedures with LP-WUR. And other remaining issues will also be discussed.

- Discussion

  - UE capability on subgrouping

RAN2 has agreed the CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, which is similar as PEI in Rel-17. It is straightforward to follow the similar capability design method for PEI.

Regarding the UE capability for CN assigned subgrouping in PEI, the Paging Subgrouping Support Indication is reported to CN from UE in the Registration Request message, and CN may use the information to determine the AMF PEIPS Assistance Information, which includes Paging Subgroup ID.

It is straightforward for LP-WUS to follow the similar capability design method for PEI. In details, UE’s capability of supporting CN assigned subgrouping is reported to CN by NAS signalling during the registration request or TAU procedure. According to the latest specification of TS 23.501, SA2 has already captured the UE capability for CN assigned subgrouping for LP-WUS as above.

Observation 1: For CN assigned subgrouping, the capability reporting design for PEI is reused for LP-WUS, i.e., UE reports capability of supporting the CN assigned subgrouping to CN during the registration request or TAU procedure, as captured in TS 23.501.

Regarding UE_ID based subgrouping capability for LP-WUS, it was agreed in RAN2#130 meeting:

On UE capabilities A UE indicating support of LP-WUS reception in IDLE/INACTIVE shall support UE-ID based subgrouping.

It could be observed that the capability of UE_ID based subgrouping could be defined as a component of the capability of LP-WUS reception in IDLE/INACTIVE.

According to the LS on UE features list [1] from RAN1, separate UE features for LP-WUS operation in IDLE/INACTIVE mode based on OOK signal (62-1) and OFDM overlaid sequence (62-1a) are defined. Accordingly, UE_ID based subgrouping capability should be introduced as one of the components for both features.

Proposal 1: The capability for supporting UE_ID based subgrouping is defined as one of the components of LP-WUS operation basic features based on OOK signal (62-1) and OFDM overlaid sequence (62-1a) in IDLE/INACTIVE.

  - Entry/exit condition for LP-WUS monitoring

We have the following agreements in RAN2#129bis meeting on the entry/exit conditions for RRM relaxation/offloading for different LR types:

- RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both. - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

However, the conclusion doesn’t cover the entry/exit thresholds for LP-WUS monitoring. In our understanding, there is no difference between LP-WUS monitoring and RRM measurement in this principle. Thus, the same principle for LP-WUS monitoring criteria should also be agreed:

Proposal 2: The conclusion on the entry/exit conditions for RRM relaxation/offloading for different LR types is applicable for the entry/exit conditions for LP-WUS monitoring, which is already captured in RRC and 38.304 running CR:

- RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

- RAN2 assumes for the entry/ exit conditions of LP-WUS monitoring: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

  - (38304-9) SubgroupID of LP-WUS outside PTW

Regarding the open issue:

Open issue 38304-9: FFS the SubgroupID for LP-WUS used outside CN PTW in RRC_INACTIVE state with CN configured PTW.

The method for determining the eDRX cycle for RRC_INACTIVE according to clause 7.1 in TS 38.304 [2] are shown below:

| CN | RAN | Paging cycle |
| --- | --- | --- |
| TeDRX, CN <1024 radio frames | TeDRX, RAN <1024 radio frames | T=min{TeDRX, CN, TeDRX, RAN} |
| TeDRX, CN <1024 radio frames | TeDRX, RAN is not configured | T=min{TeDRX, CN, TDRX, RAN} |
| TeDRX, CN >1024 radio frames | TeDRX, RAN is not configured | During CN configured PTW: T=min{TDRX, RAN, TDRX, CN, default DRX value } |
|  |  | Outside the CN configured PTW: T= TDRX, RAN |
| TeDRX, CN >1024 radio frames | TeDRX, RAN <1024 radio frames | During CN configured PTW: T=min{TDRX, CN, TeDRX, RAN, default DRX value } |
|  |  | Outside the CN configured PTW: T= TeDRX, RAN |
| TeDRX, CN >1024 radio frames | TeDRX, RAN >1024 radio frames | During the overlapped part of CN configured PTW and RAN configured PTW: T=min{TDRX, RAN, TDRX, CN, default DRX value } |
|  |  | During CN configured PTW and outside RAN configured PTW: T=min{TDRX, CN, default DRX value } |
|  |  | Outside CN configured PTW and during RAN configured PTW: T=TDRX, RAN |

The formula for determining the subgroup ID of the UE in UE_ID based subgrouping for LP-WUS, as captured by current 38.304 running CR, is shown below:

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 Ns: number of paging occasions for a PF UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

Based on the above, the determined value of N will be different between inside the CN configured PTW and outside the CN configured PTW, which will lead to different UE_ID based SubgroupID.

Observation 2: The DRX cycle of the UE operating in eDRX will be different between inside the CN configured PTW and outside the CN configured PTW for RRC_INACTIVE UE, which will lead to different UE_ID based SubgroupID.

In Rel-17, when PEI with subgrouping is configured, the same issue exists. The solution is to restrict the SubgroupID used outside CN PTW is the same as that used inside CN PTW:

In RRC_INACTIVE state with CN configured PTW the SubgroupID used outside CN PTW is the same as the SubgroupID used inside CN PTW.

The same principle could be used here for LP-WUS.

Proposal 3: (38304-9) In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

  - Coexistence between eDRX and LP-WUS

In RAN2#129bis meeting, we agreed that the LP-WUS is supported with eDRX, which is as below [3]:

- LP-WUS is supported with eDRX, FFS on exact impact if any

In our view, the mechanism for PEI with eDRX could be reused for LP-WUS, which means the LP-WUS is used to control PO monitoring. In details, it includes the following scenarios:

- When eDRX is configured as 5.12s and 10.24s, UE monitors PO per TeDRX, CN. Samely, the UE monitors the LP-WUS before the PO, i.e, with periodicity of TeDRX, CN.

- When eDRX is configured to be longer than 10.24s, UE monitors PO within the PTW per DRX cycle, thus UE should monitor the LP-WUS per DRX cycle within the PTW.

Proposal 4: For LP-WUS with eDRX, following the same mechanism as in PEI:

- When eDRX is configured to be longer than 10.24s, LP-WUS is used to control PO monitoring within the PTW.

- When eDRX is configured to be 5.12s or 10.24s, LP-WUS is used to control the PO monitoring.

  - (RRC-12) Enable/disable LP-WUS

- Regarding the open issue in [4]:

Open issue RRC-12: whether/how to enable/disable LP-WUS, e.g. by RRC/NAS

According to the discussion in email discussion [Post130][222] [LP-WUS] Enabling and disabling LP-WUS per UE (Huawei), following proposal is raised:

Observation (9/15): Many prefer to specify either NAS or RRC signalling to support enabling/disabling LP-WUS per UE. Proposal: (RRC-12) RAN2 to down-select from NAS signalling or RRC signalling to support enabling/disabling LP-WUS per UE.

Most companies tend to use dedicated signalling to support enabling/disabling LP-WUS per UE, but which specific dedicated signalling, i.e. NAS or RRC, to be used has no consensus. In our understanding, CN has more information on paging probability/UE type/service requirement information, etc., so we think NAS signalling approach is more appropriate. Besides, if it were relied on gNB control, new signalling between CN and gNB would be introduced to inform gNB on this control since the gNB couldn’t store the UE context for UEs in RRC_IDLE/INACTIVE.

Proposal 5: (RRC-12) Using NAS signalling to enable/disable the LP-WUS functionality per UE for UEs supporting LP-WUS. An LS is sent to SA2/CT1 with the corresponding conclusion.

  - (38304-10) Other wake up conditions

Regarding the open issue:

Open issue 38304-10: FFS whether/how LP-WUS with SDT is supported.

As legacy PEI design, the MR in ultra-deep sleep/monitoring LP-WUS should also wake up in case there is UL data/signalling arrival.

Proposal 6: (38304-10) MR should wake up for UE-initiated UL transmission, e.g., RACH/MO-SDT, due to UL data and signalling arrival when monitoring LP-WUS. And no specification impact is foreseen.

  - (38304-8) LP-WUS in last used cell

Regarding the open issue:

Open issue 38304-8: Whether LP-WUS is only used in the last used cell or in any cell.

For PEI, an indication in SIB1, i.e., lastUsedCellOnly was introduced as below to indicate whether the UE monitors PEI only if the last used cell is current camped cell.

lastUsedCellOnly When present, the field indicates that the UE monitors PEI only if the latest received RRCRelease without noLastCellUpdate is from this cell. A PEI-capable UE stores its last used cell information.

This was introduced to reduce the false alarm rate caused by the UE moving into one cell, in order to guarantee the maximum power saving gain for UE using PEI in this cell. But in LP-WUS, the situation is a bit different. As RAN1 has agreed to introduce subgroup number up to 31, which is much larger than PEI subgroup number. In this way, LP-WUS with more groupings, reduces the false alarm rate when waking up UE by LP-WUS, thereby greatly minimizing the impact on the power-saving gain of other UEs that are not in the same group as this UE in this cell. This weakens the motivation for introducing the lastUsedCellOnly.

Besides, as we proposed above, per UE enable/disable mechanism for LP-WUS functionality should be introduced. We believe that a reasonable network implementation should enable the using of LP-WUS only for UEs with relatively low traffic, i.e., those with low paging probability. UEs that may be frequently waken up or have a very high mobility state are not suitable for LP-WUS, as this could result in energy consumption overhead for other UEs. In this way, if the CN identifies the UE has high mobility state or will be frequently paged, it will disable the LP-WUS functionality for this UE directly. On the other hand, if the LP-WUS functionality is enabled, it illustrates that the false alarm brought by this mobile UE is low to other UEs whose last used cell is current cell. Thus, there is no need to introduce such indication for LP-WUS and UE could use the LP-WUS in any cell if its LP-WUS functionality is enabled and there is corresponding configuration provided in this cell.

Proposal 7: (38304-8) UE could use LP-WUS in any cell, i.e., there is no need to introduce lastUsedCellOnly.

  - Emergency PDU session

We have an FFS in RAN2#128:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

According to the CR in SA2#167 meeting[5], it removes the unnecessary restriction for emergency PDU session for PEI and LP-WUS with the following reason:

PSAP callback is delivered as normal call in IMS PDU session. There is no need for the UE and network to disable functionalities like PEIPS or LP-WUS when emergency PDU session is active

With this, RAN2 should follow the same principle as SA2, which means UE could monitor LP-WUS when UE has the emergency PDU session in RRC_IDLE/RRC_INACTIVE.

Proposal 8：Following the same principle as in SA2 on emergency PDU session with LP-WUS, i.e., UE could monitor LP-WUS when there is an active emergency PDU Session in RRC_IDLE/RRC_INACTIVE.

- Conclusion

In this contribution, we discuss the remaining issues for RRC_IDLE/ INACTIVE procedures with LP-WUR. Based on the discussion, we have the following observations and proposals.

Observation 1: For CN assigned subgrouping, the capability reporting design for PEI is reused for LP-WUS, i.e., UE reports capability of supporting the CN assigned subgrouping to CN during the registration request or TAU procedure, as captured in TS 23.501.

Observation 2: The DRX cycle of the UE operating in eDRX will be different between inside the CN configured PTW and outside the CN configured PTW for RRC_INACTIVE UE, which will lead to different UE_ID based SubgroupID.

UE capability on subgrouping

Proposal 1: The capability for supporting UE_ID based subgrouping is defined as one of the components of LP-WUS operation basic features based on OOK signal (62-1) and OFDM overlaid sequence (62-1a) in IDLE/INACTIVE.

Entry/exit condition for LP-WUS monitoring

Proposal 2: The conclusion on the entry/exit conditions for RRM relaxation/offloading for different LR types is applicable for the entry/exit conditions for LP-WUS monitoring, which is already captured in RRC and 38.304 running CR:

- RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

- RAN2 assumes for the entry/ exit conditions of LP-WUS monitoring: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

(38304-9) SubgroupID of LP-WUS outside PTW

Proposal 3: (38304-9) In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

Coexistence between eDRX and LP-WUSP-WUS

Proposal 4: For LP-WUS with eDRX, following the same mechanism as in PEI:

- When eDRX is configured to be longer than 10.24s, LP-WUS is used to control PO monitoring within the PTW.

- When eDRX is configured to be 5.12s or 10.24s, LP-WUS is used to control the PO monitoring.

(RRC-12) Enable/disable LP-WUS

Proposal 5: (RRC-12) Using NAS signalling to enable/disable the LP-WUS functionality per UE for UEs supporting LP-WUS. An LS is sent to SA2/CT1 with the corresponding conclusion.

(38304-10) Other wake up conditions

Proposal 6: (38304-10) MR should wake up for UE-initiated UL transmission, e.g., RACH/MO-SDT, due to UL data and signalling arrival when monitoring LP-WUS. And no specification impact is foreseen.

(38304-8) LP-WUS in last used cell

Proposal 7: (38304-8) UE could use LP-WUS in any cell, i.e., there is no need to introduce lastUsedCellOnly.

Emergency PDU session

Proposal 8：Following the same principle as in SA2 on emergency PDU session with LP-WUS, i.e., UE could monitor LP-WUS when there is an active emergency PDU Session in RRC_IDLE/RRC_INACTIVE.

- References

- R1-2504673, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121

- TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state

- RAN2#129bis Chair minutes

- R2-2503765, Discussion summary and list of RRC open issue for LP-WUS WUR

- S2-2502428, Removing unnecessary restrictions for emergency PDU sessions