3GPP TSG-RAN WG2 Meeting #131		R2-2505395

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we will further discuss the remaining issues on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE for the following open issue list from post meeting email discussion in [1] and [2].

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-7 | FFS on exit condition for serving cell RRM relaxation Editor’s NOTE: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-8 | FFS on whether/how to reduce the threshold number for LP-WUS/WUR Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and entry/exit condition of using LP-WUS, [and potential pre-condition between RRM relaxation/offloading criteria and entry/exit condition of using LP-WUS]. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-10 | FFS on low mobility criteria Editor’s NOTE: FFS on “low mobility” criteria. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| RRC-15 | FFS on impact on R16 RRM relaxation criteria (e.g. can LR can be used to evaluate Rel-16 “not-at-cell-edge” and “low mobility” criterion). As raised by Ericsson above. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| 38304-2 | FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| 38304-3 | FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. (Same as the open issue in RRC, i.e., FFS on exit condition for serving cell RRM relaxation) | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| 38304-6 | Whether UE low mobility criterion or stationary criterion should be considered for RRM relaxation/offloading. (Same as the open issue in RRC, i.e., FFS on low mobility criteria) | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

- Discussion

  - (RRC-7/38304-3) Exit condition for RRM measurement relaxation

Regarding the open issue:

| RRC-7/ 38304-3 | FFS on exit condition for serving cell RRM relaxation Editor’s NOTE: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

In R16/17 RRM relaxation, no separate exit condition is defined. That is, if the entry condition for neighboring cell RRM relaxation is not fulfilled, neighboring cell RRM relaxation is exited and UE performs the normal neighboring cell RRM measurement. We think the same principle can be applied to R19 serving/neighboring cell RRM relaxation, i.e., no need to define separate exit condition other than “not fulfilling the entry condition”, in case the R19 serving/neighboring cell RRM relaxation criteria is not merged with the LP-WUS monitoring condition, as we discussed in section 2.2. On the other hand, if the exit conditions of R19 RRM relaxation are merged with LP-WUS monitoring exit condition, then the exit conditions for R19 RRM relaxation are separate from the entry condition which is the same as the condition for LP-WUS monitoring.

Proposal 1: (RRC-7/38304-3) If LP-WUS monitoring condition is not merged with Rel-19 serving/neighboring cell RRM relaxation criteria, no need to define a separate exit condition other than ‘not fulfilling the entry condition’ for serving/neighboring cell RRM relaxation.

  - (RRC-8) Merge LP-WUS monitoring condition and RRM relaxation condition/criteria

Regarding the open issue:

| RRC-8 | FFS on whether/how to reduce the threshold number for LP-WUS/WUR Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and entry/exit condition of using LP-WUS, [and potential pre-condition between RRM relaxation/offloading criteria and entry/exit condition of using LP-WUS]. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

After the study in Rel-18, the following conclusion was made in TR 38.869 [3]:

UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbour cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore MR serving and neighbour cell measurement with further time domain relaxation than that is allowed in Rel-18 specification for IDLE/INACTIVE and/or at least serving cell RRM offloaded from MR to LR are beneficial.

It could be observed that there is no power saving gain if UE starts to monitor LP-WUS while the serving cell RRM relaxation/offloading or neighboring cell RRM relaxation has not been performed. Thus, the corresponding scenario should be avoided. One potential solution is to merge the entry/exit conditions for serving cell RRM offloading with LP-WUS monitoring condition or merge the entry/exit conditions for serving/neighboring cell RRM measurement relaxation with LP-WUS monitoring condition, i.e., when entry condition(s) for LP-WUS monitoring is satisfied, UE could relax the serving/neighboring cell RRM measurement or offloading the serving cell RRM measurement.

In RAN2#129bis meeting, we discussed how to reduce the configured threshold number for LP-WUS/WUR, and the corresponding solution direction was discussed in the offline discussion [4]. However, we didn’t reach a consensus on whether the condition for RRM relaxation or offloading should be combined with LP-WUS monitoring condition, as below.

Chair observation: only one company have strong concern on the following discussion point - Can further discuss Direction 3: - Direction 3a. merge LPWUS monitoring and serving cell offloading - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

In our understanding, both Direction 3a and Direction 3b could get noticeable power saving gain based on the study in Rel-18, thus we could support both. In details:

- For the scenario that if a cell only allows the serving cell RRM offloading or UE only supports the serving cell RRM offloading, merge the conditions for LP-WUS monitoring and serving cell RRM offloading.

- For the scenario that if a cell only allows the serving/neighboring cell RRM relaxation or UE only supports the serving/neighboring cell RRM relaxation, merge the conditions for LP-WUS monitoring and Rel-19 serving/neighboring cell RRM relaxation.

- For the scenario that if a cell allows the serving/neighboring cell RRM relaxation and serving cell RRM offloading, and UE also supports both, it is up to network configuration on whether to merge which one with the LP-WUS monitoring.

Proposal 2-1: (RRC-8) Merge LP-WUS monitoring entry/exit condition and serving cell RRM offloading if only serving cell RRM offloading is supported in the camped cell or UE only supports serving cell RRM offloading.

Proposal 2-2: (RRC-8) Merge LP-WUS monitoring entry/exit condition and Rel-19 serving/neighboring cell RRM relaxation if only Rel-19 serving/neighboring cell RRM relaxation is supported in the camped cell or UE only supports Rel-19 serving/neighboring cell RRM relaxation.

Proposal 2-3: (RRC-8) It is up to network configuration to merge LP-WUS monitoring entry/exit condition with serving cell RRM offloading or Rel-19 serving/neighboring cell RRM relaxation if both are supported in the camped cell and UE supports both.

  - (RRC-8) Analysis of the relationship between thresholds for LP-WUR

In RAN2#130 meeting, we analyzed the relationship between thresholds of criteria for serving cell offloading and R19 RRM relaxation, and between serving cell offloading and s criteria for cell reselection, which is as follows:

The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

And we will further analyze the relationship between thresholds of criteria for RRM relaxation/offloading and LP-WUS monitoring and the relationship between the threshold of criteria for R19 RRM relaxation and R17/R16 neighboring cell RRM relaxation as specified in Editor’s NOTE of RRC-8.

Issue#1: Relationship between thresholds of criteria for RRM relaxation/offloading and LP-WUS monitoring

In this part, we will discuss the relationship between thresholds of criteria for RRM relaxation/offloading and LP-WUS monitoring in case the proposal in section 2.2 is not agreed.

In this case, to achieve the power saving gain, i.e., to guarantee serving/neighboring cell RRM measurement is relaxed when UE monitors LP-WUS, thresholds of the criteria for the serving/neighboring cell RRM relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 3: (RRC-8) If merging LP-WUS monitoring and Rel-19 RRM relaxation is not agreed, the threshold of the entry condition for R19 RRM relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring.

And in case LP-WUS monitoring criterion is not merged with serving cell RRM offloading condition, considering the entry conditions of LP-WUS monitoring include at least serving cell quality via MR, serving cell measurement via MR cannot be offloaded to LR before UE starts the LP-WUS monitoring. Thus, thresholds of the criteria for the serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 4: (RRC-8) If merging LPWUS monitoring and serving cell RRM offloading is not agreed, the threshold of the entry condition for serving cell RRM offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Based on the analysis above, we provide an example to illustrate the thresholds and UE behaviours for the serving cell RRM relaxation/offloading and neighboring cell RRM relaxation compared to the threshold of entry condition on LP-WUS monitoring in the following figure.  All the criteria for serving cell RRM relaxation/offloading and neighboring cell RRM relaxation and the entry condition for LP-WUS monitoring are based on the serving cell quality via MR for simplicity.

Figure 1: Thresholds and UE behaviors for serving cell RRM offloading, serving/neighboring cell RRM relaxation and LP-WUS monitoring

Issue#2: Relationship between the thresholds of criteria for R19 RRM relaxation and R17/R16 neighboring cell RRM relaxation

RAN2 has the following agreements in RAN2#129bis meeting：

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

Considering it is agreed to merge the entry/exit condition for serving cell RRM relaxation and Rel-19 neighboring cell RRM relaxation, while when R16/R17 neighboring cell RRM relaxation criteria is fulfilled, UE still performs normal serving cell RRM measurement which illustrates the criteria for R19 RRM relaxation should be stricter than the legacy R16/R17 neighboring cell RRM relaxation criteria.

Proposal 5: (RRC-8) The corresponding threshold of R19 neighboring cell RRM relaxation should be stricter than that of R16/17 neighboring cell RRM relaxation, if configured.

  - (RRC-8) LR thresholds in entry condition

Regarding the RRC-8, which is FFS on whether/how to reduce the threshold number for LP-WUS/WUR, in RAN2#129bis meeting, one solution is provided to remove the LR based thresholds for all entry conditions in the RRM relaxation/offloading and LP-WUS monitoring procedures. However, we didn’t reach final conclusion [4].

In our understanding, since we only agreed with the intra-band scenario for MR and LR in Rel-19. In this case, the serving cell RRM measurement performance via MR band could represent the LR band to some extent. Thus, the motivation to introduce the LR based threshold for entry conditions on top of MR based threshold becomes weak. Besides, according to the following agreed RAN4 requirements:

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

It seems that the LR may not perform the RRM measurement at the initial phase when UE accesses a cell since when LR is turned on based on UE implementation, for example, when the MR measurement is better than a threshold the LR is not turned on. In this case, if the LR thresholds are configured for entry conditions, the time UE enters for the RRM relaxation or LP-WUS monitoring procedure may be delayed intentionally and the power saving gain will be impacted since even if the entry condition based on MR measurement is satisfied, UE has to wait for the entry condition based on LR measurement considering the time UE to turn on the LR measurement may be late and the LR measurement is not obtained at this time.

Thus, we think removing the LR based thresholds for all entry conditions is a reasonable direction to reduce the total number of thresholds in LP-WUR WI without the impact on power saving gain and will not bring any other issues.

Proposal 6: (RRC-8) Removing the LR based thresholds for all entry conditions for R19 RRM relaxation, serving cell RRM offloading and LP-WUS monitoring.

  - (RRC-10/38304-6) Neighboring cell measurement relaxation criteria

For the following issue:

| RRC-10 | FFS on low mobility criteria Editor’s NOTE: FFS on “low mobility” criteria. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |
| 38304-6 | Whether UE low mobility criterion or stationary criterion should be considered for RRM relaxation/offloading. (Same as the open issue in RRC, i.e., FFS on low mobility criteria) | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

Since the current criteria for serving cell RRM offloading/relaxation doesn’t consider the low mobility, and introducing the low mobility condition will result long time discussion, e.g., the UE behaviour when only one condition is configured, or when two conditions are configured but only one condition is fulfilled, etc., we prefer not to consider the low mobility criteria in Rel-19. On the other hand, the exit condition for RRM offloading is already introduced, thus the current mechanism is rigorous, considering the low mobility won’t bring much benefit or have much motivation.

Proposal 7: (RRC-10/38304-6) Low mobility condition is not needed for R19 RRM relaxation or offloading

  - (RRC-15/38304-2) Neighboring cell measurement relaxation criteria

Regarding the open issue that FFS on the enhancements on R16 criteria (38304-2/RRC-15)

| RRC-15 | FFS on impact on R16 RRM relaxation criteria (e.g. can LR can be used to evaluate Rel-16 “not-at-cell-edge” and “low mobility” criterion). As raised by Ericsson above. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |
| 38304-2 | FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

As indicated in 38304-2, for the case MR serving cell RRM measurement results are not available, which means the entry condition of serving cell RRM offloading is satisfied. Since it is agreed that the threshold of the criteria for serving cell RRM offloading should be higher than or equal to the maximum of {SIntraSearchP, SnonIntraSearchP}, thus the measurement on equal/lower priority frequency/RAT has already been stopped and UE won’t evaluate the R16/R17 RRM relaxation criterion in this case. Correspondingly, there is no need to work on the enhancement on R16 neighboring cell RRM relaxation in this case.

And for the case when LR is configured, whether the LR measurement can be used to evaluate Rel-16 “not-at-cell-edge” and “low mobility” criterion or Rel-17 “stationarity” and “stationary and not at cell edge” criterion for (e)RedCap UE when MR measurement is available, in our view, this is the optimization for legacy technique, which is not in the scope of LP-WUS WID.

Proposal 8: (RRC-15/38304-2) There is no impact on R16/R17 criterion of neighboring cell RRM relaxation regardless whether MR serving cell RRM measurement results are available or not, i.e., LR measurement won’t be used for R16/R17 RRM relaxation condition.

  - (RRC-9/38304-7) High priority frequency RRM relaxation

Regarding the open issue:

| RRC-9 | FFS on whether/how RRM relaxation is applicable for high priority frequency |
| --- | --- |
| 38304-7 | Whether Relaxed measurement and offloading measurement can be performed when there is NR inter-frequency and/or NR inter-RAT frequency with reselection priority higher than that of the camped frequency. (Same as the open issue in RRC, i.e., FFS on whether/how RRM relaxation is applicable for high priority frequency) |

In the post email discussion [2], most companies agreed nothing is needed on high priority frequency for serving cell measurement offloading or measurement relaxation with LP-WUS in 38.304 running CR. Besides, in Rel-16, the parameter highPriorityMeasRelax is introduced to indicate whether RRM measurements can be relaxed on high priority frequencies in case only low mobility criterion is satisfied.. Here in Rel-19, there is no need to introduce such indication, since in case serving cell RRM measurement is relaxed/offloaded, the RRM measurement on higher priority frequencies should be relaxed/stopped naturally, using network control is over-engineering. Anyway, final decision could be up to RAN4.

Proposal 9: (RRC-9/38304-7) RAN2 assumes no network indication is needed to control whether to relax/stop the RRM measurement on neighboring cells with higher priority frequency. It can be revisited based on RAN4 input.

- Conclusion

In this contribution, we discuss the open issues for RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE. Based on the discussion, we have the following observations and proposals.

(RRC-7/38304-3) Exit condition for RRM measurement relaxation

Proposal 1: (RRC-7/38304-3) If LP-WUS monitoring condition is not merged with Rel-19 serving/neighboring cell RRM relaxation criteria, no need to define a separate exit condition other than ‘not fulfilling the entry condition’ for serving/neighboring cell RRM relaxation.

(RRC-8) Merge LP-WUS monitoring condition and RRM relaxation condition/criteria

Proposal 2-1: (RRC-8) Merge LP-WUS monitoring entry/exit condition and serving cell RRM offloading if only serving cell RRM offloading is supported in the camped cell or UE only supports serving cell RRM offloading.

Proposal 2-2: (RRC-8) Merge LP-WUS monitoring entry/exit condition and Rel-19 serving/neighboring cell RRM relaxation if only Rel-19 serving/neighboring cell RRM relaxation is supported in the camped cell or UE only supports Rel-19 serving/neighboring cell RRM relaxation.

Proposal 2-3: (RRC-8) It is up to network configuration to merge LP-WUS monitoring entry/exit condition with serving cell RRM offloading or Rel-19 serving/neighboring cell RRM relaxation if both are supported in the camped cell and UE supports both.

(RRC-8) Analysis of the relationship between thresholds for LP-WUR

Proposal 3: (RRC-8) If merging LP-WUS monitoring and Rel-19 RRM relaxation is not agreed, the threshold of the entry condition for R19 RRM relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 4: (RRC-8) If merging LPWUS monitoring and serving cell RRM offloading is not agreed, the threshold of the entry condition for serving cell RRM offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 5: (RRC-8) The corresponding threshold of R19 neighboring cell RRM relaxation should be stricter than that of R16/17 neighboring cell RRM relaxation, if configured.

(RRC-8) LR thresholds in entry condition

Proposal 6: (RRC-8) Removing the LR based thresholds for all entry conditions for R19 RRM relaxation, serving cell RRM offloading and LP-WUS monitoring.

(RRC-10/38304-6) Neighboring cell measurement relaxation criteria

Proposal 7: (RRC-10/38304-6) Low mobility condition is not needed for R19 RRM relaxation or offloading

(RRC-15/38304-2) Neighboring cell measurement relaxation criteria

Proposal 8: (RRC-15/38304-2) There is no impact on R16/R17 criterion of neighboring cell RRM relaxation regardless whether MR serving cell RRM measurement results are available or not, i.e., LR measurement won’t be used for R16/R17 RRM relaxation condition.

(RRC-9/38304-7) High priority frequency RRM relaxation

Proposal 9: (RRC-9/38304-7) RAN2 assumes no network indication is needed to control whether to relax/stop the RRM measurement on neighboring cells with higher priority frequency. It can be revisited based on RAN4 input.

- References

- R2-2505393, Discussion summary and list of RRC open issue for LP-WUS WUR(vivo)

- R2-2505235, Summary of [Post130][211][LPWUS] Running CR for 38.304 (CATT)

- 3GPP TR 38.869 v18.0.0

- R2-2503032, [AT129bis][202][LPWUS] Proposals for RRM measurement relaxation (vivo)