3GPP TSG-RAN WG2 Meeting #131	R2-2505404

Bengaluru, India, 25th – 29th August 2025

Source:	vivo

Title:	Remaining issues on DSR enhancements for XR

Agenda Item:	8.7.4

Document for:	Discussion and Decision

- Introduction

In this contribution, the remaining issues on DSR from the email discussions [POST130][509][XR] UE capabilities CRs (Xiaomi) and [POST130][505][XR] MAC running CR and open issues (Qualcomm) are discussed, comprising DSR capability, DSR cancelation and BSR cancellation due to DSR transmission.

- Discussion

  - [UE capability-01] DSR capability

In the email discussion regarding DSR capability after RAN2#130, there is the following open issue[1]:

(UE capability-01): Whether a UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18.

In RAN2#130 [2], there are the following agreements on signaling design for R19 DSR:

(RRC-7) If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold. New LCID is used for R19 DSR

According to the above agreements, two aspects of the following can be derived:

- Firstly, for one UE, either all LCGs that require DSR are configured with R19 DSR or all LCGs that require DSR are configured with R18 DSR, i.e. for the same UE, it will never happen that some LCGs are configured with R18 DSR while some other ones are configured with R19 DSR, simultaneously;

- Secondly, relying on R19 DSR capability, the NW can also configure single portion DSR report for a LCG, i.e., the R18 DSR for an LCG can be equivalently achieved by configuring a triggering threshold plus a reporting threshold which is equal to the triggering threshold for the LCG.

In such sense, a UE with R19 DSR capability is not required to also support R18 DSR capability.

In practice, a gNB may only support R18 DSR. In this condition, a UE with R19 DSR capability may also implement R18 DSR so that delay aware scheduling can be achieved when it is served by such gNB, but this should be totally left for UE implementation.

- (UE capability-01) there is no pre-requisite for a UE supporting R19 DSR to also support R18 DSR.

  - [MAC-1] Cancellation of R19 DSR

In the email discussion regarding MAC after RAN2#130, there is the following open issue [3]:

[MAC-1] Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

For R18 DSR and R19 DSR, the same triggering condition as R18 DSR is used, i.e. the UE determines whether to trigger DSR for an LCG based on the configured triggering threshold. The data whose remaining time below the triggering threshold is referred as delay-critical data. In essence, R18 DSR / R19 DSR for an LCG is triggered when there is delay-delay critical data presence for the LCG but no pending DSR for the LCG.

- [MAC-1] The triggering criteria is the same between R18 DSR and R19 DSR, i.e. due to the presence of delay-critical data for an LCG when the LCG has no pending DSR.

For an LCG with pending R18 DSR, the DSR is cancelled when the DSR trigger condition is no longer fulfilled, i.e. all delay-critical data does not exist for the LCG, no matter the delay-critical data of the LCG has been transmitted or discarded. Similarly, this DSR cancellation condition is reused for R19 DSR, i.e. a pending DSR for an LCG is canceled when all delay-critical data for the LCG disappear, though there may be some other delay-reporting data for the LCG.

From our perspective, there is no need to report a DSR for the said other delay-reporting data for the LCG as it is not too late for the UE to trigger a DSR for the LCG when the said other delay-reporting data becomes delay-critical data. To reduce the overhead and UE complexity, it is preferred to keep unified DSR cancellation criteria between R18 DSR and R19 DSR.

- [MAC-1] Keep unified DSR cancellation criteria between R18 DSR and R19 DSR, i.e. no special enhancement is needed for R19 DSR cancellation.

  - [MAC-2] BSR cancellation due to DSR transmission

In the email discussion regarding MAC after RAN2#130, there is the following open issue[3]:

[MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

To identify the feasibility of this issue, let’s review the respective mechanisms of BSR MAC CE and DSR MAC CE generation according to the existing MAC spec and running CRs.

For BSR, there are the following related description in R18 MAC spec [4]:

For Regular and Periodic BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall: 1> if for at least one LCG configured with additionalBS-TableAllowed, the amount of UL data available for transmission when the MAC PDU containing the BSR is to be built is within the buffer sizes specified in Table 6.1.3.1-3: 2> report Refined Long BSR for all LCGs which have data available for transmission; 1> else: 2> if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built: 3> report Long BSR for all LCGs which have data available for transmission. 2> else if one LCG has data available and is configured with additionalBS-TableAllowed and the amount of UL data available for transmission when the MAC PDU containing the BSR is to be built is greater than the largest buffer size specified in Table 6.1.3.1-3: 3> report Long BSR. 2> else: 3> report Short BSR.

From the above spec, we can observe:

- For BSR MAC CE generation, the UE includes the respective total buffer sizes for all LCGs with available data.

In the latest R18 MAC spec [4] clause 6.1.3.72	Delay Status Report MAC CE

The DSR MAC CE shall include delay information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. The Remaining Time, the BT, and the Buffer Size fields for an LCG shall be reported in two consecutive octets. These three fields for different LCGs shall be included in a DSR MAC CE in ascending order based on the LCGi.

In Rel-19 MAC running CR [5], this paragraph is updated to the following:

A DSR MAC CE shall include delay status of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built.

From the above spec and running CR part, we can observe:

- For DSR MAC CE generation, the UE includes the delay status (i.e. delay-critical data size/remaining time for R18 DSR and delay-reporting data size/remaining time for R19 DSR) of all LCGs with pending DSRs.

Based on the difference between BSR generation and DSR generation, we can observe:

- Upon reception of a BSR MAC CE, the gNB can determine the respective total buffer sizes of all LCGs, while upon reception of a DSR MAC CE, the gNB can only identify the delay-critical/delay-reporting data sizes of the LCGs with pending DSRs.

If BSR is cancelled due to DSR transmission, the gNB may over-estimate the UL buffer size of the UE as usual considering the possible presence of non-delay-critical/non-delay-reporting data and over-allocate the UL radio resource for the UE. As the resource consumption for BSR transmission is almost ignorable compared to the potentially over-allocated UL resources, it is not beneficial for the UE to cancel the pending BSR at the cost of UL radio resource over-allocation risk.

- [MAC-2] The pending BSRs should not be cancelled due to the transmission of a DSR MAC CE.

- Conclusion

In this contribution, we have discussed remaining issues regarding DSR. Based on the discussions, there are the following observations:

Observation 1	[MAC-1] The triggering criteria is the same between R18 DSR and R19 DSR, i.e. due to the presence of delay-critical data for an LCG when the LCG has no pending DSR.

Observation 2	For BSR MAC CE generation, the UE includes the respective total buffer sizes for all LCGs with available data.

Observation 3	For DSR MAC CE generation, the UE includes the delay status (i.e. delay-critical data size/remaining time for R18 DSR and delay-reporting data size/remaining time for R19 DSR) of all LCGs with pending DSRs.

Observation 4	Upon reception of a BSR MAC CE, the gNB can determine the respective total buffer sizes of all LCGs, while upon reception of a DSR MAC CE, the gNB can only identify the delay-critical/delay-reporting data sizes of the LCGs with pending DSRs.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	(UE capability-01) there is no pre-requisite for a UE supporting R19 DSR to also support R18 DSR.

Proposal 2	[MAC-1] Keep unified DSR cancellation criteria between R18 DSR and R19 DSR, i.e. no special enhancement is needed for R19 DSR cancellation.

Proposal 3	[MAC-2] The pending BSRs should not be cancelled due to the transmission of a DSR MAC CE.

- References

- R2-2505138, Open issues of Rel-19 XR UE capabilities, Xiaomi

- RAN2#130, Chair notes

- R2-2505070, List of open issues in MAC, Qualcomm

- 3GPP TS 38.321 v18.6.0

- R2-2505069, Introduction to R19 XR enhancements