3GPP TSG-RAN WG2 #131	R2-2505413

# Bengaluru, India, 25-29 August 2025

Agenda item:	8.18.2

Source:	Qualcomm Incorporated

Title: 	Views on MAC and RRC Open Issues

Document for:	Discussion/Decision

# 1. Introduction

After the two email discussions [POST130][510][LTE Broadcast] and [POST130][511][LTE Broadcast], several issues were listed as open in the respective reports [1] and [2]. This document provides our views on how to resolve those open issues.

# 2. Remaining MAC open issues

The MAC email discussion report [2] listed the following MAC open issues:

MAC-1 Whether and how HARQ handling is described in the MAC specification for the Rel-19 time-interleaved PMCH transmission.

In our view, it is better to at least update the figure 4.2.1-1to indicate that HARQ is involved in T/F-interleaved PMCH. Additionally, we think it is better to update the procedural text to cover this case as proposed by rapporteur in [2].

Proposal 1: For open issue MAC-1: Incorporate the rapporteur’s proposed changes #1 and #2 from the email discussion report R2-2505554.

MAC-2 Whether Extended MSI is applicable for the Rel-19 time-interleaved PMCH transmission.

In our view, there has been no explicit agreement to exclude applicability of Extended MSI for the Rel-19 time-interleaved PMCH transmission.

The only change now in the non-extended MAC CE is to add “(excluding subframes containing MSI or MCCH for time-interleaved MCH)” clarification for counting subframes. Without this clarification in the Extended MSI MAC CE, there is risk that one future implementor may use Extended MSI MAC CE (since there is no explicit prohibition), use time-interleaving and not exclude those subframes in the counting. This is possible because we don’t have a “This MAC CE does not apply when time-interleaving is used” restriction in MAC.

Now, to add such restriction, we would need explicit agreement coming from RAN1. In lack of such explicit agreement from RAN1, making the same changes in both MAC CEs is the straightforward way to keep the specs complete and aligned.

Observation 1: Current RAN1 agreements do not exclude the Extended MSI for time-interleaved PMCH.

Observation 2: Not updating anything in the Extended MSI MAC CE (.1.3.7a in MAC) means the specs remain incomplete and ambiguous.

Proposal 1: For open issue MAC-2: Mirror the same changes from 6.1.3.7 to 6.1.3.7a in MAC CR.

# 2. Remaining RRC open issues

Following open issues have been listed in [1]. We provide our views on how to resolve them.

RRC-1: Rel-19 IEs for MBMS Interest Indication according to latest RAN1 agreements: Agreement Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s): - Frequency - New values of sub-carrier spacing = {2.5 kHz, 1.25 kHz} - New values for bandwidth = {n30, n35, n40} - A new UE baseband resource Scaling factor (e.g., utilized soft buffer at the UE) used for receiving the time interleaved broadcast

Since RAN1 will be further discussing this, RAN2 could wait for RAN1 to finalize and then capture in the RRC CR as needed. However, as baseline, we can introduce the signaling (including all the applicable SCS and BW) and only keep the FFS for Scaling Factor, as shown below.

MBMSInterestIndication-v1610-IEs ::=	SEQUENCE {

mbms-ROM-InfoList-r16				SEQUENCE (SIZE(1..maxMBMS-ServiceListPerUE-r13)) OF MBMS-ROM-Info-r16		OPTIONAL,

nonCriticalExtension				MBMSInterestIndication-v19xy-IEs SEQUENCE {}		OPTIONAL

}

MBMSInterestIndication-v19xy-IEs ::=	SEQUENCE {

mbms-ROM-InfoList-r19		SEQUENCE (SIZE(1..maxMBMS-ServiceListPerUE-r13)) OF MBMS-ROM-Info-r19		OPTIONAL,

nonCriticalExtension		SEQUENCE {}		OPTIONAL

}

MBMS-ROM-Info-r19 ::=	SEQUENCE {

mbms-ROM-Freq-r19					ARFCN-ValueEUTRA-r9,

mbms-ROM-SubcarrierSpacing-r19		ENUMERATED {kHz15, kHz7dot5, kHz2dot5, kHz1dot25},

mbms-Bandwidth-r19					ENUMERATED {n6, n15, n25, n30, n35, n40, n50, n75, n100},

mbms-ScalingFactor-r19				TYPE_FFS

}

Proposal 3: For open issue RRC-1: Introduce Rel-19 specific MBS Interest Indication IE (MBMSInterestIndication-v19xy-IEs) taking the TP from R2-2505413 as baseline.

RRC-2: Time-interleaved PMCH transmission (as configured) exceeding UE’s soft buffer size or supported TBS according to latest RAN1 agreements: Agreement In case the time interleaved PMCH transmission (as configured) exceeds a UE’s capability(/es) in terms of soft buffer size or maximum TBS supported in a TTI, the UE is not expected to receive the corresponding time interleaved PMCH transmission.

In our view, the above agreement has no direct RAN2 impact.

Proposal 4: For open issue RRC-2: No RAN2 spec impact.

RRC-3: FFS and Editor’s Noted in RRC running CR -- Editor’s Note: for the following parameter, exact candidate values are ‘ffs’ as they are still under discussion in RAN1 and not included in the RAN1 parameter list yet. Following is added as placeholder. pmch-CyclicShiftAlpha-r19 ENUMERATED {alphaOne, alphaOther, ffs} OPTIONAL -- Need OR pmch-CyclicShiftAlpha Indicates parameter α for cyclic shift for PMCH, see TS 36.211 [21] clause 6.5.1. Editor’s Note: the description may need further update once RAN1 finalizes the parameter list.

RAN2 can wait for RAN1 if there is further update on this and update the CR further during RRC CR review phase. If there is no RAN1 update, RAN2 can simply remove FFS and Editor’s Notes during RRC CR review.

Proposal 5: For open issue RRC-3: Wait for RAN1 further update, if any, and finalize during RRC CR review.

RRC-4: UE capability signaling

In the last meeting, UE capability aspects were postponed. RAN2 would need to introduce a few UE capability signaling according to the UE Feature List provided by RAN1 in [3]. A TP is provided in R2-2505741 [4] to be merged to RRC running CR.

Proposal 6: For open issue RRC-4: Take the TP for UE capability from R2-2505741 as baseline and merge into the RRC running CR.

# 3. Summary

Based on the discussion above, we have following observations and proposals to resolve the remaining open issues in MAC and RRC:

Observation 1: Current RAN1 agreements do not exclude the Extended MSI for time-interleaved PMCH.

Observation 2: Not updating anything in the Extended MSI MAC CE (.1.3.7a in MAC) means the specs remain incomplete and ambiguous.

Proposal 1: For open issue MAC-1: Incorporate the rapporteur’s proposed changes #1 and #2 from the email discussion report R2-2505554.

Proposal 2: For open issue MAC-2: Mirror the same changes from 6.1.3.7 to 6.1.3.7a in MAC CR.

Proposal 3: For open issue RRC-1: Introduce Rel-19 specific MBS Interest Indication IE (MBMSInterestIndication-v19xy-IEs) taking the TP from R2-2505413 as baseline.

Proposal 4: For open issue RRC-2: No RAN2 spec impact.

Proposal 5: For open issue RRC-3: Wait for RAN1 further update, if any, and finalize during RRC CR review.

Proposal 6: For open issue RRC-4: Take the TP for UE capability from R2-2505741 as baseline and merge into the RRC running CR.

# 4. References

- R2-2505412, [POST130][510][LTE Broadcast] RRC Open Issues, Qualcomm Incorporated, EBU

- R2-2505554, [POST130][511][LTE Broadcast] MAC Open Issues, Samsung

- R2-2505011, LS on updated Rel-19 RAN1 UE features lists for LTE after RAN1#121 (R1-2504678; contact: NTT DOCOMO, AT&T)

- R2-2505741, Introduction of LTE-based 5G Broadcast Phase 2, Huawei, HiSilicon, Qualcomm Incorporated