3GPP TSG-RAN WG2 Meeting #131	R2-2505422

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.8.4

Source:	Samsung

Title:	Open issues on Broadcast service area

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In this contribution, we continue to share our views on open issues for the MBS broadcast intended service area.

# Discussion

## SA4 LS reply

Regarding introducing intended service area in Service Announcement message, SA4 has replied the LS.

SA4 thanks RAN2 for bringing up the topic of “MBS via NTN”. SA4 is still studying the realization for including an Intended Service Area into Service announcement, leveraging the RAN2 Work Item code NR_NTN_Ph3-Core. SA4 has agreed to extend the Service Announcement and add a targeted service area in form of a list of polygon or circle shapes. However, SA4 is still discussing the technical realization and possibly a generalization, so that the target service area can be extended to represents the intended service area in case of NR NTN. # SA4 confirms, that inclusion of the geographical information, i.e. in form of list of polygon or circle shapes in the Service Announcement can be supported within the Rel-19 timeframe.

In our understanding of the LS, SA4 is defining a more generic TargetServiceArea information that may potentially be used for ISA in NTN, which however is not limited to NTN and is not same as the ISA introduced in NTN AS signalling. SA4 has agreed to use geographical areas and civic addresses as of now in TargetServiceArea information.

Observation 1: According to SA4 LS, SA4 has agreed to introduce a more generic TargetServiceArea information that may potentially be used for ISA in NTN, which however is not limited to NTN and is not the same as the ISA introduced in NTN AS signalling.

Based on this understanding, we think ISA by AS signalling and TargetServiceArea in the Service Announcement are independent solutions. We are not sure TargetServiceArea in the Service Announcement can be used equivalently to ISA in AS signalling.

Proposal 1: RAN2 confirms that ISA by AS signalling is independent from SA4’s solution of including TargetServiceArea in the Service Announcement. TargetServiceArea in the Service Announcement may not be used equivalently to ISA in AS signaling.

# Conclusion

In this contribution we discussed the following proposals.

Observation 1: According to SA4 LS, SA4 has agreed to introduce a more generic TargetServiceArea information that may potentially be used for ISA in NTN, which however is not limited to NTN and is not the same as the ISA introduced in NTN AS signalling.

Proposal 1: RAN2 confirms that ISA by AS signalling is independent from SA4’s solution of including TargetServiceArea in the Service Announcement. TargetServiceArea in the Service Announcement may not be used equivalently to ISA in AS signaling.

# Reference

RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

# Appendix

TS 38.300

##### 16.10.6.5.1	Service Continuity in RRC_IDLE or RRC_INACTIVE

Mobility procedures for MBS reception allow the UE to start or continue receiving MBS service(s) when changing cells. The gNB may indicate in the MCCH the list of neighbour cells providing the same MBS broadcast service(s) as provided in the serving cell. This allows the UE, e.g., to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. To avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies, the UE is made aware of which frequency is providing which MBS broadcast services via PTM, through User Service Description, as defined in TS 26.517 [46], or the combination of the following:

-	User Service Description (USD);

-	SIB21, as defined in clause 7.3.1.

NOTE:	UE can request unicast reception of the service after moving to a cell not providing the MBS broadcast service(s) using PTM transmission.

- TS 26.517

## 5.0	Overview

MBS User Service Announcement is needed in order to advertise MBS User Services in advance of, and potentially during, the MBS User Service Sessions described. MBS User Service Announcement (as defined in clauses 4.5.7 and 4.5.8 of TS 26.502 [3]) is provided by means of an MBS User Service Description, the syntax of which is defined in this clause.

- TS 26.502

### 4.5.7	MBS User Service Announcement parameters

This entity models an MBS User Service Announcement, which is compiled by the MBSF and used to advertise the current or imminent availability of an MBS User Service in the MBS System. The baseline parameters for an MBS User Service Announcement are listed in table 4.5.7-1 below:

Table 4.5.7-1: Baseline parameters of MBS User Service Announcement entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| External service identifiers | 1..* | MBS Application Provider | A unique identifier used by the MBSF Client to distinguish between MBS User Services. If assigned in a globally unique manner, this identifier may be useful to the MBSF Client in correlating the MBS User Service with the same service delivered by a different system. |
| Service class | 1..1 |  | The class of the MBS User Service, expressed as a term identifier from a controlled vocabulary. |
| Start date–time | 0..1 |  | The point in time from which this MBS User Service Announcement is valid. If not present, the announcement is already valid. |
| End date–time | 0..1 |  | The point in time after which this MBS User Service Announcement is no longer valid. If not present, the announcement is valid indefinitely. |
| Service names | 1..* |  | A set of distinguishing names for the MBS User Service, one per language. |
| Service descriptions | 1..* |  | A set of descriptions of the MBS User Service, one per language. |
| Main service language | 0..1 |  | The main language of the MBS User Service. |
| MBS Distribution Session Announcements | 1..* | MBSF | The set of MBS Distribution Session Announcements (see clause 4.5.8) currently associated with this MBS User Service Announcement. |

### 4.5.8	MBS Distribution Session Announcement parameters

This entity models an MBS Distribution Session Announcement, which is compiled by the MBSF and used to advertise the current or imminent availability of an MBS Distribution Session in the MBS System. The baseline parameters for an MBS Distribution Session Announcement are listed in table 4.5.8-1 below:

Table 4.5.8-1: Baseline parameters of MBS Distribution Session Announcement entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| MBS Session Identifier | 1..1 | MB-SMF | The Temporary Mobile Group Identity (TMGI) or Source-Specific Multicast (SSM) IP address of the MBS Distribution Session from which this announcement is derived. |
| MBS Frequency Selection Area (FSA) Identifier | 0..1 | MBS Application Provider or MB-SMF | (Broadcast MBS Session only.) Identifies a preconfigured area within which, and in proximity to, the cell(s) are announcing the MBS FSA ID and the associated frequency corresponding to this MBS Distribution Session Announcement (see NOTE) |
| Radio parameters | 0..* | MBSF | (Broadcast MBS Session only.) Radio transmission parameters of this MBS Distribution Session in its target service areas. The values are obtained by the MBSF from the OAM using the MBS Frequency Selection Area (FSA) Identifier (see above) as the lookup key. |
| Target UE classes | 0..* | MBS Application Provider | (Broadcast MBS Session only.) Indicates whether the MBS Distribution Session described by this announcement is suitable for consumption by NR RedCap UEs and/or non-NR RedCap UEs as defined in clause 6.19 of TS 23.247 [5]. |
| Distribution method | 1..1 | MBS Application Provider | The distribution method (as defined in clause 6) of the MBS Distribution Session from which this announcement is derived. |
| Session Description parameters | 1..* | MBSF | Additional parameters needed to receive the MBS Distribution Session from which this announcement is derived, including relevant User Plane traffic flow parameters. |
| Service protection description | 0..1 | MBSSF | The security parameters for the MBS Distribution Session (see clause W.4.2 of TS 33.501), including: - Which form of transport security protection is in force, whether UICC key management (see TS 33.246 [19]) is selected and/or whether 2G GBA security (see TS 33.246 [19]) is selected. - The MBS Session Key (MSK) identifier and key domain. - The address of the key management server (FQDN of the MBSSF) when user plane security is in force. |
| Time service endpoints | 0..N | MBSF | A set of endpoints provided by the MBS AS and used by the MBS Client to synchronise its clock with the needed precision. |
| NOTE: Used to guide frequency selection by the UE for a broadcast MBS Session. |  |  |  |