3GPP TSG-RAN WG2 Meeting #131													     R2-2505441

Bengaluru, India, August 25 – 29, 2025

Agenda item:	8.3.2.2

Source:	Apple

Title:	Event prediction

Document for:	Discussion and Decision

# 1	Introduction

RAN2#130 have made the following agreements on event prediction:

- Agreements

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

In this contribution we provide our additional inputs to progress this discussion.

# 2	Discussion

RAN2 have agreed to support prediction for A1-A6 events, without too much detail agreed yet (marked FFS).

On the high level, there are two possible approaches:

- Enhanced the existing A1-A6 events, or

- Defined new “predicted A1-A6” events

Probably both alternatives can work, but the latter is obviously cleaner, since the way ASN.1 is defined for events A1-A6 wouldn’t allow for clean extension of these events.

Observation 1: we can either extend the existing A1-A6 with “predicted” flag or define new “predicted A1-A6” events.

The remaining stage-3 level details can be discussed in the WI phase. There is no point in dragging this discussion, so it’s best to agree and move on to other issues. Hence the proposal.

Proposal 1: to define new “predicted A1-A6” events, e.g. predictedA1-predictedA6.

To illustrate the following point, we introduce the term “event window”, which is illustrated in Figure 1 below with a detailed explanation to follow.

Figure 1: "event window" term illustration

Let’s first consider the case of indirect measurement event prediction, in which case a model predicts an event at some point in time (let’s call it “t”) in the future. Naturally, t must be within the configured PW (which we’ve already agreed to introduce). Furthermore, in this example, t2 (which is t + maxETD) must also be within PW – this actually depends on the exact definition of PW, but either way the point below still stands even if PW is defined as the time from t0 till the predicted event.

Since the accuracy of an indirect event prediction model is calculated assuming a certain max ETD, the accuracy is meaningless if the max ETD of the model is not known to the network.

Observation 2: for indirect event prediction, the accuracy of the model is meaningless without the knowledge of max ETD.

Now turning to direct prediction, we recall that we’ve introduced the term “occurrence window” as shown in the following Figure 2.

Figure 2: "occurrence window" for direct prediction

We further assumed during the evaluation that occurrence window is equal to PW.

Observation 3: in evaluation, we assumed that occurrence window in direct prediction is the same as PW – however it doesn’t have to be. In fact it would be more efficient for network to configure different PW and occurrence windows.

This, however, does not (and should not) be always true. A better (and much more useful) way to define occurrence window is “somewhere within the PW”, that is the occurrence window should be somewhere at the end of the PW as illustrated below in Figure 3.

Figure 3: "occurrence window" in direct prediction

As one can easily see, the figure 3 (for direct prediction) is nearly identical to the figure 2 (for indirect prediction), with the only difference being that we use the term “max ETD” in one and “occurrence window” in another, whereas

Occurrence Window=maxETD2

With such trivial relation between occurrence window and max ETD it surely makes no sense to use two separate terms to refer to the same thing. It is best to define one term, that clearly identifies the concept, which is applicable to both direct and indirect prediction.

Observation 4: max ETD in indirect prediction and occurrence window in direct prediction essentially refer to the same thing.

Therefore, we propose to introduce the term “event window”, which unifies the terms “max ETD” for indirect prediction and “occurrence window” for direct prediction. Event window is identified by times t1 and t2, where t1 is equal or larger than t0 and the event is predicted to occur (with specified accuracy) between t1 and t2.

Proposal 2: to introduce the term “event window”, which unifies the terms “max ETD” for indirect prediction and “occurrence window” for direct prediction. Event window is identified by times t1 and t2, where t1 is equal or larger than t0 and the event is predicted to occur (with specified accuracy) between t1 and t2.

Proposal 3: for event prediction, event window should be considered as another inference parameter together with PW.

Finally, we would like to discuss the so called “HO models” defined in the TR 38.744 [2] as follows:

Option 1: the UE signals predicted event, the network may perform HO preparation based on this indication, then the UE indicates the real event, based on which the network triggers HO.

Figure 4: HO option 1

Option 2: the UE signals predicted event, the network triggers HO based on the predicted event.

Figure 5: HO option 2

Figure 6: HO option 3

Option 3: the UE signals predicted event at the time a real event would be triggered, the network triggers HO based on this event.

With regards to option 3, the network can implicitly configure it by selecting the PW accordingly. Therefore, no additional explicit signaling (besides PW) is needed. In fact, options 1 and 2 can be also implicitly supported with the signalling to be defined anyway (it’s reasonnable to assume that we will end up having signaling allowing the network to configure real and/or predicted events).

Observation 5: assuming we define signaling to configure PW and whether the UE should report real and/or predicted events, nothing else is needed to differentiate between HO options 1, 2, and 3.

Hence the proposal:

Proposal 4: no need to differentiate between HO models 1, 2, and 3 – the network can decide how to use the prediction (e.g. based on model prediction accuracy, PW and event window).

Finally, we think it’s worth clarifying how inference parameters (PW, event window, etc) are signaled. Naturally, the network should be in control. However, the network cannot assume that the UE supports all possible values of the inference parameters. Therefore, the UE should first indicate possible values (as ranges or lists) to the network, and the network configures a particular set of values based on what the UE supports.

Proposal 5: inference parameters can be configured by the network, selected from the range (or list) of values supported by the UE (as indicated in capability and applicability signaling).

# 3	Proposals

Observation 1: we can either extend the existing A1-A6 with “predicted” flag or define new “predicted A1-A6” events.

Proposal 1: to define new “predicted A1-A6” events, e.g. predictedA1-predictedA6.

Observation 2: for indirect event prediction, the accuracy of the model is meaningless without the knowledge of max ETD.

Observation 3: in evaluation, we assumed that occurrence window in direct prediction is the same as PW – however it doesn’t have to be. In fact it would be more efficient for network to configure different PW and occurrence windows.

Observation 4: max ETD in indirect prediction and occurrence window in direct prediction essentially refer to the same thing.

Proposal 2: to introduce the term “event window”, which unifies the terms “max ETD” for indirect prediction and “occurrence window” for direct prediction. Event window is identified by times t1 and t2, where t1 is equal or larger than t0 and the event is predicted to occur (with specified accuracy) between t1 and t2.

Proposal 3: for event prediction, event window should be considered as another inference parameter together with PW.

Observation 5: assuming we define signaling to configure PW and whether the UE should report real and/or predicted events, nothing else is needed to differentiate between HO options 1, 2, and 3.

Proposal 4: no need to differentiate between HO models 1, 2, and 3 – the network can decide how to use the prediction (e.g. based on model prediction accuracy, PW and event window).

Proposal 5: inference parameters can be configured by the network, selected from the range (or list) of values supported by the UE (as indicated in capability and applicability signaling).

# 4	References

RAN2#130 meeting minutes