3GPP TSG-RAN WG2 Meeting #131	R2-2505444

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.7.4

Source:	Apple

Title:	Remaining Issues of DSR Enhancements for Rel-19 XR

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

The following open issues have been identified for Rel-19 DSR enhancements [1][2][3]:

- [MAC-1]		Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

- [MAC-2]		Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

- [RLC-13] 		Whether the delay-reporting or non-delay-reporting information is visible or not to RLC?

- [PDCP-1]		Whether the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” needs to be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

This paper presents some of our views on these open issues.

# Discussions

## (MAC-1) DSR Cancellation

In our understanding, the open issue [MAC-1] is basically related to the following options of DSR cancellation:

- A pending DSR is cancelled when all PDCP SDUs with remaining time (or PDU Set Remaining Time) less than remainingTimeThreshold are transmitted or discarded.

- A pending DSR is cancelled when all PDCP SDUs with remaining time (or PDU Set Remaining Time) less than the largest dsr-ReportingThreshold are transmitted or discarded.

For Multi-Entry DSR, if we follow Option 1, the DSR may be triggered and then cancelled quite frequently if the remainingTimeThreshold is configured to be quite small, assuming that the UE can often get UL-SCH resource to transmit at least the PDCP SDUs with remaining time smaller than remainingTimeThreshold. Whenever DSR is triggered, the UE may need to perform some pre-processing to e.g. calculate the data volume, and/or to trigger a scheduling request (SR). If the DSR is cancelled, all the efforts that have been conducted since DSR triggering may go in vain, which is a wastage of system resource. From this perspective, it is more desirable to minimize DSR cancellation. Thus, we think Option 2 can better serve the purpose of Rel-19 DSR, which offers more opportunities for the UE to provide the full picture of buffer delay status to the network.

Proposal 1: (MAC-1) For Multi-Entry DSR, a pending DSR is cancelled when all PDCP SDUs with remaining time (or PDU Set Remaining Time) less than the largest dsr-ReportingThreshold are transmitted or discarded.

## (MAC-2) BSR Cancellation based on DSR

In Rel-18 when DSR was first introduced, RAN2 has agreed that DSR should be a separate type of MAC CE that is independent from BSR, rather than a special type of BSR format. With this in mind, we think it is better to entirely decouple these two mechanisms, as introducing inter-dependency between two types of MAC CEs may increase complexity. Thus, we suggest not to pursue the optimization on BSR cancellation based on what has been indicated in a DSR.

Proposal 2: (MAC-2) RAN2 does not pursue cancellation of pending BSR when the data volume is already indicated in a DSR MAC CE.

## (RLC-13) Visibility of Non-Delay-Reporting Information to RLC

In the running CR discussion, it has been raised that RLC may not need to distinguish between delay-reporting SDU and non-delay-reporting SDU. We think indeed it is unnecessary to have both definitions of “Delay-reporting RLC SDU” and “Non-delay-reporting RLC SDU” in the RLC specification. As long as the PDCP indicates that the RLC SDU corresponding to a PDCP SDU should be included in the data volume calculation, it does not really matter for the RLC to know if the RLC SDU corresponds to a delay reporting PDCP SDU or non-delay reporting PDCP SDU. In fact, having such differentiation in RLC specification may create some confusion for the readers. Hence, “Non-delay-reporting RLC SDU” does not need to be visible in the RLC specification.

Proposal 3: (RLC-13) There is no need to distinguish “Delay-reporting RLC SDU” and “Non-delay-reporting RLC SDU” in TS 38.322.

## (PDCP-1) Definition of PDCP Delay-Reporting Data

It has been suggested that, the procedure description to exclude a PDCP SDU from a reporting threshold if the PDCP SDUs that are already associated to the other reporting thresholds, can be merged into the definitions of both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU. From the data volume calculation procedure point of view, the UE would only know whether a PDCP SDU is already associated to a specific threshold when the data volume calculation procedure is initiated. It is worth noting that, when PDU Set Discarding is configured and PDU Set Remaining Time has to be taken into account, the UE should calculate the buffer size for the reporting thresholds in an ascending order of dsr-ReportingThreshold, to have a correct association. Thus, logically the UE would first calculate the buffer size corresponding to the i-th threshold, before the UE can calculate the buffer size corresponding to the k>i-th threshold. Thus, deciding whether a PDCP SDU should be excluded for a threshold is a step of data volume calculation.

Since the UE can only know this during the procedure of data volume calculation specified in Clause 5.15, and the procedure of calculation based on the ascending order of dsr-ReportingThreshold is already captured in Clause 5.15 of the running CR, we think it is more clear to keep the description of “excluding the PDCP SDUs already associated to another threshold” in the same clause. It is more awkward if the reader of the specification needs to jump between sections to understand how reporting threshold association should be determined.

Proposal 4: (PDCP-1) There is no need to move the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” to the definition section of TS 38.323.

# Conclusions

This paper presented our views on DSR enhancements for Rel-19. We have the following proposals to address the remaining open issues that have been identified in the running CR discussions:

Proposal 1: (MAC-1) For Multi-Entry DSR, a pending DSR is cancelled when all PDCP SDUs with remaining time (or PDU Set Remaining Time) less than the largest dsr-ReportingThreshold are transmitted or discarded.

Proposal 2: (MAC-2) RAN2 does not pursue cancellation of pending BSR when the data volume is already indicated in a DSR MAC CE.

Proposal 3: (RLC-13) There is no need to distinguish “Delay-reporting RLC SDU” and “Non-delay-reporting RLC SDU” in TS 38.322.

Proposal 4: (PDCP-1) There is no need to move the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” to the definition section of TS 38.323.

# References

- Rapporteur summary of discussion [POST130][505][XR]MAC Running CR and Open Issues Qualcomm)

- Rapporteur summary of discussion [POST130][508][XR] RLC Running CR and Open Issues (vivo)

- Rapporteur summary of discussion [POST130][507][XR] PDCP Running CR and Open Issues (LGE)