3GPP TSG-RAN WG2 Meeting #131	R2-2505448

Bengaluru, India, 25-29 August 2025

Agenda item:	8.2.3

Source:	Apple

Title:	Discussion on Random Access for Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1 Introduction

RAN2#130[1] has the following agreement on A-IoT RA design:

- 1 For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952). Capture a NOTE that other implementation are allowed. X, Y will be signalled by paging message - 2 The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages. R2D trigger message is not sent in CFRA procedure. Come back if RAN1/4 sees any issues. Send LS to RAN1/RAN4 - 3 FFS R2D byte alignment dependent on TBS size discussion - Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) - A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - · Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - · Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - · Option A (the boundary being the subsequent paging only) is excluded. - For option C, further discuss in terms of complexity at the device vs reader flexibility. - Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. - After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify. - Confirm a device is not expected to maintain both AS ID and RN16. After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader. - This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission. How to capture device behavior is FFS - For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not. FFS for CFRA - FFS for need for release message

In this paper, we discuss the remaining issues regarding the normative work for Ambient IoT random access:

# 2 Discussion

### 2.1 CBRA Msg2

In the last RAN2 meeting, companies agree to down-select two options (option B and Option C) in regards of when the device would expect a responding Msg 2 after transmitting Msg 1.

- Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

- Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

For those two options, we think option B is more friendly to device implementation because the device could get immediate response (Msg2) in the same Access Slot as it chooses to transmit Msg1. The device can then transmit Msg3 as scheduled in Msg2 and complete the CBRA procedure. For Option C, the device has a much longer wait (K Access slots) and uncertainty of when the responding Msg2 will be sent.

For the benefits of Option C, we can see two motivations for the reader side:

- Allows the gNB reader to aggregate more Msg2 in a single R2D transmission as it can gather Msg1s in K access slots.

- Allows the gNB reader a longer processing delay to read the incoming Msg1 and determine Msg2 contents for each individual Msg1 received.

However, those two motivations cannot be both achieved with Option C. The gNB reader, if unable to timely process the incoming Msg1 received in the same Access slot, will not be able to aggregate those devices (detected in the recent slot) in one single “Random ID response” message. Therefore, the benefit of Msg2/RN16 aggregation will not exist if the gNB reader’s processing capability is limited.

On the other hand, if the motivation is solely for aggregation, then it is more important for reader to properly design the AO allocations to ensure a proper number of Msg1 will be correctly decoded in each Access Slot. Moreover, the allowed number of FDMed transmission for Msg1 may not be exactly as the same as the allowed number of FDMed transmission of Msg3 because Msg3 transmission need a “bigger” resource than Msg1. So, it seems perfectly OK if that only a partial set of AOs I(e.g., 4 out of 8 FDM resources) is occupied in an Access slot by incoming Msg1s, gNB should simply respond to those Msg1 with a “Random ID Response” message including 4 FDMed Msg2 (echo of RN16)s. There is no strong justification for wait longer.

Given the above analysis, we think there is not much benefit to have a very large K, which would have a negative effect for device complexing and device energy consumption. RAN2 should either agree with Option B or only choose K=2 in Option C, if option C is used.

Proposal 1	For Msg2 reception boundary, RAN2 choose either Option B or Option C (k=2).

Then, regarding the FFS issue on whether to include “Frequency index” along with RN16, we think this is not needed because the chance of RN16 collision is extremely low.

In the table below, we have calculated the probability of at least two RN16s chosen by accessing A-IoT devices are identical in the case of Option B and Option C with a variety of different K values.,

Note that those collision probability has assumed that all the frequency shifts (R values) are all used by Msg1 transmissions. This is the most favorable scenario to maximize the collision chances, but in reality, it is very unlikely that all slots are used. In slotted aloha, the maximal efficiency of Slotted Aloha random access scheme is achieved with a ratio of 0.368, which means, ideally, only 36.8% of access slots are likely to be selected without reducing the overall system efficiency. So, the practical collision probability is even lower than the upper bounds provided in the table below:

.

|  | # of freq resource = 2 | # of freq resource = 4 | # of freq resource = 8 |
| --- | --- | --- | --- |
| Option B | 1/65536 | 0.009% | 0.04% |
| Option C (k=2) | 0.009% | 0.04% | 0.18% |
| Option C (k=4) | 0.04% | 0.18% | 0.38% |

According to the table above, we can see even the maximum chance of RN16 collision is around 0.38%, which means it rarely occurs. Even if it occurs, the reader can simply not respond to this collided RN16, and those two devices will be triggered to re-access in the next paging round. It is unreasonable to waste 3-bit frequency index for every RN16 to overcome this extremely rare case.

Proposal 2	Not to include frequency index along with RN16 in Msg2.

### 2.2 AS ID release

For CBRA, if there is a new transaction ID included in A-IoT paging message, the paged device will be forced to generate new RN16, and effectively overwrite the prior AS ID. So, the AS ID will not be kept indefinitely. Also, when device is out of energy, the AS ID will be forgotten by the device. Before that, the AS ID can be used by the reader to reach the device for sending R2D message(s). Even if the reader decide not to use an AS ID (for a discovered device) anymore, there is no need for an “early release” as the reader should have enough AS ID space to use. Also, we are not sure whether the reader’s MAC layer (w/o upper layer input) can really determine a device ID can be released “early”.

Same for AS ID assigned in CFRA procedure , the release by a new paging message (either CBRA or CFRA) could work.

Therefore, we think for both CBRA and CFRA., paging message can sufficiently release the AS IDs. There is no need to have the device to release an AS ID w/o generating a new one.

Proposal 3	Not to introduce “AS ID release” message in Rel-19 A-IoT MAC.

# 3 Conclusion

In this contribution, we discuss the Random-Access design for Ambient IoT, and have the following proposals:

Proposal 1	For Msg2 reception boundary, RAN2 choose either Option B or Option C (k=2).

Proposal 2	Not to include frequency index along with RN16 in Msg2.

Proposal 3	Not to introduce “AS ID release” message in Rel-19 A-IoT MAC.

# 4 References

[1] Chairman Notes, RAN2#130