3GPP TSG-RAN WG2 Meeting #131		R2-2505452
Bengaluru, India, Aug 25 – 29, 2025

Agenda Item: 	8.19.1

Source:	Apple Inc

Title:	Indication to the UE on the LTE operation via satellite

Document for:		Discussion and Decision

# Introduction

While 3GPP has officially created work packages for satellite operation for NR and for LTE IoT devices, there has been activity across the globe in providing LTE coverage via satellite.

Even when 3GPP is not involved in such usage, there are issues impacting the UE and the network and in this paper, we intend to raise a technical issue that the UEs must deal with when availing services using LTE via satellite.

# Open issue with LTE operation using satellite

## General LTE operation with satellite

Even when there is no official 3GPP work package (and specification output), several operators across the world are planning to deploy LTE using the satellite service, with the intention to provide LTE coverage to the UEs where the UE is expected use the LTE service without any NTN specific changes. To that effect [7][8] provide contributions and discussion in CT1 and SA1 WG, where the discussion revolves around the network providing additional information (via PLMNs for eg) to the UE about the LTE satellite operation.

Observation 1: Even when LTE via satellite is not part of 3GPP official work-items, there is interest in the ecosystem to provide LTE coverage with satellite.

Observation 2: While 3GPP impact is not expected (and the UEs are expected to operate without changes to use the LTE service via satellite) there are discussions to bring in some add-ons to specification to allow for UE control of LTE satellite access (for eg., via PLMN selection)

## Regulatory requirements for Satellite access by UEs

Satellite access by the UEs also requires that the UEs be compliant with local and global regulatory requirements. 3GPP has acknowledged this and TR22.926[1] has documented these different regulatory scenarios for 3GPP satellite-based systems. However, one aspect of network selection is missing, which relates to the UE handling on device switch-on or recovery from lack of coverage.

[2][3][4][5][6] provide regulations in certain countries which prohibit the use of satellite technology (either generally or allow the usage for specific purposes e.g. disaster scenarios), prohibit the use of satellite phones, require a license to use satellite services (without which the use of satellite services is prohibited), etc.

From TR 22.926, the clause 8.2 requires that UEs to select a network in the country where the UE is located, to ensure that the national regulatory requirements are fulfilled.

Observation 3: To ensure that the national regulatory requirements are fulfilled, the UE is required to select a network in the country where the UE is located.

Observation 4: Local regulations in multiple geographical locations prohibit the use of satellite technology (either generally or allow the usage for specific purposes e.g. disaster scenarios), prohibit the use of satellite phones, or require a license to use satellite services (without which the use of satellite services is prohibited).

### Regulatory compliance issue

One of the main motivating factors for the LTE satellite deployment is to address the out-of-service cases where terrestrial LTE network might not be able to provide coverage. In cases where there is no terrestrial coverage (out-of-service areas), there is scope for the UE to be not compliant with some regulations.

Observation 5: Main motivating factor for LTE satellite deployment is to resolve the coverage issues that terrestrial LTE networks might not be able to address.

Country A

Country B

Country C

MCC A MNC X

Figure 8.2.1-1: Satellite access covering multiple countries (from TR 22.926)

Referring to the diagram above (copied from TR 22.926), let’s assume that the regulations in country B prohibit the use of satellite access. The UE is in country B in an area with no terrestrial network coverage, and the UE is under the coverage area of a satellite network with a PLMN ID indicating country A (where satellite access is not prohibited). Such scenario is very likely to happen, given that it’s satellite coverage.

Since the LTE satellite coverage is intended to be indistinguishable (no 3GPP specification changes for the LTE satellite operation) to the UE, in the absence of any other reliable information, the UE could inadvertently consider itself to be under terrestrial coverage and try and register to the network, thus violating the regulations of country B.

Note that this is not a violation if the LTE service was provided with a terrestrial network from country A.

While this is possible with any satellite coverage, for NR NTN and LTE IoT NTN, 3GPP has already addressed the case of preventing UEs from accessing satellite cells by making it mandatory for the 5G network to provide an indication that the cell is using satellite access using a combination of broadcast information 'cellBarred' and 'cellBarredNTN-r17' (see TS 38.331 clause 5.2.2.4). The indications are used today to ensure UEs that do not support satellite access avoids selecting a satellite access network, and conversely, for UEs that support satellite access to select it.

It is worth noting that LTE IoT NTN systems also have the indication as in 5G, to ensure that UEs that do not support LTE IoT NTN do not access such coverage. However, for LTE covered by satellite, no such indication exists, which means that the UE is not able to make the same distinction.

Observation 6: Without any indication about the satellite coverage of LTE, UEs in out-of-service areas might try to register to satellite based LTE coverage in areas where such actions are not allowed per regulations.

Observation 7: 3GPP addressed this for NR NTN and LTE IoT NTN with explicit indication in the SIB1 of the respective RAT.

## Solution

RAN2 has a history of indicating the satellite service to the UE (NR NTN or LTE IoT NTN) via the necessary IEs in LTE and NR SIB1 already. If a new IE is added to the LTE SIB1 which indicates that the LTE coverage is via a satellite, the UE has all the information it needs to comply with national/local regulations in areas where such compliance is expected.

This indication would be an optional IE introduced to LTE SIB1, where operators who intend to provide LTE coverage via satellite in areas which border the geo-locations with such regulatory compliance requirement, can make use of this IE to indicate to the UEs.

Observation 8: Following the prior RAN2 history of introducing an indication in SIB1 about the satellite operation (for NR NTN and LTE IoT-NTN), RAN2 can introduce a new indication in LTE SIB1 for LTE satellite operation, that the operators who intend to provide LTE coverage via NTN in areas which border the geo-locations with such regulatory compliance requirement, can make use of this IE to indicate to the UEs.

Proposal 1: Introduce an optional IE in LTE SIB1 to indicate to the UE about the LTE NTN operation by the NW.

Proposal 2: Introduce the IE from Rel-19 but make it release independent.

Proposal 3: RAN2 to agree that the access stratum simply forwards the SIB1 indication to the upper layers when the UE reads this from LTE SIB1.

# Conclusion

Observation 1: Even when LTE via satellite is not part of 3GPP official work-items, there is interest in the ecosystem to provide LTE coverage with satellite.

Observation 2: While 3GPP impact is not expected (and the UEs are expected to operate without changes to use the LTE service via satellite) there are discussions to bring in some add-ons to specification to allow for UE control of LTE satellite access (for eg., via PLMN selection)

Observation 3: To ensure that the national regulatory requirements are fulfilled, the UE is required to select a network in the country where the UE is located.

Observation 4: Local regulations in multiple geographical locations prohibit the use of satellite technology (either generally or allow the usage for specific purposes e.g. disaster scenarios), prohibit the use of satellite phones, or require a license to use satellite services (without which the use of satellite services is prohibited).

Observation 5: Main motivating factor for LTE satellite deployment is to resolve the coverage issues that terrestrial LTE networks might not be able to address.

Observation 6: Without any indication about the satellite coverage of LTE, UEs in out-of-service areas might try to register to satellite based LTE coverage in areas where such actions are not allowed per regulations.

Observation 7: 3GPP addressed this for NR NTN and LTE IoT NTN with explicit indication in the SIB1 of the respective RAT.

Observation 8: Following the prior RAN2 history of introducing an indication in SIB1 about the satellite operation (for NR NTN and LTE IoT-NTN), RAN2 can introduce a new indication in LTE SIB1 for LTE satellite operation, that the operators who intend to provide LTE coverage via NTN in areas which border the geo-locations with such regulatory compliance requirement, can make use of this IE to indicate to the UEs.

Proposal 1: Introduce an optional IE in LTE SIB1 to indicate to the UE about the LTE NTN operation by the NW.

Proposal 2: Introduce the IE from Rel-19 but make it release independent.

Proposal 3: RAN2 to agree that the access stratum simply forwards the SIB1 indication to the upper layers when the UE reads this from LTE SIB1.

# Reference

[1] TS 22.926 Guidelines for extraterritorial 5G Systems (5GS)
[2] https://in.usembassy.gov/travel-alert-for-u-s-citizens-january-10-2025/   
[3] https://www.bicma.gov.bt/data/publications/rules-regulations-guidelines/Guidelines_for_use_of_Satellite_Phones_in_Bhutan_2019.pdf

[4] https://rura.rw/fileadmin/Documents/ICT/Laws/Draft_Regulation_governing_Satellites_services_in_Rwanda_18032022.pdf

[5] https://btrc.portal.gov.bd/sites/default/files/files/btrc.portal.gov.bd/page/2f2e55a5_f856_40a4_b0f3_6839b3905318/2022-02-07-07-53-74499b9aa26bfb32f7c6ea30f1c3760e.pdf

[6] https://car.bjmp.gov.ph/wp-content/uploads/2023/07/04.14.2023-BJMP-JSIOC-MC-149-SATELLITE-PHONE-POLICY.pdf

[7] S1-252285 Satellite PLMN Selection    Vodafone

[8] S1-252929 Lower Selection-priority for PLMN Selection  Vodafone, Deutsche Telekom, Verizon

# TP

We provide a text proposal below on how the changes would look like:

#### 5.2.2.7 Actions upon reception of the SystemInformationBlockType1 message Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall: 1> if the upper layers indicate the selected core network type as 5GC: 2> if the cellAccessRelatedInfoList-5GC contains an entry with the plmn-Identity or plmn-Index of the selected PLMN: 3> in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList-5GC containing the selected PLMN; 1> else if the cellAccessRelatedInfoList contains an entry with the PLMN-Identity of the selected PLMN: 2> in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList containing the selected PLMN; 1> if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and 1> if the UE is a category 0 UE according to TS 36.306 [5]; and 1> if category0Allowed is not included in SystemInformationBlockType1: 2> consider the cell as barred in accordance with TS 36.304 [4]; 1> if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators: 2> disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED; 2> forward the cellIdentity to upper layers; 2> forward the trackingAreaCode to upper layers; 2> forward the trackingAreaList to upper layers, if present; 1> else: 2> if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list: 3> consider the cell as barred for IAB-MT in accordance with TS 36.304 [4]; 3> perform barring as if intraFreqReselection is set to allowed, and as if the csg-Indication is set to FALSE; 2> else: 3> if the frequency band indicated in the freqBandIndicator or freqBandIndicatorAerial is part of the frequency bands supported by the UE and it is not a downlink only band; or 3> if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList or multiBandInfoListAerial are part of the frequency bands supported by the UE and they are not downlink only bands: 4> forward the cellIdentity to upper layers; 4> forward the trackingAreaCode to upper layers; 4> forward the trackingAreaList to upper layers, if present; 4> forward the satelliteOperation to upper layers, if present; 4> forward the PLMN identity to upper layers; 4> if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers: 5> if the serving cell does not belong to the configured ran-NotificationAreaInfo: 6> initiate an RNA update as specified in 5.3.17.2; 4> forward the ims-EmergencySupport to upper layers, if present; 4> forward the eCallOverIMS-Support to upper layers, if present; 4> if the UE is capable of 5G NAS: 5> forward the ims-EmergencySupport5GC to upper layers, if present; 5> forward the eCallOverIMS-Support5GC to upper layers, if present; 5> forward cp-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN; 5> forward up-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN; 4> if the UE is aerial UE and for the frequency band selected by the UE (from freqBandIndicatorAerial or multiBandInfoListAerial), the freqBandInfoAerial or the multiBandInfoListAerial is present and the UE capable of multiNS-Pmax does not support any of the additionalSpectrumEmission in the NS-PmaxListAerial within the freqBandInfoAerial or multiBandInfoListAerial: 5> consider the cell as barred in accordance with TS 36.304 [4]; 5> perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE, upon which the procedure ends; 4> else if the UE is aerial UE and for the frequency band selected by the UE (from freqBandIndicatorAerial or multiBandInfoListAerial), the freqBandInfoAerial or the multiBandInfoListAerial is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxListAerial within the freqBandInfoAerial or multiBandInfoListAerial: 5> apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxListAerial within freqBandInfoAerial or multiBandInfoListAerial; 5> if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxListAerial: 6> apply the additionalPmax; 5> else: 6> apply the p-Max; 4> else if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo or the multiBandInfoList-v10j0 is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo or multiBandInfoList-v10j0: 5> apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo or multiBandInfolist-v10j0; 5> if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList: 6> apply the additionalPmax; 5> else: 6> apply the p-Max; 4> else: 5> apply the additionalSpectrumEmission in SystemInformationBlockType2 and the p-Max; 3> else: 4> consider the cell as barred in accordance with TS 36.304 [4]; and 4> perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

#### – SystemInformationBlockType1 SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. SystemInformationBlockType1-BR uses the same structure as SystemInformationBlockType1. Signalling radio bearer: N/A RLC-SAP: TM Logical channels: BCCH and BR-BCCH Direction: E-UTRAN to UE SystemInformationBlockType1 message -- ASN1START SystemInformationBlockType1-BR-r13 ::= SystemInformationBlockType1 SystemInformationBlockType1 ::= SEQUENCE { cellAccessRelatedInfo SEQUENCE { plmn-IdentityList PLMN-IdentityList, trackingAreaCode TrackingAreaCode, cellIdentity CellIdentity, cellBarred ENUMERATED {barred, notBarred}, intraFreqReselection ENUMERATED {allowed, notAllowed}, csg-Indication BOOLEAN, csg-Identity CSG-Identity OPTIONAL -- Need OR }, cellSelectionInfo SEQUENCE { q-RxLevMin Q-RxLevMin, q-RxLevMinOffset INTEGER (1..8) OPTIONAL -- Need OP }, p-Max P-Max OPTIONAL, -- Need OP freqBandIndicator FreqBandIndicator, schedulingInfoList SchedulingInfoList, tdd-Config TDD-Config OPTIONAL, -- Cond TDD si-WindowLength ENUMERATED { ms1, ms2, ms5, ms10, ms15, ms20, ms40}, systemInfoValueTag INTEGER (0..31), nonCriticalExtension SystemInformationBlockType1-v890-IEs OPTIONAL } SystemInformationBlockType1-v890-IEs::= SEQUENCE { lateNonCriticalExtension OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs) OPTIONAL, nonCriticalExtension SystemInformationBlockType1-v920-IEs OPTIONAL } -- Late non critical extensions SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { multiBandInfoList MultiBandInfoList OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v9e0-IEs OPTIONAL } SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE { freqBandIndicator-v9e0 FreqBandIndicator-v9e0 OPTIONAL, -- Cond FBI-max multiBandInfoList-v9e0 MultiBandInfoList-v9e0 OPTIONAL, -- Cond mFBI-max nonCriticalExtension SystemInformationBlockType1-v10j0-IEs OPTIONAL } SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE { freqBandInfo-r10 NS-PmaxList-r10 OPTIONAL, -- Need OR multiBandInfoList-v10j0 MultiBandInfoList-v10j0 OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v10l0-IEs OPTIONAL } SystemInformationBlockType1-v10l0-IEs ::= SEQUENCE { freqBandInfo-v10l0 NS-PmaxList-v10l0 OPTIONAL, -- Need OR multiBandInfoList-v10l0 MultiBandInfoList-v10l0 OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v10x0-IEs OPTIONAL } SystemInformationBlockType1-v10x0-IEs ::= SEQUENCE { -- This field is only for late non-critical extensions from Rel-10 or Rel-11 onwards lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SystemInformationBlockType1-v12j0-IEs OPTIONAL } SystemInformationBlockType1-v12j0-IEs ::= SEQUENCE { schedulingInfoList-v12j0 SchedulingInfoList-v12j0 OPTIONAL, -- Need OR schedulingInfoListExt-r12 SchedulingInfoListExt-r12 OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v15g0-IEs OPTIONAL } SystemInformationBlockType1-v15g0-IEs ::= SEQUENCE { bandwidthReducedAccessRelatedInfo-v15g0 SEQUENCE { posSchedulingInfoList-BR-r15 SchedulingInfoList-BR-r13 } OPTIONAL, -- Need OR nonCriticalExtension SEQUENCE {} OPTIONAL } -- Regular non critical extensions SystemInformationBlockType1-v920-IEs ::= SEQUENCE { ims-EmergencySupport-r9 ENUMERATED {true} OPTIONAL, -- Need OR cellSelectionInfo-v920 CellSelectionInfo-v920 OPTIONAL, -- Cond RSRQ nonCriticalExtension SystemInformationBlockType1-v1130-IEs OPTIONAL } SystemInformationBlockType1-v1130-IEs ::= SEQUENCE { tdd-Config-v1130 TDD-Config-v1130 OPTIONAL, -- Cond TDD-OR cellSelectionInfo-v1130 CellSelectionInfo-v1130 OPTIONAL, -- Cond WB-RSRQ nonCriticalExtension SystemInformationBlockType1-v1250-IEs OPTIONAL } SystemInformationBlockType1-v1250-IEs ::= SEQUENCE { cellAccessRelatedInfo-v1250 SEQUENCE { category0Allowed-r12 ENUMERATED {true} OPTIONAL -- Need OP }, cellSelectionInfo-v1250 CellSelectionInfo-v1250 OPTIONAL, -- Cond RSRQ2 freqBandIndicatorPriority-r12 ENUMERATED {true} OPTIONAL, -- Cond mFBI nonCriticalExtension SystemInformationBlockType1-v1310-IEs OPTIONAL } SystemInformationBlockType1-v1310-IEs ::= SEQUENCE { hyperSFN-r13 BIT STRING (SIZE (10)) OPTIONAL, -- Need OR eDRX-Allowed-r13 ENUMERATED {true} OPTIONAL, -- Need OR cellSelectionInfoCE-r13 CellSelectionInfoCE-r13 OPTIONAL, -- Need OP bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { si-WindowLength-BR-r13 ENUMERATED { ms20, ms40, ms60, ms80, ms120, ms160, ms200, spare}, si-RepetitionPattern-r13 ENUMERATED {everyRF, every2ndRF, every4thRF, every8thRF}, schedulingInfoList-BR-r13 SchedulingInfoList-BR-r13 OPTIONAL, -- Cond SI-BR fdd-DownlinkOrTddSubframeBitmapBR-r13 CHOICE { subframePattern10-r13 BIT STRING (SIZE (10)), subframePattern40-r13 BIT STRING (SIZE (40)) } OPTIONAL, -- Need OP fdd-UplinkSubframeBitmapBR-r13 BIT STRING (SIZE (10)) OPTIONAL, -- Need OP startSymbolBR-r13 INTEGER (1..4), si-HoppingConfigCommon-r13 ENUMERATED {on,off}, si-ValidityTime-r13 ENUMERATED {true} OPTIONAL, -- Need OP systemInfoValueTagList-r13 SystemInfoValueTagList-r13 OPTIONAL -- Need OR } OPTIONAL, -- Cond BW-reduced nonCriticalExtension SystemInformationBlockType1-v1320-IEs OPTIONAL } SystemInformationBlockType1-v1320-IEs ::= SEQUENCE { freqHoppingParametersDL-r13 SEQUENCE { mpdcch-pdsch-HoppingNB-r13 ENUMERATED {nb2, nb4} OPTIONAL, -- Need OR interval-DLHoppingConfigCommonModeA-r13 CHOICE { interval-FDD-r13 ENUMERATED {int1, int2, int4, int8}, interval-TDD-r13 ENUMERATED {int1, int5, int10, int20} } OPTIONAL, -- Need OR interval-DLHoppingConfigCommonModeB-r13 CHOICE { interval-FDD-r13 ENUMERATED {int2, int4, int8, int16}, interval-TDD-r13 ENUMERATED { int5, int10, int20, int40} } OPTIONAL, -- Need OR mpdcch-pdsch-HoppingOffset-r13 INTEGER (1..maxAvailNarrowBands-r13) OPTIONAL -- Need OR } OPTIONAL, -- Cond Hopping nonCriticalExtension SystemInformationBlockType1-v1350-IEs OPTIONAL } SystemInformationBlockType1-v1350-IEs ::= SEQUENCE { cellSelectionInfoCE1-r13 CellSelectionInfoCE1-r13 OPTIONAL, -- Need OP nonCriticalExtension SystemInformationBlockType1-v1360-IEs OPTIONAL } SystemInformationBlockType1-v1360-IEs ::= SEQUENCE { cellSelectionInfoCE1-v1360 CellSelectionInfoCE1-v1360 OPTIONAL, -- Cond QrxlevminCE1 nonCriticalExtension SystemInformationBlockType1-v1430-IEs OPTIONAL } SystemInformationBlockType1-v1430-IEs ::= SEQUENCE { eCallOverIMS-Support-r14 ENUMERATED {true} OPTIONAL, -- Need OR tdd-Config-v1430 TDD-Config-v1430 OPTIONAL, -- Cond TDD-OR cellAccessRelatedInfoList-r14 SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF CellAccessRelatedInfo-r14 OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v1450-IEs OPTIONAL } SystemInformationBlockType1-v1450-IEs ::= SEQUENCE { tdd-Config-v1450 TDD-Config-v1450 OPTIONAL, -- Cond TDD-OR nonCriticalExtension SystemInformationBlockType1-v1530-IEs OPTIONAL } SystemInformationBlockType1-v1530-IEs ::= SEQUENCE { hsdn-Cell-r15 ENUMERATED {true} OPTIONAL, -- Need OR cellSelectionInfoCE-v1530 CellSelectionInfoCE-v1530 OPTIONAL, -- Need OP crs-IntfMitigConfig-r15 CHOICE { crs-IntfMitigEnabled NULL, crs-IntfMitigNumPRBs ENUMERATED {n6, n24} } OPTIONAL, -- Need OR cellBarred-CRS-r15 ENUMERATED {barred, notBarred}, plmn-IdentityList-v1530 PLMN-IdentityList-v1530 OPTIONAL, -- Need OR posSchedulingInfoList-r15 PosSchedulingInfoList-r15 OPTIONAL, -- Need OR cellAccessRelatedInfo-5GC-r15 SEQUENCE { cellBarred-5GC-r15 ENUMERATED {barred, notBarred}, cellBarred-5GC-CRS-r15 ENUMERATED {barred, notBarred}, cellAccessRelatedInfoList-5GC-r15 SEQUENCE (SIZE (1..maxPLMN-r11)) OF CellAccessRelatedInfo-5GC-r15 } OPTIONAL, -- Need OP ims-EmergencySupport5GC-r15 ENUMERATED {true} OPTIONAL, -- Need OR eCallOverIMS-Support5GC-r15 ENUMERATED {true} OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v1540-IEs OPTIONAL } SystemInformationBlockType1-v1540-IEs ::= SEQUENCE { si-posOffset-r15 ENUMERATED {true} OPTIONAL, -- Need ON nonCriticalExtension SystemInformationBlockType1-v1610-IEs OPTIONAL } SystemInformationBlockType1-v1610-IEs ::= SEQUENCE { eDRX-Allowed-5GC-r16 ENUMERATED {true} OPTIONAL, -- Need OR transmissionInControlChRegion-r16 ENUMERATED {true} OPTIONAL, -- Cond BW-reduced campingAllowedInCE-r16 ENUMERATED {true} OPTIONAL, -- Need OR plmn-IdentityList-v1610 PLMN-IdentityList-v1610 OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v1700-IEs OPTIONAL } SystemInformationBlockType1-v1700-IEs ::= SEQUENCE { cellAccessRelatedInfo-NTN-r17 SEQUENCE { cellBarred-NTN-r17 ENUMERATED {barred, notBarred}, plmn-IdentityList-v1700 PLMN-IdentityList-v1700 OPTIONAL -- Need OR } OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v1800-IEs OPTIONAL } SystemInformationBlockType1-v1800-IEs ::= SEQUENCE { freqBandIndicatorAerial-r18 FreqBandIndicator-r11 OPTIONAL, -- Need OR freqBandInfoAerial-r18 NS-PmaxListAerial-r18 OPTIONAL, -- Need OR multiBandInfoListAerial-r18 MultiBandInfoListAerial-r18 OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType1-v1900-IEs OPTIONAL } SystemInformationBlockType1-v1900-IEs ::= SEQUENCE { satelliteOperation-r19 ENUMERATED {true} OPTIONAL, -- Need OR nonCriticalExtension SEQUENCE {} OPTIONAL } PLMN-IdentityList ::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo PLMN-IdentityInfo ::= SEQUENCE { plmn-Identity PLMN-Identity, cellReservedForOperatorUse ENUMERATED {reserved, notReserved} } PLMN-IdentityList-v1530 ::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1530 PLMN-IdentityInfo-v1530 ::= SEQUENCE { cellReservedForOperatorUse-CRS-r15 ENUMERATED {reserved, notReserved} } PLMN-IdentityList-r15::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-r15 PLMN-IdentityList-v1610::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1610 PLMN-IdentityList-v1700::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1700 PLMN-IdentityInfo-r15 ::= SEQUENCE { plmn-Identity-5GC-r15 CHOICE{ plmn-Identity-r15 PLMN-Identity, plmn-Index-r15 INTEGER (1..maxPLMN-r11) }, cellReservedForOperatorUse-r15 ENUMERATED {reserved, notReserved}, cellReservedForOperatorUse-CRS-r15 ENUMERATED {reserved, notReserved} } PLMN-IdentityInfo-v1610 ::= SEQUENCE { cp-CIoT-5GS-Optimisation-r16 ENUMERATED {true} OPTIONAL, -- Need OR up-CIoT-5GS-Optimisation-r16 ENUMERATED {true} OPTIONAL, -- Need OR iab-Support-r16 ENUMERATED {true} OPTIONAL -- Need OR } PLMN-IdentityInfo-v1700 ::= SEQUENCE { trackingAreaList-r17 TrackingAreaList-r17 OPTIONAL -- Need OP } SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo SchedulingInfoList-v12j0 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-v12j0 SchedulingInfoListExt-r12 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfoExt-r12 SchedulingInfo ::= SEQUENCE { si-Periodicity SI-Periodicity-r12, sib-MappingInfo SIB-MappingInfo } SchedulingInfo-v12j0 ::= SEQUENCE { sib-MappingInfo-v12j0 SIB-MappingInfo-v12j0 OPTIONAL -- Need OR } SchedulingInfoExt-r12 ::= SEQUENCE { si-Periodicity-r12 SI-Periodicity-r12, sib-MappingInfo-r12 SIB-MappingInfo-v12j0 } SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13 SchedulingInfo-BR-r13 ::= SEQUENCE { si-Narrowband-r13 INTEGER (1..maxAvailNarrowBands-r13), si-TBS-r13 ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712, b808, b936} } SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type SIB-MappingInfo-v12j0 ::= SEQUENCE (SIZE (1..maxSIB-1)) OF SIB-Type-v12j0 -- Note: The IE SIB-Type (without suffix) will not be extended any further in this release of the specification. If needed, the IE SIB-Type-v12j0 will be used for new SIB(s). SIB-Type ::= ENUMERATED { sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9, sibType10, sibType11, sibType12-v920, sibType13-v920, sibType14-v1130, sibType15-v1130, sibType16-v1130, sibType17-v1250, sibType18-v1250, ..., sibType19-v1250, sibType20-v1310, sibType21-v1430, sibType24-v1530, sibType25-v1530, sibType26-v1530, sibType26a-v1610, sibType27-v1610, sibType28-v1610, sibType29-v1610 } SIB-Type-v12j0 ::= ENUMERATED { sibType19-v1250, sibType20-v1310, sibType21-v1430, sibType24-v1530, sibType25-v1530, sibType26-v1530, sibType26a-v1610, sibType27-v1610, sibType28-v1610, sibType29-v1610, sibType30-v1700, sibType31-v1700, sibType32-v1700, sibType33-v1800, spare2, spare1, ...} SI-Periodicity-r12 ::= ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512} SystemInfoValueTagList-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13 SystemInfoValueTagSI-r13 ::= INTEGER (0..3) CellSelectionInfo-v920 ::= SEQUENCE { q-QualMin-r9 Q-QualMin-r9, q-QualMinOffset-r9 INTEGER (1..8) OPTIONAL -- Need OP } CellSelectionInfo-v1130 ::= SEQUENCE { q-QualMinWB-r11 Q-QualMin-r9 } CellSelectionInfo-v1250 ::= SEQUENCE { q-QualMinRSRQ-OnAllSymbols-r12 Q-QualMin-r9 } CellAccessRelatedInfo-r14 ::= SEQUENCE { plmn-IdentityList-r14 PLMN-IdentityList, trackingAreaCode-r14 TrackingAreaCode, cellIdentity-r14 CellIdentity } CellAccessRelatedInfo-5GC-r15 ::= SEQUENCE { plmn-IdentityList-r15 PLMN-IdentityList-r15, ran-AreaCode-r15 RAN-AreaCode-r15 OPTIONAL, -- Need OR trackingAreaCode-5GC-r15 TrackingAreaCode-5GC-r15, cellIdentity-5GC-r15 CellIdentity-5GC-r15 } CellIdentity-5GC-r15 ::= CHOICE{ cellIdentity-r15 CellIdentity, cellId-Index-r15 INTEGER (1..maxPLMN-r11) } TrackingAreaList-r17 ::= SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode PosSchedulingInfoList-r15 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF PosSchedulingInfo-r15 PosSchedulingInfo-r15 ::= SEQUENCE { posSI-Periodicity-r15 ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}, posSIB-MappingInfo-r15 PosSIB-MappingInfo-r15 } PosSIB-MappingInfo-r15 ::= SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r15 PosSIB-Type-r15 ::= SEQUENCE { encrypted-r15 ENUMERATED { true } OPTIONAL, -- Need OP gnss-id-r15 GNSS-ID-r15 OPTIONAL, -- Need OP sbas-id-r15 SBAS-ID-r15 OPTIONAL, -- Need OP posSibType-r15 ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType3-1, ..., posSibType1-8-v1610, posSibType2-20-v1610, posSibType2-21-v1610, posSibType2-22-v1610, posSibType2-23-v1610, posSibType2-24-v1610, posSibType2-25-v1610, posSibType4-1-v1610, posSibType5-1-v1610, posSibType1-9-v1700, posSibType1-10-v1700, posSibType2-17a-v1770, posSibType2-18a-v1770, posSibType2-20a-v1770, posSibType1-11-v1800, posSibType1-12-v1800, posSibType2-26-v1800, posSibType2-27-v1800 }, ... } -- ASN1STOP SystemInformationBlockType1 field descriptions. bandwithReducedAccessRelatedInfo Access related information for BL UEs and UEs in CE. NOTE 3.. campingAllowedInCE Indicates whether non-BL UE is allowed to camp in the non-standalone BL cell in enhanced coverage mode when S-criterion for normal coverage is fulfilled. The field is not applicable for standalone BL cell.. category0Allowed The presence of this field indicates category 0 UEs are allowed to access the cell.. cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 4.. cellAccessRelatedInfoList-5GC This field contains a PLMN list and a list allowing signalling of access related information per PLMN for PLMNs that provides connectivity to 5GC. One PLMN can be included in only one entry of this list. NOTE4. cellBarred, cellBarred-CRS barred means the cell is barred, as defined in TS 36.304 [4].. cellBarred-5GC, cellBarred-5GC-CRS barred means the cell is barred for connectivity to 5GC, as defined in TS 36.304 [4].. cellBarred-NTN barred means the cell is barred for connectivity to NTN, as defined in TS 36.304 [4]. E-UTRAN always includes cellBarred-NTN and sets cellBarred to 'barred' in an NTN cell.. cellIdentity Indicates the cell identity. NOTE 2.. cellId-Index The index of the cell ID in the PLMN lists for EPC, indicates UE the corresponding cell ID is used for 5GC. Value 1 indicates the cell ID of the 1st PLMN list for EPC in the SIB1. Value 2 indicates the cell ID of the 2nd PLMN list for EPC, and so on.. cellReservedForOperatorUse, cellReservedForOperatorUse-CRS As defined in TS 36.304 [4].. cellSelectionInfoCE Cell selection information for BL UEs and UEs in CE. If absent, coverage enhancement S criteria is not applicable. NOTE 3.. cellSelectionInfoCE1 Cell selection information for BL UEs and UEs in CE supporting CE Mode B. E-UTRAN includes this IE only if cellSelectionInfoCE is present in SystemInformationBlockType1-BR. NOTE 3.. cp-CIoT-5GS-Optimisation Indicates whether the UE is allowed to establish the connection with Control plane CIoT 5GS optimisation, see TS 24.501 [95].. crs-IntfMitigConfig crs-IntfMitigEnabled indicates CRS interference mitigation is enabled for the cell, as specified in TS 36.133 [16], clause 3.6.1.1. For BL UEs supporting ce-CRS-IntfMitig, presence of crs-IntfMitigNumPRBs indicates CRS interference mitigation is enabled in the cell, as specified in TS 36.133 [16], clauses 3.6.1.2 and 3.6.1.3, and the value of crs-IntfMitigNumPRBs indicates number of PRBs, i.e. 6 or 24 PRBs, for CRS transmission in the central cell BW when CRS interference mitigation is enabled. For UEs not supporting this feature, the behaviour is undefined if this field is configured and the field cellBarred in SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs or UEs in CE) is set to notbarred.. csg-Identity Identity of the Closed Subscriber Group the cell belongs to.. csg-Indication If set to TRUE the UE is only allowed to access the cell if it is a CSG member cell, if selected during manual CSG selection or to obtain limited service, see TS 36.304 [4].. eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services via EPC for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS via EPC is not supported by the network in the cell. NOTE 2.. eCallOverIMS-Support5GC Indicates whether the cell supports eCall over IMS services via 5GC as defined in TS 23.401 [41]. If absent, eCall over IMS via 5GC is not supported by the network in the cell. NOTE 2.. eDRX-Allowed The presence of this field indicates if idle mode extended DRX is allowed in the cell for the UE connected to EPC. The UE shall stop using extended DRX in idle mode if eDRX-Allowed is not present when connected to EPC.. eDRX-Allowed-5GC The presence of this field indicates if idle mode extended DRX is allowed in the cell for the UE connected to 5GC. The UE shall stop using extended DRX in idle mode if eDRX-Allowed-5GC is not present when connected to 5GC.. encrypted The presence of this field indicates that the posSibType is encrypted as specified in TS 36.355 [54].. fdd-DownlinkOrTddSubframeBitmapBR The set of valid subframes for FDD downlink or TDD transmissions, see TS 36.213 [23]. If this field is present, SystemInformationBlockType1-BR-r13 is transmitted in RRCConnectionReconfiguration, and if RRCConnectionReconfiguration does not include systemInformationBlockType2Dedicated, UE may assume the valid subframes in fdd-DownlinkOrTddSubframeBitmapBR are not indicated as MBSFN subframes. If this field is not present, the set of valid subframes is the set of non-MBSFN subframes as indicated by mbsfn-SubframeConfigList. If neither this field nor mbsfn-SubframeConfigList is present, all subframes are considered as valid subframes for FDD downlink transmission, all DL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD DL transmission, and all UL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD UL transmission. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission.. fdd-UplinkSubframeBitmapBR The set of valid subframes for FDD uplink transmissions for BL UEs, see TS 36.213 [23]. If the field is not present, then UE considers all uplink subframes as valid subframes for FDD uplink transmissions. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission.. freqBandIndicatorPriority If the field is present and supported by the UE, the UE shall prioritize the frequency bands in the multiBandInfoList field in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator field. Otherwise, the UE applies frequency band according to the rules defined in multiBandInfoList. NOTE 2.. freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs, TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs and TS 36.102 [113], table 6.2A.3-1, for NTN capable UE, for the frequency band in freqBandIndicator. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10.. freqHoppingParametersDL Downlink frequency hopping parameters for BR versions of SI messages, MPDCCH/PDSCH of paging, MPDCCH/PDSCH of RAR/Msg4 and unicast MPDCCH/PDSCH. If not present, the UE is not configured downlink frequency hopping.. gnss-ID The presence of this field indicates that the posSibType is for a specific GNSS.. hsdn-Cell This field indicates this is a HSDN cell as specified in TS 36.304 [4].. hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around.. iab-Support This field combines both the support of IAB-node and the cell status for IAB-node. If the field is present, the cell supports IAB-nodes and the cell is also considered as a candidate for cell (re)selection for IAB-nodes; if the field is absent, the cell does not support IAB and/or the cell is barred for IAB-node.. ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services via EPC for UEs in limited service mode. If absent, IMS emergency call via EPC is not supported by the network in the cell for UEs in limited service mode. NOTE 2.. ims-EmergencySupport5GC Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode via 5GC. If absent, IMS emergency call via 5GC is not supported by the network in the cell for UEs in limited service mode. NOTE 2.. intraFreqReselection Used to control cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 36.304 [4]. NOTE 2.. multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42], table 5.5-1 and TS 36.102 [113], table 5.2-1, for NTN capable UE that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE.. multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs, TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs and TS 36.102 [113], table 6.2A.3-1, for NTN capable UE, for the frequency bands in multiBandInfoList (i.e. without suffix) and multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0.. plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN. If plmn-IdentityList-v1530 is included, E-UTRAN includes the same number of entries, and listed in the same order, as in plmn-IdentityList (without suffix). If plmn-IdentityList-v1610 is included, E-UTRAN includes the same number of entries, and listed in the same order, as in plmn-IdentityList-r15. If plmn-IdentityList-v1700 is included, E-UTRAN includes the same number of entries, and listed in the same order, as in plmn-IdentityList (without suffix). NOTE 2.. plmn-Index Index of the PLMN in the plmn-IdentityList fields included in SIB1 for EPC, indicating the same PLMN ID is connected to 5GC. Value 1 indicates the 1st PLMN in the 1st plmn-IdentityList included in SIB1, value 2 indicates the 2nd PLMN in the same plmn-IdentityList, or when no more PLMNs are present within the same plmn-IdentityList, then the PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on. NOTE 6.. p-Max Value applicable for the cell. If absent the UE applies the maximum power according to its capability as specified in TS 36.101 [42], clause 6.2.2. NOTE 2. This field is ignored by IAB-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [107].. posSchedulingInfoList-BR Indicates additional scheduling information of positioning SI messages for BL UEs and UEs in CE. E-UTRAN always includes this field if posSchedulingInfoList-r15 is included in SystemInformationBlockType1-BR, and includes the same number of entries, and listed in the same order, as in posSchedulingInfoList-r15.. posSIB-MappingInfo List of the posSIBs mapped to this SystemInformation message.. posSibType The positioning SIB type is defined in TS 36.355 [54].. q-QualMin Parameter "Qqualmin" in TS 36.304 [4]. If cellSelectionInfo-v920 is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1.. q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1.. q-QualMinOffset Parameter "Qqualminoffset" in TS 36.304 [4]. Actual value Qqualminoffset = field value [dB]. If cellSelectionInfo-v920 is not present or the field is not present, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell.. q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1.. q-RxLevMinOffset Parameter Qrxlevminoffset in TS 36.304 [4]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell.. satelliteOperation The presence of this field indicates that the current cell coverage is via a satellite.. sbas-ID The presence of this field indicates that the posSibType is for a specific SBAS.. schedulingInfoList Indicates scheduling information of SI messages. The schedulingInfoList-v12j0 (if present) provides additional SIBs mapped into the SI message scheduled via schedulingInfoList (without suffix). If E-UTRAN includes schedulingInfoList-v12j0, it includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix).. schedulingInfoListExt Indicates scheduling information of additional SI messages. The UE concatenates the entries of schedulingInfoListExt to the entries in schedulingInfoList, according to the general concatenation principles for list extension as defined in 5.1.2. If the schedulingInfoListExt is present, E-UTRAN ensures that the total number of entries of this field plus schedulingInfoList (without suffix) shall not exceed the value of maxSI-Message.. sib-MappingInfo List of the SIBs mapped to this SystemInformation message. There is no mapping information of SIB2; it is always present in the first SystemInformation message listed in the schedulingInfoList (without suffix) list. If present, sib-MappingInfo-v12j0 indicates one or more additional SIBs mapped to the concerned SI message listed in the schedulingInfoList (without suffix) list. If schedulingInfoList-v12j0 or schedulingInfoListExt-r12 is present, E-UTRAN does not include any value indicating SIB of type 19 or higher in sib-MappingInfo (without suffix). If schedulingInfoList-v12j0 is present, E-UTRAN ensures that the total number of entries of this field plus sib-MappingInfo (without suffix) shall not exceed the value of maxSIB-1.. si-HoppingConfigCommon Frequency hopping activation/deactivation for BR versions of SI messages and MPDCCH/PDSCH of paging.. si-Narrowband This field indicates the index of a narrowband used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.211 [21], clause 6.4.1 and TS 36.213 [23], clause 7.1.6. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..maxAvailNarrowBands-r13-1) as specified in TS 36.211 [21].. si-RepetitionPattern Indicates the radio frames within the SI window used for SI message transmission. Value everyRF corresponds to every radio frame, value every2ndRF corresponds to every 2 radio frames, and so on. The first transmission of the SI message is transmitted from the first radio frame of the SI window.. si-Periodicity, posSI-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on. If the si-posOffset is configured, the posSI-Periodicity of rf8 cannot be used.. si-posOffset This field, if present and set to true indicates that the SI messages in PosSchedulingInfoList are scheduled with an offset of 8 radio frames compared to SI messages in SchedulingInfoList. si-posOffset may be present only if the shortest configured SI message periodicity for SI messages in SchedulingInfoList is 80ms.. si-TBS This field indicates the transport block size information used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.213 [23], Table 7.1.7.2.1-1, for a 6 PRB bandwidth and a QPSK modulation.. schedulingInfoList-BR Indicates additional scheduling information of SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix).. si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h.. si-WindowLength, si-WindowLength-BR Common SI scheduling window for all SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on. In case si-WindowLength-BR-r13 is present and the UE is a BL UE or a UE in CE, the UE shall use si-WindowLength-BR-r13 and ignore the original field si-WindowLength (without suffix). UEs other than BL UEs or UEs in CE shall ignore the extension field si-WindowLength-BR-r13.. startSymbolBR For BL UEs and UEs in CE, indicates the OFDM starting symbol for any MPDCCH, PDSCH scheduled on the same cell except the PDSCH carrying SystemInformationBlockType1-BR, see TS 36.213 [23]. Values 1, 2, and 3 are applicable for dl-Bandwidth greater than 10 resource blocks. Values 2, 3, and 4 are applicable otherwise.. systemInfoValueTagList Indicates SI message specific value tags for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix).. systemInfoValueTagSI SI message specific value tag as specified in clause 5.2.1.3. Common for all SIBs within the SI message other than MIB, SIB1, SIB10, SIB11, SIB12, SIB14, SIB31 and SIB33.. systemInfoValueTag Common for all SIBs other than MIB, MIB-MBMS, SIB1, SIB1-MBMS, SIB10, SIB11, SIB12, SIB14, SIB31 and SIB33. Change of MIB, MIB-MBMS, SIB1 and SIB1-MBMS is detected by acquisition of the corresponding message.. tdd-Config Specifies the TDD specific physical channel configurations. NOTE 2.. trackingAreaCode/trackingAreaCode-5GC A trackingAreaCode that is common for all the PLMNs listed. NOTE2. NOTE 5.. trackingAreaList A list of tracking area codes for the PLMN listed. For the first entry in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area code in trackingAreaCode (without suffix) and the tracking area codes in trackingAreaList. If this field is absent, trackingAreaCode (without suffix) applies. For other entries in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area codes in trackingAreaList. If this field is absent, the list of tracking area codes of the preceding entry in plmn-IdentityList-v1700 applies. The total number of signalled tracking area codes across all PLMNs cannot be more than maxTAC-r17.. transmissionInControlChRegion Indicates, for BL UEs and UEs in CE, LTE control channel region may be used for DL broadcast transmission. NOTE 3.. up-CIoT-5GS-Optimisation Indicates whether the UE is allowed to resume the connection with User plane CIoT 5GS optimisation, see TS 24.501 [95]..