3GPP TSG-RAN WG2 #131	R2-2505460

India, Bangalore, August 25th – 29rd, 2025

Agenda item:	   8.3.2.1

Source:		Lenovo

Title:                             Functionality Management for measurement prediction

Document for:		Discussion and Decision

# Introduction

As instructed by the SID, the AI/ML for mobility SI will study the RRM measurement prediction and measurement event prediction. In this paper, we discuss essential functionality management aspects for RRM measurement prediction, considering its use cases and the respective scenarios of temporal, frequency, and spatial domain predictions.

# Discussion

For AI mobility use cases, initial agreements have been made in RAN #130 on functionality management based on the progress of AI/ML PHY BM use case. To apply AI/ML functionalities for inference, the respective configuration and thus the functionalities must be applicable from both the network and UE perspectives. Therefore, determining applicability is a crucial step for the smooth functioning of all AI/ML features. Both Option A and Option B for inference configuration or set of inference parameters are considered. Upon receiving the set of parameters for inference or the full inference configuration from the network, the UE is expected to report the applicability of the configured functionalities. It has been agreed in the last RAN2 meeting that, for AI/ML Mobility, UE reports the applicability via RRCReconfigurationComplete and/or UAI message.

Agreements

- Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case)

•	Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress)

•	Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

•	Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

•	Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes.

•	Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress).

•	No prohibit timer is introduced.

- Associated ID should be optionally configurable for training and  inference (i.e. it may not mandatorily required for all training/inference configurations).  FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

How UE can determine the applicability of an AI/ML functionality has been intensively discussed in RAN2, and it has been concluded that the applicability status depends on the configuration, network-side additional conditions, UE-side additional conditions, and the model availability at the UE. The network-side additional conditions and UE-side additional conditions are essential to maintain the consistency in training and inference of a functionality. The UE-side additional conditions can be internally known to the UE, which do not require specification impact. Whereas, network-side additional conditions, if available, must be considered necessary to decide the applicability status of the functionalities/configurations by the UE. We believe this criterion for applicability determination should be common to AI/ML framework and is not limited to certain use cases. It is beneficial for different scenarios and conditions experienced by the network, which can also be extended beyond the serving cell. Following the progress of AI/ML for Air interface, additional conditions are represented by associated ID(s) and as agreed in last RAN2 meeting, for AI Mobility: associated ID is optionally configurable for training and inference (it may not be mandatorily required for all training/inference). It is important to discuss the  impact of associated ID(s) on the applicability of functionality when network-side additional conditions changes during UE mobility. Associated ID(s) beyond cell-specific ID(s) will be useful for an uninterrupted transition in inference during/after handovers, and for efficient LCM processes, such as monitoring model performance.

Proposal 1: RAN2 to clarify for which cases the associated ID can be absent during training and inference.

Considering associated ID(s) is optional, the UE needs to determine applicability based on the respective network configuration, UE internal conditions, and model availability only.

Proposal 2. If associated ID is absent, UE determines applicability based on network configuration, UE internal conditions, and model availability.

As the UE reports applicability status in RRCReconfigurationComplete and subsequent update in UAI, it is not clear for which inference configurations the UE should include update in UAI: (a) only the ones configured in previous RRCReconfiguration message or the complete list of inference configuration that UE is configured with.

Proposal 3: UE reports the initial applicability report in RRCReconfigurationComplete for all inference configuration or inference related parameters configured in the respective RRCReconfiguration message.

Proposal 4: UE reports subsequent change in applicability via UAI for the inference configurations available at UE.

Along with the inapplicability reporting, UE further includes a single cause value of inapplicability. It has been unclear what this cause value can refer to, and the details of cause value are FFS based on progress in AI/ML PHY. In RAN #130, for AI/ML PHY, it was agreed to replace the inapplicability cause with a release configuration flag due to model inavailability in the local device. For example, if the UE does not have a possibility to load a trained model, it is reasonable that the respective configuration must be released by the network. A similar approach can be used inapplicability reporting for AI Mobility, as the cause value may not provide sufficient information to the network.

Proposal 5: A “release configuration” flag instead of an inapplicability cause is introduced to indicate UE’s preference to release a configuration.

## 2.1. RRM Measurement Prediction

The high-level categorization of RRM measurement prediction is done in two ways: prediction in the temporal domain, frequency domain, and spatial domain. At the same time, pure temporal prediction is studied as the baseline for measurement predictions.

The AI/ML for mobility SID suggests studying both UE-sided and NW-sided models for RRM measurement prediction. The two possible approaches that were studied in this SI are beam-level and cell-level predictions. As cell-level measurements are the focus of this study, they can be either generated or predicted based on the beam-level or cell-level inputs. RAN2 agreement lists three possible cases for cell-level RRM measurement prediction. It is important to discuss how LCM aspects apply to these different types of measurement prediction cases for NW-sided and UE-sided models.

For intra-cell measurement predictions with different cases, the final output will be predicted cell-level measurements, while the inputs will be different for types of measurement predictions (temporal/frequency/spatial). RAN2 is suggested to discuss whether temporal domain and frequency domain RRM measurement prediction can be applied, to generate respective inferences at the same time. The UE can determine applicability and report applicability for each case of measurement prediction individually (e.g., temporal prediction, frequency prediction, spatial prediction). This will be important to define the granularity of functionality.

Proposal 6: Temporal frequency and spatial domain predictions can be configured simultaneously, and UE can report their applicability/inapplicability individually in the applicability report.

For all cases of RRM measurement prediction, configuration from the network is always required, which is used for or in the process of measurement inference. However, the measurement configuration may be different for each case and depends on whether the cell-level measurements are predicted based on predicted beam-level results, cell-level results, or beam-level results of the same cell or a different cell. In some cases, it may be a configuration of a set of beams, as agreed in the R19 AI/ML air interface for BM. Based on the progress of LCM in AI/ML air interface and agreements from the last meeting, the legacy measurement configuration framework can be used as a baseline for inference configuration and reporting, which can be extended further, considering different cases of measurement prediction.

The network configuration will differ for the different cases of RRM measurement prediction. UE can be configured with different periodic configurations for RRM measurement predictions, for example, periodic temporal domain inference configuration and periodic frequency domain inference configuration. Upon receiving inference configurations, UE will determine their applicability individually for each case, and thus, one or more periodic inference configurations can be applicable. For certain applicable configurations, UE can start inference immediately upon receiving the configuration and sending the applicability report; whereas, UE can be configured to trigger inference upon an event being fulfilled, e.g., based on radio conditions thresholds. It would be beneficial for RAN2 to discuss the UE behaviour in this case and activation for inference of such applicable inference configurations, and how multiple applicable configurations can be handled. Each set of inference configuration for the sub-cases of measurement prediction can be associated with an individual identifier (e.g., measId) such that independent measurement configurations for the respective cases/sub-cases can be activated and reported, if applicable.

Proposal 7: After receiving the full inference configuration for measurement prediction and reporting the applicable AI/ML functionalities, the inference can be triggered by UE immediately (e.g., for periodic configurations) or upon a certain event.

Proposal 8: RAN2 clarifies if multiple periodic RRM measurement predictions (e.g., temporal/spatial predictions) can be activated at the same time.

Proposal 9: For RRM measurement prediction, one set of inference configuration can be identified with a measId.

Based on the agreements on RAN2#130 meeting, for UE-sided model monitoring, UE may generate monitoring results by comparing inference output and monitoring data, and report the monitoring result to NW. The ideal case is that both an inference output and the corresponding monitoring data are derived at the same time. In other words, the time point (or time period) of inference output and monitoring data is in perfect alignment. However, due to the limited UE capability, there might be a misalignment between the time instance of the inference output and the monitoring (measured) data. For example, at time point t0, there is an inference output, while for the reason that UE may request uplink grant for uplink data transmission at time point t0, hence in this case, the generation of the monitoring data, e.g., delay 0.5 ms or 1 ms, etc., may be delayed. For a NW-sided model, the misalignment is not considered as a problem that needs a standard solution, since the performance metrics are up to NW implementation, e.g., choosing which monitoring data and inference output to use for comparison. However,  for a UE-sided model case, RAN2 should discuss the misalignment issue, e.g., how to generate the monitoring result by determining which monitoring data and inference output to compare.

Proposal 10: For UE-side monitoring of RRM prediction, RAN2 studies the case when the time instance of a prediction result and the time instance of a measured result (that can be used to compare with the prediction result) are not aligned.

# Conclusion

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

Proposal 1: RAN2 to clarify for which cases the associated ID can be absent during training and inference.

Proposal 2. If associated ID is absent, UE determines applicability based on network configuration, UE internal conditions, and model availability.

Proposal 3: UE reports the initial applicability report in RRCReconfigurationComplete for all inference configuration or inference related parameters configured in the respective RRCReconfiguration message.

Proposal 4: UE reports subsequent change in applicability via UAI for the inference configurations available at UE.

Proposal 5: A “release configuration” flag instead of an inapplicability cause is introduced to indicate UE’s preference to release a configuration.

RRM Measurement prediction

Proposal 6: Temporal frequency and spatial domain predictions can be configured simultaneously, and UE can report their applicability/inapplicability individually in the applicability report.

Proposal 7: After receiving the full inference configuration for measurement prediction and reporting the applicable AI/ML functionalities, the inference can be triggered by UE immediately (e.g., for periodic configurations) or upon a certain event.

Proposal 8: RAN2 clarifies if multiple periodic RRM measurement predictions (e.g., temporal/spatial predictions) can be activated at the same time.

Proposal 9: For RRM measurement prediction, one set of inference configuration can be identified with a measId.

Proposal 10: For UE-side monitoring of RRM prediction, RAN2 studies the case when the time instance of a prediction result and the time instance of a measured result (that can be used to compare with the prediction result) are not aligned.

# References

[1] RP-250971, TR 38.744 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

[3] TS 38.300 running CR for AI for air interface