3GPP TSG-RAN2#131 	R2-2505464

Bengaluru, India, 25th Aug – 29th Aug, 2025

Agenda item:		8.12.3 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on UEI beam reporting impact

Document for:	Discussion and Decision

1.	Introduction

During post e-mail discussion [1], open issues were discussed, and RAN2 made some consensus for almost issues, but some issue seems to need further discussion.

In this contribution, we present our view on remaining issues.

2.	Discussion

In the last meeting, one problematic case was raised that at the time UEIBR (UE initiated Beam Reporting) is initiated, there may be no valid PUCCH/PUSCH resource to transmit UEIBR due to TAT expiry.

In [1], RAN2 discussed this case with some options and down-selected two options as follows.

  - Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts.

  - Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH. Discuss how to indicate the cause of RACH (e.g., MAC CE in Msg3).

In Option 1, the UE does nothing until UL resynchronization or initiates RACH if there is UL data or BFR. This is aligned with legacy behaviour upon TAT expiry.

In Option 2, the UE initiates RACH if UEIBR is initiated upon TAT expiry. Currently, RACH is not initiated for CSI reporting, so this introduces some specification change.

We think that Option 1 is aligned with the basic principle. Currently, when TAT expires, the UE initiates RACH only for UL data arrival (via BSR -> SR -> RACH) or BFR, not for A/SP/P CSI reporting. We think that this is because UL data and Beam Failure are considered important, whereas A/SP/P CSI are not. In our understanding, UEIBR is not as important or urgent information as UL data or BFR. Thus, we think that RACH initiation for UEIBR, i.e. Option 2, is too much solution.

In addition, companies supporting Option 2 argued that the new resource for UEIBR can be reconfigured as early as possible. However, we don’t see strong motivation to prematurely reconfigure the new resource for UEIBR.

TAT expiry may happen when there is no data transmission. Otherwise, the network would control TAT from expiring. Anyway, if UL data is occurred or BFR is happened, the UE initiates RACH and beam can be recovered. Thus, it is unclear why UEIBR resource is reconfigured prematurely and beam management continuously performed, even though there is no data transmission. Even if RACH is initiated for UEIBR, the UEIBR is likely to be outdated information after reconfiguring the new resource for UEIBR.

Proposal 1. When UEIBR is initiated but TAT expires, UE does not initiate RACH as in legacy.

Currently, UEIBR is triggered in PHY layer and PHY layer does not know whether TAT expires or not. Thus, when TAT expires but UEIBR is initiated, MAC layer needs to indicate to cancel the triggered UEIBR.

Proposal 2. When UEIBR is initiated but TAT expires, MAC indicates to cancel the triggered UEIBR to PHY layer.

3.	Conclusion

In this document, we present our view on open issue, and made proposals as follows.

Proposal 1. When UEIBR is initiated but TAT expires, UE does not initiate RACH as in legacy.

Proposal 2. When UEIBR is initiated but TAT expires, MAC indicates to cancel the triggered UEIBR to PHY layer.

4.	Reference

- [Post130][219][MIMO_Ph5] Running CR for 38.321 (Samsung)