# 3GPP TSG RAN WG2 Meeting #131      	                           R2-2505500
Bengaluru, Indian, August 25 – 29, 2025

Agenda item:	8.5.4

Source:	Apple

Title:	Remaining issues on common signal transmission adaptation

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective of adaptation of common signal transmission includes paging adaptation, PRACH adaptation and SSB adaptation, as copied below:

By RAN2#130 [2], good progress of common signal adaptation was made. However, there are still some leftover issues on common signal adaptation. In this contribution, we further discuss remaining issues on adaptation of common signal transmission.

# 2 Discussion

## 2.1 Paging adaptation

In RAN2#130 [2], the following agreements on configuration of Rel-19 paging adaptation was made:

Agreements on Rel-19 paging adaptation configuration

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

- The maximum offset value is extended to 32 radio frames.

- The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

And the following agreements on configuration of Rel-19 PEI was made:

Agreements on Rel-19 PEI configuration

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum number of PEI occasion per paging frame is extended to 8.

- The maximum offset value is extended to 32 radio frames.

However, the detailed formats of these two Rel-19 new paging configurations are still controversial in post-meeting email discussion of RRC running CR [3]. They are related to the following RRC open issues identified by RRC CR Rapporteur:

- RRC open issue 1: Detailed format of firstPDCCH-MonitoringOccasionOfPO-r19 (Q13/Samsung003/ LGE003/Apple003)

- RRC open issue 2: Detailed format of firstPDCCH-MonitoringOccasionOfPEI-O-r19 (Q14/Samsung003 LGE003/Apple003)

- RRC open issue 3: Whether any other Rel-19 PEI configuration (Q15)

- RRC open issue 4: Whether group paging adaptation configuration (Samsung004)

- RRC Open issue 5: Whether support paging adaptation in non-initial BWP for (e)redcap UE (OPPO005)

We provide our view on above RRC open issues in this section.

### 2.1.1 Open issue 1: detailed format of firstPDCCH-MonitoringOccasionOfPO-r19 (Q13/Samsung003/LGE003/Apple003)

This issue can be described with Q13a:

- Q13a. Please comment on whether the maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to 32 radio frames: - at symbol level - at slot level - by restricting the maximum value range of pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 field parameter for different SCS corresponding to pagingAdaptationNAndPagingFrameOffset-r19 (i.e. N) - by restricting the configuration to evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them. - other

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                   SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneT-SCS15KHZhalfT                                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT              SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZoneThirtySecondT       SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-SCS60KHZoneThirtySecondT  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneEighthT-sCS120KHZoneThirtySecondT                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneSixteenthT                                              SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

}                                                                                                         OPTIONAL           -- Need R

pagingAdaptationNAndPagingFrameOffset

Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]) for paging adaptation. A value of oneThirtySecondT corresponds to T/32, a value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on.

If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]):

- for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

- for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

- for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

- for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT, oneThirtySecondT}

- for ssb-periodicityServingCell of 160 ms, N can be set to {oneSixteenthT, oneThirtySecondT}

If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

Please note that existing RRC running CR has implemented above solution iii:

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                           SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                            SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}

firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE {

sCS480KHZoneEighthT                      SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

firstPDCCH-MonitoringOccasionOfPO Indicates the first PDCCH monitoring occasion of each PO of the PF on this BWP, see TS 38.304 [20]. The field sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS120KHZoneSixteenthT can be applied for SCS 480kHz, corresponding to sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS480KHZquarterT-SCS120KHZoneSixteenthT in IE DownlinkConfigCommonSIB respectively.

We think it is unnecessary to mandate above NW restriction related to ssb-periodicityServingCell when pagingSearchSpace is set to zero, which can be left to NW implementation in legacy as illustrated as follows (i.e. legacy spec doesn’t capture any NW configuration restriction in its field description).

Observation 1: It is unnecessary to capture any NW configuration restriction related to ssb-periodicityServingCell when pagingSearchSpace is set to zero, which can be left to NW implementation as in legacy.

Regarding Option ii (introduce slot level offset rather than existing symbol level offset in firstPDCCH-MonitoringOccasionOfPO), it is an optimization for RRC overhead reduction. However, as it was agreed to only extend N to 32 which is just double from legacy, the RRC overhead is not an issue anymore.

Observation 2: As it was agreed to only extend N to 32 which is just double from legacy, the RRC overhead is not an issue anymore. Thus, no need to introduce slot level offset in firstPDCCH-MonitoringOccasionOfPO.

Option iv is also an optimization for RRC overhead reduction. Similar to Option i, we don’t think it needs to be pursued because RRC overhead is not an issue anymore.

Based on above analysis, we prefer Option i to keep the same offset granularity of per symbol as legacy. To implement it, we identify two gaps to use legacy firstPDCCH-MonitoringOccasionOfPO to support Rel-19 paging adaptation:

- As highlighted above, legacy firstPDCCH-MonitoringOccasionOfPO only supports up to 4 PO per PF. However, Rel-19 paging adaptation supports up to 8 PO per PF. Thus, we need to extend maxPO-perPF-r19 to 8.

- As highlighted above, legacy firstPDCCH-MonitoringOccasionOfPO only covers up to 16 frames because smallest N is 1/16 in legacy paging. As N can take 1/32 in Rel-19 paging adaptation, we need to introduce new IE with extended range to cover the case of 32 frames.

An ASN.1 example to implement above changes can be found below:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                         SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..143359)

}

maxPO-perPF-r19                             INTEGER ::= 8       -- Maximum number of paging occasion per paging frame

Thus, we propose:

Proposal 1 (RRC open issue Q13/Samsung003/ LGE003/Apple003): As legacy, keep IE firstPDCCH-MonitoringOccasionOfPO-r19 with symbol level offset and without NW restriction on ssb-periodicityServingCell. Adopt the following ASN.1 format to support Ns=8 and N=T/32:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                         SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..143359)

}

maxPO-perPF-r19                             INTEGER ::= 8       -- Maximum number of paging occasion per paging frame

### 2.1.2 Open issue 2: detailed format of firstPDCCH-MonitoringOccasionOfPEI-O-r19 (Q14/Samsung003/LGE003/Apple003)

This issue is similar to open issue 1 on firstPDCCH-MonitoringOccasionOfPO. We think the legacy firstPDCCH-MonitoringOccasionOfPEI-O can’t well support the case of Ns=8 in the new PEI-O configuration:

- Legacy firstPDCCH-MonitoringOccasionOfPEI-O only supports up to 4 PO per PF. However, Rel-19 paging adaptation supports up to 8 PO per PF. Thus, we need to extend maxPEI-perPF-r19 to 8.

- Legacy firstPDCCH-MonitoringOccasionOfPEI-O only covers up to 16 frames because smallest N is 1/16 in legacy paging. As N can take 1/32 in Rel-19 paging adaptation, we need to introduce new IE with extended range to cover the case of 32 frames.

An ASN.1 example to implement above changes can be found below:

firstPDCCH-MonitoringOccasionOfPEI-O-r19  CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..143359)

}

maxPEI-perPF-r19                        INTEGER ::= 8       -- Maximum number of PEI occasion per paging frame

Thus, we propose:

Proposal 2 (RRC open issue Q14/Samsung003/ LGE003/Apple003): As legacy, keep IE firstPDCCH-MonitoringOccasionOfPEI-O-r19 with symbol level offset. Adopt the following ASN.1 format to support up to 8 PEI-O per PF and 32 radio frames:

firstPDCCH-MonitoringOccasionOfPEI-O-r19  CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..143359)

}

maxPEI-perPF-r19                        INTEGER ::= 8       -- Maximum number of PEI occasion per paging frame

### 2.1.3 Open issue 3: whether any other Rel-19 PEI configuration (Q15)

This issue is:

- Q15a: Please comment on whether the following parameters should be introduced for Rel-19 PEI configuration: - po-NumPerPEI-r19 - payloadSizeDCI-2-7-r19 - pei-FrameOffset-r19 - subgroupsNumPerPO-r19 - subgroupsNumForUEID-r19

We don’t think any new IE is needed.

Proposal 3 (RRC open issue Q15): No need to introduce new parameters for Rel-19 PEI configuration.

### 2.1.4 Open issue 4: Whether group paging adaptation configuration (Samsung004)

The issue is:

| Samsung 004 | #### pagingAdaptation-NS-r19, pagingAdaptationNAndPagingFrameOffset and firstPDCCH-MonitoringOccasionOfPO should be grouped together in one IE. pagingAdaptation-NS-r19, pagingAdaptationNAndPagingFrameOffset should be mandatory inside the IE. pagingAdaptation-r19 ::= SEQUENCE { pagingAdaptation-NS-r19 ENUMERATED {eight,four, two, one} pagingAdaptationNAndPagingFrameOffset-r19 CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15), oneThirtySecondT INTEGER (0..31) } firstPDCCH-MonitoringOccasionOfPO-r19 CHOICE { } OPTIONAL -- Need R } OPTIONAL -- Need R | Open issue |
| --- | --- | --- |

We agree with Samsung004 because it is invalid case if NW only provides firstPDCCH-MonitoringOccasionOfPO-r19   but doesn’t provide pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset.

Proposal 4 (RRC open issue Samsung004): Adopt Samsung004 as follows:

pagingAdaptation-r19 ::=             SEQUENCE {

pagingAdaptation-NS-r19                  ENUMERATED {eight,four, two, one}

pagingAdaptationNAndPagingFrameOffset-r19        CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15),

oneThirtySecondT                     INTEGER (0..31)

}

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

}        OPTIONAL           -- Need R

} OPTIONAL           -- Need R

### 2.1.5 Open issue 5: whether support paging adaptation in non-initial BWP for (e)redcap UE (OPPO005)

In current RRC running CR [4], the IE pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 is captured in both DownlinkConfigCommonSIB and PDCCH-ConfigCommon, where the later one is used for Rel-19 paging in initialDownlinkBWP-RedCap.

PDCCH-ConfigCommon ::=              SEQUENCE {

controlResourceSetZero              ControlResourceSetZero                                  OPTIONAL,   -- Cond InitialBWP-Only

commonControlResourceSet            ControlResourceSet                                      OPTIONAL,   -- Need R

searchSpaceZero                     SearchSpaceZero                                         OPTIONAL,   -- Cond InitialBWP-Only

Omit unrelated part

pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneThirtySecondT                                        SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneThirtySecondT                                        SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneThirtySecondT                                        SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneThirtySecondT                                       SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneThirtySecondT                                       SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

}                                                                                           OPTIONAL,    -- Cond OtherBWP

Omit unrelated part

}

| OtherBWP | This field is optionally present, Need R, if this BWP is not the initialDownlinkBWP and pagingSearchSpace is configured in this BWP. Otherwise this field is absent. |
| --- | --- |

This issue is also captured as one open issue of 38.304 running CR [4]:

[304 open issue 1] Samsung004: whether firstPDCCH-MonitoringOccasionOfPO-r19 can be signaled for paging in non-initial BWP for (e)redcap UE.

As background, one cell supporting (e)redcap can provide two initial BWPs: one is normal initial BWP applicable for all UEs, and the other one is (e)redcap specific initial BWP applicable only for (e)redcap UE. The (e)redcap UE may select to camp in the redcap specific initial DL BWP which contains CORESET0 and CD-SSB. In this case, (e)redcap UE receives paging in redcap specific initial DL BWP and initiates RACH on (e)redcap specific initial UL BWP upon reception of paging.

Observation 3: If (e)redcap UE camps in the redcap specific initial DL BWP which contains CORESET0 and CD-SSB, it receives paging in redcap specific initial DL BWP and initiates RACH on (e)redcap specific initial UL BWP upon reception of paging.

We think the spec impact to enable paging adaptation in redcap specific initial DL BWP only involves RAN2 and seems to be manageable in Rel-19. However, we do not agree to extend to enable SSB adaptation and RACH adaptation in redcap specific initial BWP, which needs both RAN1 and RAN2 specification work (e.g. whether paging short message indicates RACH adaptation in initial BWP or redcap specific initial BWP). Thus, we propose:

Proposal 5 (RRC open issue OPPO005, 304 open issue 1): Support paging adaptation in initialDownlinkBWP-RedCap-r17 for (e)redcap UE by capturing pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 in PDCCH-ConfigCommon and TS 38.304. RAN2 do not pursue further optimization of common signal adaptation for (e)redcap UE.

## 2.2 Adaptation of PRACH in time domain

### 2.2.1 How to disable RACH adaptation for MSG1 repetition

In RAN2#130 [2], it was discussed How to disable RACH adaptation for Msg1 repetition. Two options were identified but no conclusion is made:

RAN2 will select one of the following options. (CE here means MSG1 repetition).

- Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

- Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17)

We think both options can work. Between them, Option-1c relies on NW implementation to avoid mixing CE and non-CE RACH adaptation whose spec impact is smaller (i.e. only capturing configuration restriction in TS 38.331). While Option-1b needs further spec changes in TS 38.321. As we prefer to avoid UE impacts, we slightly prefer Option 1c.

Proposal 6: On how to disable RACH adaptation for MSG1 repetition, adopt Option-1c (i.e. rely on NW implementation to avoid mixing CE and non-CE RACH adaptation in the same RACH-ConfigCommon).

## 2.3 SSB adaptation

### 2.3.1 RRC open issue 6: Xiaomi004

This issue is:

| Xiaomi004 | Parameters related to SSB adaptation is not captured | Open issue |
| --- | --- | --- |

It seems that RRC parameters of SSB adaptation and procedure text on L3 measurement are not captured yet. Thus, we propose to capture it.

Proposal 7 (RRC open issue Xiaomi004): Capture RRC parameters of SSB adaptation and procedure text on L3 measurement in running RRC CR.

# 3 Conclusion

In this contribution, we further discuss remaining issues on common signal transmission adaptation. Our observations are:

Observation 1: It is unnecessary to capture any NW configuration restriction related to ssb-periodicityServingCell when pagingSearchSpace is set to zero, which can be left to NW implementation as in legacy.

Observation 2: As it was agreed to only extend N to 32 which is just double from legacy, the RRC overhead is not an issue anymore. Thus, no need to introduce slot level offset in firstPDCCH-MonitoringOccasionOfPO.

Observation 3: If (e)redcap UE camps in the redcap specific initial DL BWP which contains CORESET0 and CD-SSB, it receives paging in redcap specific initial DL BWP and initiates RACH on (e)redcap specific initial UL BWP upon reception of paging.

Based on observations, our proposals are:

Paging adaptation

Proposal 1 (RRC open issue Q13/Samsung003/ LGE003/Apple003): As legacy, keep IE firstPDCCH-MonitoringOccasionOfPO-r19 with symbol level offset and without NW restriction on ssb-periodicityServingCell. Adopt the following ASN.1 format to support Ns=8 and N=T/32:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                         SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..143359)

}

maxPO-perPF-r19                             INTEGER ::= 8       -- Maximum number of paging occasion per paging frame

Proposal 2 (RRC open issue Q14/Samsung003/ LGE003/Apple003): As legacy, keep IE firstPDCCH-MonitoringOccasionOfPEI-O-r19 with symbol level offset. Adopt the following ASN.1 format to support up to 8 PEI-O per PF and 32 radio frames:

firstPDCCH-MonitoringOccasionOfPEI-O-r19  CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..143359)

}

maxPEI-perPF-r19                        INTEGER ::= 8       -- Maximum number of PEI occasion per paging frame

Proposal 3 (RRC open issue Q15): No need to introduce new parameters for Rel-19 PEI configuration.

Proposal 4 (RRC open issue Samsung004): Adopt Samsung004 as follows:

pagingAdaptation-r19 ::=             SEQUENCE {

pagingAdaptation-NS-r19                  ENUMERATED {eight,four, two, one}

pagingAdaptationNAndPagingFrameOffset-r19        CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15),

oneThirtySecondT                     INTEGER (0..31)

}

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

}        OPTIONAL           -- Need R

} OPTIONAL           -- Need R

Proposal 5 (RRC open issue OPPO005, 304 open issue 1): Support paging adaptation in initialDownlinkBWP-RedCap-r17 for (e)redcap UE by capturing pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 in PDCCH-ConfigCommon and TS 38.304. RAN2 do not pursue further optimization of common signal adaptation for (e)redcap UE.

RACH adaptation

Proposal 6: On how to disable RACH adaptation for MSG1 repetition, adopt Option-1c (i.e. rely on NW implementation to avoid mixing CE and non-CE RACH adaptation in the same RACH-ConfigCommon).

SSB adaptation

Proposal 7 (RRC open issue Xiaomi004): Capture RRC parameters of SSB adaptation and procedure text on L3 measurement in running RRC CR.

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#130, Chair Notes.

[3] R2-250xxxx, Running RRC CR for enhancements for network energy efficiency, Ericsson.

[4] R2-2505497, Summary report of [POST130][108][NES] 38.304 CR (Apple), Apple.