3GPP TSG-RAN WG2 Meeting #131	R2-2505505

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.7.5

Source:	Huawei, HiSilicon

Title:	Discussion on RLC AM enhancements

Document for: Discussion and Decision

# 1	Introduction

In the Rel-19 XR WID approved in RAN#103 [1], one of the objectives is about the RLC re-transmission enhancements for RLC AM. In this contribution, we will provide our views on unnecessary retransmission avoidance and timely RLC retransmission.

# 2. Discussion

The following agreements related to timely RLC retransmissions were reached in RAN2#130 [2] meeting as given below:

- (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this - (RLC-9) No additional conditions are needed for the polling enhancement. - RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. - (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. - Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance - The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity. - (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

## 2.1	Enhancement on remaining time based retransmission

It was agreed that the discardTimer maintained in PDCP layer can be reused to determine whether the RLC SDU (segment) that has been submitted to the MAC entity can be considered for remaining time based retransmission when the corresponding discardTimer of the RLC SDU becomes below a threshold before the associated feedback is received. To be more specific, when the remaining time of the running discardTimer of an PDCP SDU becomes below the configured threshold, the transmitting PDCP entity can send an indication to the Tx RLC entity for triggering remaining time based retransmission. Upon reception of the indication, the Tx RLC entity can consider the RLC SDU (segment) for retransmission if it has been transmitted, but the associated feedback was not yet received.

Furthermore, although it was agreed that RLC SDU (segment) can be retransmitted at most once, the co-existence between the legacy NACK based retransmission and the newly introduced discardTimer based retransmission will inevitably introduce redundant retransmission and capacity loss. For example, there can be a case that the RLC SDU (segment) is just retransmitted, e.g. due to NACK, right before the remaining time based retransmission is triggered. As shown in Figure 1, if an RLC SDU (segment) has been retransmitted because of the associated NACK, it may be retransmitted soon due to the remaining time based retransmission, provided that its associated ACK has not been received before the reception of the retransmission indication.

Observation 1: Retransmission of an RLC SDU (segment) can be triggered by both associated NACK and the remaining time based retransmission indication, and resultant redundant retransmission will lead to capacity loss.

We believe that one ARQ retransmission for RLC SDU (segment) can ensure the transmission reliability sufficiently, regardless of whether it was triggered by NACK or remaining time based retransmission. Therefore, to avoid such repeated retransmission and resource waste, it is suggested that if the associated NACK of an RLC SDU (segment) was already received (meaning that the RLC SDU (segment) is/was already considered for retransmission), the remaining time based retransmission will not be triggered even when the retransmission indication is received.

Figure 1 remaining time based retransmission after legacy NACK based retransmission

Proposal 1:  Only RLC SDU or segment for which no feedback was received is considered for remaining time based retransmission by a Tx RLC entity, as given in Appendix A.

## 2.2	Enhancement on unnecessary retransmission avoidance

Moreover, according to the current running CR as shown below, the Tx RLC entity shall not consider the outdated RLC SDU (segment) for retransmission.

5.2.3 AM data transfer 5.2.3.1 Transmit operations 5.2.3.1.1 General … If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission or retransmission. … 5.3.2 Retransmission … When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; and - if stopReTxDiscardedSDU is configured and no discard indication for the RLC SDU has been received from upper layers, or if stopReTxDiscardedSDU is not configured: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. …

“Consider for retransmission” and “pending for retransmission” are different processing stages, where an RLC SDU (segment) is firstly considered for retransmission, then pending in retransmission buffer for grant of the retransmission. The current running CR remains unclear about what the Tx RLC entity should do if a discard indication is received when the RLC SDU (segment) is already pending for grant. We believe that the outdated RLC SDU (segment) should not be retransmitted as well, although it has been pending for retransmission. Additionally, as shown below [3], the RLC data PDUs that are pending for retransmission are counted for BSR or DSR data calculation, even though they may have been outdated.

5.5 Data volume calculation For the purpose of MAC buffer status reporting, the UE shall consider the following as RLC data volume: - RLC SDUs and RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs that are pending for initial transmission; - RLC data PDUs that are pending for retransmission (RLC AM). For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume: - delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs pending for initial transmission, and containing a del ay-critical RLC SDU or a delay-critical RLC SDU segment; - RLC data PDUs that are pending for retransmission (RLC AM).

Therefore, we believe that the pending status for an RLC SDU (segment) already pending for retransmission should be cancelled if the associated discard indication is received. The TP is provided in Appendix A.

Proposal 2:  RAN2 clarifies that the Tx RLC entity should cancel the pending status for the RLC SDU (segment) that has been pending for retransmission when its discard indication is received, as given in Appendix B.

## 2.3 Enhancement on polling triggering

There is one remaining issue to be further discussed from last meeting in how to address window stalling issue when t-PollRetransmit expires, as shown below:

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

The issue is raised by the introduction of unnecessary retransmission avoidance, for which it was agreed in the previous meeting that outdated RLC SDU should not be retransmitted. As a consequence, there is no available RLC SDU to include the poll information, when t-PollRetransmit expires if there is only outdated RLC SDU in the retransmission buffer. We believe this case is not essential. To solve the issue, based on our understanding, the simplest solution is to maintain the Rel-18 specification of the expiry of t-PollRetransmit. To be more specific, the newly introduced configuration stopReTxDiscardedSDU is not applicable for expiry of t-PollRetransmit (Clause 5.3.3.4), and as it is a rare case the potential overhead introduced, e.g. simply retransmitting one outdated RLC SDU, is fairly minor.

Proposal 3: (RLC-11) For specification simplicity, RAN2 maintains the specification of t-PollRetransmit expiry (Clause 5.3.3.4) unchanged as legacy, e.g. Rel-18.

Furthermore, the current running CR has captured the remaining time based polling triggering, as given below:

5.3.3 Polling 5.3.3.2 Transmission of a AMD PDU … Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); or - if an indication is received from upper layer (e.g., PDCP) that the condition for remaining-time-based RLC polling has been met for an RLC SDU: - include a poll in the AMD PDU as described below.

In the running CR, the remaining time based polling triggering relies on a prerequisite that there is an available AMD PDU to transmit upon notification of a transmission opportunity. However, there remains a case that no available RLC SDU can be transmitted when the remaining time based polling indication is received, e.g. both the transmission and retransmission buffer are empty. In such case, the Tx RLC entity can consider a specific RLC SDU for retransmission to include the poll. Finally, a TP is provided in the Appendix B.

Observation 2: As per running CR, the remaining time based polling triggering is dependent on whether there is an available AMD PDU to transmit.

Proposal 4: RAN2 considers the case when remaining time based polling is triggered but there is no available RLC SDU to transmit.

Proposal 5: If there is no available RLC SDU to carry the polling when the polling is triggered based on the remaining time, a specific RLC SDU, e.g. RLC SDU with the highest SN among the RLC SDUs submitted or any RLC SDU which has not been positively acknowledged, can be considered for retransmission, as given in Appendix C.

## 2.4 UE capability for unnecessary retransmission avoidance

- Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

As the combined solution was agreed where the Tx RLC entity can stop retransmitting the outdated RLC SDU and Rx RLC entity can stop receiving the outdated RLC SDU, the UE capability of whether supporting the both capabilities are discussed. As the Tx based solution cannot work individually in RLC AM, we suggest that the UE supporting Tx based feature shall also be able to support the Rx based feature.

Proposal 6: UE supporting Tx based feature shall also be able to support the Rx based feature.

# 3. Conclusion

This contribution makes the following proposals:

Observation 1: Retransmission of an RLC SDU (segment) can be triggered by both associated NACK and the remaining time based retransmission indication, and resultant redundant retransmission will lead to capacity loss.

Observation 2: As per running CR, the remaining time based polling triggering is dependent on whether there is an available AMD PDU to transmit.

Proposal 1:  Only RLC SDU or segment for which no feedback was received is considered for remaining time based retransmission by a Tx RLC entity, as given in Appendix A.

Proposal 2:  RAN2 clarifies that the Tx RLC entity should cancel the pending status for the RLC SDU (segment) that has been pending for retransmission when its discard indication is received, as given in Appendix B.

Proposal 3: (RLC-11) For specification simplicity, RAN2 maintains the specification of t-PollRetransmit expiry (Clause 5.3.3.4) unchanged as legacy, e.g. Rel-18.

Proposal 4: RAN2 considers the case when remaining time based polling is triggered but there is no available RLC SDU to transmit.

Proposal 5: If there is no available RLC SDU to carry the polling when the polling is triggered based on the remaining time, a specific RLC SDU, e.g. RLC SDU with the highest SN among the RLC SDUs submitted or any RLC SDU which has not been positively acknowledged, can be considered for retransmission, as given in Appendix C.

Proposal 6: UE supporting Tx based feature shall also be able to support the Rx based feature.

# 4. Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #103

[2] Chair note of RAN2#130, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_130/Inbox/Chair_Notes/R2_130%20R19%20XR%20and%20LTE%20Broadcast%20notes%20(Dawid)%202025-05-23_EOM.docx

[3] TS 38.322

- Appendix A

## 5.3	ARQ procedures

### 5.3.2	Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

        - STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: and

- if stopReTxDiscardedSDU is configured and no discard indication for the RLC SDU has been received from upper layers, or if stopReTxDiscardedSDU is not configured:

  - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

When receiving an indication from upper layer (e.g. PDCP) that the condition for remaining-time-based RLC retransmission has been met for an RLC SDU, the transmitting side of the AM RLC entity shall:

- if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers; and

- if the RLC SDU or the RLC SDU segment(s) thereof has not been positively acknowledged:

  - consider the RLC SDU or the RLC SDU segment(s) for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

- if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

  - set the RETX_COUNT associated with the RLC SDU to zero.

- else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

  - increment the RETX_COUNT.

- if RETX_COUNT = maxRetxThreshold:

  - indicate to upper layers that max retransmission has been reached.

- Appendix B

5.2.3	AM data transfer

5.2.3.1	Transmit operations

5.2.3.1.1	General

…

The transmitting side of an AM RLC entity can receive a positive acknowledgement (confirmation of successful reception by its peer AM RLC entity) for an RLC SDU by the following:

- STATUS PDU from its peer AM RLC entity.

When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall:

- send an indication to the upper layers of successful delivery of the RLC SDU;

- set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet.

If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission or retransmission, or shall cancel the pending status if the corresponding RLC SDU or RLC segment(s) is pending for retransmission.

…

## 5.3	ARQ procedures

### 5.3.2	Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

        - STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: and

- if stopReTxDiscardedSDU is configured and no discard indication for the RLC SDU has been received from upper layers, or if stopReTxDiscardedSDU is not configured:

  - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

When receiving an indication from upper layer (e.g. PDCP) that the condition for remaining-time-based RLC retransmission has been met for an RLC SDU, the transmitting side of the AM RLC entity shall:

- if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers; and

- if the RLC SDU or the RLC SDU segment(s) thereof has not been acknowledged:

  - consider the RLC SDU or the RLC SDU segment(s) for retransmission.

if stopReTxDiscardedSDU is configured and the corresponding RLC SDU has been indicated as discarded by upper layer, the transmitting side of the AM RLC entity shall:

- if the RLC SDU or the RLC SDU segment(s) thereof has been pending for retransmission:

  - consider the RLC SDU or the RLC SDU segment(s) as not pending for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

- if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

  - set the RETX_COUNT associated with the RLC SDU to zero.

- else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

  - increment the RETX_COUNT.

- if RETX_COUNT = maxRetxThreshold:

  - indicate to upper layers that max retransmission has been reached.

- Appendix C

## 5.3	ARQ procedures

### 5.3.3	Polling

#### 5.3.3.1	General

An AM RLC entity can poll its peer AM RLC entity in order to trigger STATUS reporting at the peer AM RLC entity.

#### 5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

- increment PDU_WITHOUT_POLL by one;

- increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

- if PDU_WITHOUT_POLL >= pollPDU; or

- if BYTE_WITHOUT_POLL >= pollByte:

  - include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

- if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1) after the transmission of the AMD PDU; or

- if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

  - include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

if an indication is received from upper layer (e.g., PDCP) that the condition for remaining-time-based RLC polling has been met for an RLC SDU:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer

  - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

  - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling)

    - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

    - consider any RLC SDU which has not been positively acknowledged for retransmission

  - include a poll in an AMD PDU as described below.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

- set the P field of the AMD PDU to "1";

- set PDU_WITHOUT_POLL to 0;

- set BYTE_WITHOUT_POLL to 0.

Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall:

- set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer;

- if t-PollRetransmit is not running:

  - start t-PollRetransmit.

- else:

  - restart t-PollRetransmit.