3GPP TSG-RAN WG2#131	R2-2505508

Bengaluru, India, Aug 25th – 29th 2025

Source: 	Fujitsu

Agenda item:	8.5.4

Title:	Remaining issues on adaptation of common signal/channel

Document for:	Discussion and decision

# Introduction

Rel-19 NES WID was updated and approved in RAN#105 [1]. The WID describes the following objective on the adaptation of common signal/channel transmissions.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we would like to provide our views about remaining open issues for finalization of the adaptation of common signal/channel transmissions.

# Discussion

## Adaptation of PRACH in time domain

Supporting RA type

In RAN1#121 meeting, RAN1 did not reach a consensus to support 2-step RA for PRACH adaptation as follows:

Conclusion

There is no consensus to support adaptation of RACH in time domain for 2-step RA in Rel-19.

On the other hand, in RAN2#130 meeting, RAN2 discussed about supporting RA type for PRACH adaptation and concluded that only 4-step RA is supported unless RAN1 decides supporting 2-step RACH this meeting. It apparently means that 2-step RA is not pursed in Rel-19.

Proposal 1:	RAN2 to confirm only 4-step RA is supported in PRACH adaptation in Rel-19.

(PRACH adaptation issue) CE feature handling

The following agreements about PRACH adaptation were made in RAN2 #130 meeting [2].

Agreements on common channel adaptation 15. RAN2 will select one of the following options. (CE here means MSG1 repetition). - Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon. - Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17)

Because RAN2 agreed that the additional RACH configuration (addlRACH-Config-Adaptation-r19) is placed within RACH-ConfigCommon, then the preamble partitions (featureCombinationPreamblesList-r17) may be applied to both legacy RACH and additional RACH. However, the additional RACH does not support CE feature then it should be addressed how to avoid the combination between CE and the additional RACH.

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

<<skip unchanged part>>

[[

ra-PrioritizationForSlicing-r17         RA-PrioritizationForSlicing-r17                          OPTIONAL,   -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17     SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL -- Cond AdditionalRACH

]],

[[

addlRACH-Config-Adaptation-r19         RandomAccessAdaptationConfig-r19                                OPTIONAL     -- Need R

]]

}

There are two options to address this issue, and both options would stick to the previous agreement. For Option-1b, a new UE behavior should be specified to avoid using additional RACH resource set if CE is applied. Meanwhile, for Option-1c, the NW should avoid configuring the additional RACH configuration with CE feature in the RACH-ConfigCommon. In our understanding, such a feature confliction should be handled by NW implementation. Moreover, when the preamble repetition is required for random access procedure, it implies the UL coverage is not good and the cell needs more RA occasions to support repetitions. Consequently, CE feature is not suitable with additional RACH. In this sense, we slightly prefer to adopt Option-1c.

Proposal 2:	For CE feature handling with PRACH adaptation, RAN2 to adopt Option-1c, i.e., rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon.

## SSB adaptation

(SSB adap issue) Parameters related to SSB adaptation

RAN1 has sent the LS about SSB adaptation related parameters [3]. According to the LS, RAN1 indicated seven SSB adaptation parameters shown in Annex A. SSB periodicity can be switched by DCI format 2_9 which is already introduced in Rel-18 NES (for Cell DTX/DRX). Therefore, DCI related configuration (Search space for DCI format 2_9 and DCI size) can be reused, and no additional information is required to enable SSB adaptation.

Observation 1:	the following information is already supported and implemented in Rel-18.

- SearchSpace (for DCI format 2_9)

- sizeDCI-2-9-r18

Based on the RAN1 LS, a new RNTI will be introduced to switch the SSB periodicity per cell group. From RAN2 perspective, it means that the new RNTI is introduced and placed into the CellGroupConfig. On the other hand, the other information will be configured per SCell. Given the RAN1#120bis agreement below, it can be configured at most two additional SSB periodicity per SCell.

Agreement (RAN1#120bis)

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

•	Xmax=2

Observation 2:	the NW can configure two additional SSB burst periodicity per SCell at the maximum.

Hence it should be defined as a list (i.e., ToAddModList and ToReleaseList) under additional SSB configuration. In addition, for simplicity, we propose to adopt the same implementation as in the OD-SSB configuration. In other words, it can be placed into the SCellConfig in the CellGroupConfig.

Proposal 3:	(SSB adap issue) RAN2 agree to place the following information into the CellGroupConfig.

- ssbPeriodicityIndication-RNTI

Proposal 4:	(SSB adap issue) RAN2 agree to place the following information as a list (i.e., ToAddModList and ToReleaseList) into the SCellConfig in the CellGroupConfig.

- addl-ssb-Periodicity

- addl-ssb-Offset

- addl-ssb-halfFrameIndex

- positionInDCI-ssbPeriodicityIndicationForScell

Other than the RAN1 list, in RAN2#129bis meeting, RAN2 agreed new SMTC configuration(s) per SSB periodicity, i.e., “For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration”. Thus, additional SMTC configuration would be introduced per SSB periodicity and listed into the SCellConfig as well.

Proposal 5: (SSB adap issue) RAN2 agree to introduce additional SMTC configuration per addl-SSB-periodicity.

Given the observation and proposals, TP for SSB adaptation is shown in Annex B.

# Conclusion

In this contribution, we have some discussions about the adaptation of common signals or channels for finalization, and the following proposals are made:

Proposal 1:	RAN2 to confirm only 4-step RA is supported in PRACH adaptation in Rel-19.

Proposal 2:	For CE feature handling with PRACH adaptation, RAN2 to adopt Option-1c, i.e., rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon.

Proposal 3:	(SSB adap issue) RAN2 agree to place the following information into the CellGroupConfig.

- ssbPeriodicityIndication-RNTI

Proposal 4:	(SSB adap issue) RAN2 agree to place the following information as a list (i.e., ToAddModList and ToReleaseList) into the SCellConfig in the CellGroupConfig.

- addl-ssb-Periodicity

- addl-ssb-Offset

- addl-ssb-halfFrameIndex

- positionInDCI-ssbPeriodicityIndicationForScell

Proposal 5: (SSB adap issue) RAN2 agree to introduce additional SMTC configuration per addl-SSB-periodicity.

# References

- RP-242354, Revised WID: Enhancements of network energy savings for NR

- R2-2504671, Report from session on R18 SL, R18/19 MOB, and R19 NES

- R2-2505000, LS on Rel-19 higher layers parameters list Post RAN1#121

# Annex A: parameter list for SSB adaptation in RAN1

Table A: RAN1 defined SSB adaptation related parameters [3]

| Parameter name in the spec | New or existing? | Description | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- |
| addl-ssb-Periodicity | New | Additional SSB burst periodicity for the Scell. | per secondary serving cell |
| addl-ssb-Offset | New | Indicate SFN offset from the SFN which satisfies (SFN index *10) modulo (addl-SSB periodicity) = 0 | per addl-SSB-periodicity |
| addl-ssb-halfFrameIndex | New | Indicate whether SSB according to the addl-SSB-periodicity is in the first half or the second half of the frame. | per addl-SSB-periodicity |
| ssbPeriodicityIndication-RNTI | New | The RNTI value for scrambling CRC of DCI format 2_9 for SSB burst periodicity switching for Scell(s). | per cell group |
| SearchSpace | Existing | The search space for monitoring DCI format 2_9 for SSB burst periodicity switching. | Per serving cell on which DCI 2_9 is monitored (SearchSpace) |
| sizeDCI-2-9-r18 | Existing | The size of DCI format 2_9 for SSB burst periodicity switching(see TS 38.212, subclause 7.3.1.3.10). | Per serving cell on which DCI 2_9 is monitored |
| positionInDCI-ssbPeriodicityIndicationForScell | New | The starting bit position of an information block of DCI format 2_9 for SSB burst periodicity switching of this serving cell (see TS 38.212, subclause 7.3.1.3.10). | per secondary serving cell |

# Annex B: TP for SSB adaptation

### 6.3.2	Radio resource control information elements

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

<<skip unchanged part>>

[[

ncr-FwdConfig-r18                          SetupRelease { NCR-FwdConfig-r18 }                                 OPTIONAL,  -- Cond NCR

autonomousDenialParameters-r18             SetupRelease {AutonomousDenialParameters-r18}                      OPTIONAL,   -- Need M

nonCollocatedTypeMRDC-r18                  ENUMERATED { true }                                                OPTIONAL,   -- Need R

nonCollocatedTypeNR-CA-r18                 ENUMERATED { true }                                                OPTIONAL,   -- Need R

uplinkTxSwitchingMoreBands-r18             SetupRelease { UplinkTxSwitchingMoreBands-r18 }                    OPTIONAL    -- Need M

]],

[[

ssbPeriodicityIndication-RNTI-r19          RNTI-Value                                                      OPTIONAL,   -- Need R

]]

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

<<skip unchanged part>>

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]],

[[

addl-SSB-ConfigToAddModList-r19                   SEQUENCE (SIZE (1..2)) OF addl-SSB-Config-r19    OPTIONAL,   -- Need N

addl-SSB-ConfigToReleaseList-r19                  SEQUENCE (SIZE (1..2)) OF addl-SSB-ConfigId-r19    OPTIONAL    -- Need N

]]

}

- Addl-SSB-Config

The Addl-SSB-Config IE is used to configure an additional SSB configuration.

Addl-SSB-Config information element

-- ASN1START

-- TAG-ADDL-SSB-CONFIG-START

addl-SSB-Config-r19 ::=         SEQUENCE {

addl-SSB-ConfigId-r19    		INTEGER (0..1),

addl-ssb-Periodicity-r19		ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }		OPTIONAL, -- Need R

addl-ssb-Offset-r19				INTEGER (0..15)				OPTIONAL, -- Need R

addl-ssb-halfFrameIndex-r19		ENUMERATED {zero, one}		OPTIONAL, -- Need R

positionInDCI-ssbPeriodicityIndicationForScell-r19			INTEGER (0..maxDCI-2-9-Size-1-r18)			OPTIONAL, -- Need R

SMTC-r19 						SSB-MTC                     OPTIONAL, -- Need R

...

}

-- TAG-ADDL-SSB-CONFIG-STOP

-- ASN1STOP