3GPP TSG-RAN WG2 Meeting #131        	                       R2-2505511

Bengaluru, India, 25th-29th August, 2025

Agenda item:	8.1.3

Source:	CMCC

Title:	Discussion on remaining issues on NW side data collection

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed the remaining issues on NW side data collection and achieved the following agreements.

Agreements

- (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced.  In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data).   We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it.   Notify RAN3

- Buffer threshold to trigger data availability indication should be set based on specific size, e.g., KB instead of percentage

- UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

- No additional signaling from the UE is required when the low power issue is resolved

- No additional signaling from the UE is required when the buffer full issue is resolved

Agreements

1	Data is collected on per data logging configuration basis and UE indicates data logging configuration ID.    An indication of the “gap” is needed.  “Gap” is time interval larger than the configured logging periodicity.    FFS if timestamp and relative time stamp for each group is needed per “group”.

Agreement

1	The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

This contribution will discuss the remaining open issues on for NW side data collection [2].

- Discussion

## Open issue RRC-21: Time related content of logged data

In last meeting, RAN2 agreed that an indication of the “gap” is needed, and “gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

In legacy logged MDT reporting, the absolute time info absoluteTimeStamp (48bits, as specified in [3]) when the logged measurement is provided and the relative time info relativeTimeStamp (as specified in [3]) which indicates the time of logging measurement results would be included in UEInformationResponse message. And the time info is used for the network to analyze when the issue occurred and optimize the configuration.

Considering the gap may be caused by event-triggered logging, handover, full buffer and low power cases, the network cannot be aware of when the event is satisfied since it depends on UE’s measurements. On the other hand, the gap duration may vary among different groups which cannot be known by the network. Therefore, the indication should be needed for each group.

Observation 1: The network cannot be aware of when the event is satisfied since it depends on UE’s measurements, and the gap duration may vary among different groups which cannot be known by the network.

Furthermore, the following options can be considered to indicate the gap per group:

- Option 1: The absolute time stamp indicating when the logged measurement is provided (similar to legacy MDT), and the relative time stamp indicating the first data sample per group

- Option 2: The absolute time stamp indicating the first data sample per group

Figure 1. An example for data logging

Taking the Fig.1 as an example, if option 1 is adopted, the UE will indicate the absolute time stamp t0 and the relative time stamp for the first data sample for each group, i.e. t1 for Group1, t2 for Group2, t3 for Group3. If we follow the legacy mechanism, the size of absolute time stamp is 48bits, and the size of relative time stamp is 13 bits, the UE needs to report 87 bits (48bits + 13bits*3) to indicate the time info.

AbsoluteTimeInfo-r16 ::= BIT STRING (SIZE (48))

relativeTimeStamp-r16 INTEGER (0..7200)

If option 2 is adopted, the UE will indicate the absolute time stamp for the first data sample per group, i.e. the absolute time stamp t1 for Group1, t2 for Group2, t3 for Group3, and the UE is assumed to report 144 bits (48bits*3).

Based on the above analysis, option 1 is better than option 2 from the perspective of signaling overhead, and option 1 is also aligned with legacy logged MDT mechanism where the absolute time stamp is provided by the network. Therefore, option 1 is preferred to indicate the gap per group.

Proposal 1: The gap indication is needed for each group, i.e. the absolute time stamp indicating when the logged measurement is provided, and the relative time stamp indicating the first data sample per group.

## Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

RAN2#127 made the following agreement regarding NW-side data collection:

1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

- RAN2#129bis made the following agreement:

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

- In current immediate MDT mechanism, the UE follows the RRM measurement behavior, i.e., the UE starts to collect measurement data upon reception of the measurement configuration and the configured event is satisfied (if any). According to reply LS on data collection requirement from RAN1 [4], the latency requirement of training data collection is relaxed (e.g., minutes, hours, days, or no latency requirement). We think the motivation and gain of introducing dynamic activation/deactivation is unclear, and would introduce the additional complexity. Thus, we prefer that the UE starts the measurement upon reception of the measurement configuration and the configured event is satisfied (if any), and the dynamic activation/deactivation is not supported.

- Proposal 2: The dynamic activation/deactivation of data collection configuration for logging is not supported.

## Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

RAN2#128 made the following agreement:

- Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2).

- When the UE reports low power state indication, the network is expected to release the data collection configuration. To avoid wasting the collected data, the network may want to retrieve the available logged data before releasing the data collection configuration, even if the buffer threshold is not reached. Therefore, UE reports the data availability indication when the UE has data below the threshold and low power state is sent. The additional cause is not needed.

- Proposal 3: The UE reports data availability indication when the UE has data below the threshold and low power state is sent. The additional cause is not needed.

## Open issue RRC-31: The release of logged AI/ML data in UE’s buffer

- As per Rapp’s suggestion, RAN2 needs to discuss and capture in the spec when the logged data for AI/ML in UE buffer will be released. In addition to the retain/release of logged data during HO, the following can be also considered:

- - Power off or de-registration

- - 48 hrs after the release of logged measurement configuration

- - Explicit indication from the serving gNB

- In legacy logged MDT [5], the UE should store non-retrieved measurements for 48 hours from the moment the duration timer for logging expired. And all logged measurement configuration and the log shall be removed by the UE at switch off or detach.

- 5.1.1.3 Measurement reporting - 5.1.1.3.1 Availability Indicator <omitted> In case the network does not retrieve Logged MDT measurements, UE should store non-retrieved measurements for 48 hours from the moment the duration timer for logging expired. There is no requirement to store non-retrieved data beyond 48 hours. In addition, all logged measurement configuration and the log shall be removed by the UE at switch off or detach.

- We think the above bullets are aligned with the legacy principle, and can be supported.

- Proposal 4: In case the network does not retrieve logged AI/ML data, UE should store non-retrieved data for 48 hours after the release of data collection configuration. The UE will release all logged AI/ML data at switch off or de-registration, or receiving the explicit indication from the serving gNB.

# 3	Conclusion

Here are the proposals for NW side data collection.

Open issue RRC-21: Time related content of logged data

Observation 1: The network cannot be aware of when the event is satisfied since it depends on UE’s measurements, and the gap duration may vary among different groups which cannot be known by the network.

Proposal 1: The gap indication is needed for each group, i.e. the absolute time stamp indicating when the logged measurement is provided, and the relative time stamp indicating the first data sample per group.

Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Proposal 2: The dynamic activation/deactivation of data collection configuration for logging is not supported.

Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

- Proposal 3: The UE reports data availability indication when the UE has data below the threshold and low power state is sent. The additional cause is not needed.

Open issue RRC-31: The release of logged AI/ML data in UE’s buffer

Proposal 4: In case the network does not retrieve logged AI/ML data, UE should store non-retrieved data for 48 hours after the release of data collection configuration. The UE will release all logged AI/ML data at switch off or de-registration, or receiving the explicit indication from the serving gNB.

# 4	Reference

- Draft_RAN2_130_Meeting_Report_v3

- R2-2505778 RRC open issues for AIML for NR air interface

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- R1-2310681 Reply LS on Data Collection Requirements and Assumptions  Source: RAN1  To: RAN2

- TS 37.320 Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2