3GPP TSG RAN WG2 #131			R2-2505528

Bengaluru, India, Aug. 25th – 29th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on adaptation of common signal and channel

Agenda Item:	8.5.4

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. The following agreements have been made in RAN2 so far [2]-[5]. In this contribution, we provide our views on adaptation of common signal and channel.

Agreements on adaptation@RAN2#130

1.	Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19 unless RAN1 decides supporting 2-step RACH this meeting.

2.	addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

3.	Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1. addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

4.	Agree that the following information is included in additional RO configuration (see the agreed table in the discussion of R2-2504579)

5.	If adaptation for additional PRACH resources is supported in active UL BWP, network can configure that paging search space in active DL BWP if network want to activate additional RO to the corresponding UEs.

6.	RRC connected UE monitors the DCI-based adaptation indication at least once every once every SI modification period.

7.	RAN2 will not do anything to handle the UEs that missed the short message.

8.	No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. RAN2 understand there is no MAC impact.

9.	RAN2 de-prioritizes the discussion of NES (including all NES scopes) and SBFD co-existence.

10.	BI is commonly applied to both legacy RO and additional RO.

11.	For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO. No RAN2 spec impact is foreseen.

12.	RACH adaptation is not applied to the following cases (for non-serving cell case): 1) PDCCH order for additional RACH, 2) L3 HO command, 3) LTM CSC MAC-CE, 4) PDCCH order for LTM early UL sync.

13.	RACH adaptation is not applied to BFR.

14.	RAN2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

15.	RAN2 will select one of the following options. (CE here means MSG1 repetition).

Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17).

Agreements on adaptation@RAN2#129bis

1.	For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

2.	For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

3.	Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

4.	Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

5.	For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

6.	Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Agreements on adaptation@RAN2#129

On Paging adaptation

1.	For N, values smaller than T/32 are not supported.

2.	The maximum possible value for Ns is 8.

3.	Introduce a separate PEI configuration.

4.	Paging adaptations are configured semi-statically and updated via system information update notification.

5.	A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

on SSB adaptation

1.	RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

2.	RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

3.	For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

- 5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

- 5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

Agreements on adaptation@RAN2#127bis

Agreements on paging adaptation

1.	Select option-b as baseline for R19 NES paging enhancement.

2.	R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

3.	Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

4.	Legacy UE is not barred.

5.	Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

- Discussion

2.1	Paging adaptation

2.2.1	Issue 1 of RRC adaptation

In the TS38.331 CR rapporteur’s summary, the following remaining issues were identified [6].

Q13a. Please comment on whether the maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to 32 radio frames: - at symbol level - at slot level - by restricting the maximum value range of pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 field parameter for different SCS corresponding to pagingAdaptationNAndPagingFrameOffset-r19 (i.e. N) - by restricting the configuration to evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them. - Other Q13b: Please comment on which values should be adopted, i.e., x

On the maximum offset value, we think no optimization is not needed and just following the legacy principle is enough. RAN2 decided that only N=T/32 and Ns=8 should be introduced, concerning the signalling overhead due to the parameters that point out PDCCH MO of PO, which is estimated based on the legacy manner. We understand that, in the legacy manner, the maximum offset value (e.g., 139 for SCS=15kHz and N=T) is determined by the total number of symbols that one PF covers. Here, the maximum offset value is doubled according to increase of SCS and decrease of PF, considering a PO can be placed at subsequent radio frame after the radio frame assigned as PF associated with the PO.

In this sense, for the parameter of Rel-19, the maximum offset value for sCS15KHZoneT remains 139 (and other combinations are also the same value as the corresponding legacy value), while for sCS480KHZoneSixteenthT should be extended to 35,839, i.e., option.iii should be taken.

PCCH-Config

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

[[

pagingAdaptation-NS-r19                                  ENUMERATED {eight,four, two, one}                 OPTIONAL,  -- Need R

pagingAdaptationNAndPagingFrameOffset-r19               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15),

oneThirtySecondT                     INTEGER (0..31)

}                                                                                                          OPTIONAL,  -- Need R

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZoneThirtySecondT                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-SCS60KHZoneThirtySecondT                                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneEighthT-sCS120KHZoneThirtySecondT                                                   SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneSixteenthT                                                      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

}                                                                                                              OPTIONAL           -- Need R

]]

PDCCH-ConfigCommon

pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneThirtySecondT                                                SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneThirtySecondT                                                SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

Proposal 1: (Issue 1 of RRC adaptation)

- Support option.iii for the maximum offset value for pagingAdaptatoinFirst PDCCH-MonitoringOccasion-OfPO-r19, e.g.,

  - sCS15KHZoneT: INTEGER (0..139)

  - sCS15KHZoneThirtySecondT: INTEGER (0.. 4479)

  - sCS480KHZoneSixteenthT: INTEGER (0.. 35839)

2.2.2 Issue 2 of RRC adaptation

Q14a. Please comment on whether the maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter is extended to 32 radio frames: - at symbol level - at slot level - by restricting the maximum value range of Rel-19 pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for different SCS corresponding to pagingAdaptationNAndPagingFrameOffset-r19 (i.e. N) - by restricting the configuration to evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them. - Other Q14b: Please comment on which values should be adopted, i.e., x

Similar to the above discussion, the legacy principle should be applied to the parameters to define PDCCH monitoring occasion for PEI in Rel-19 paging adaptation as well. Therefore, the same values should be introduced as the one for PO.

Proposal 2: (Issue 2 of RRC adaptation)

- Support option.iii for the maximum offset value for pagingAdaptatoinFirstPDCCH-MonitoringOccasion-OfPEI-r19, e.g.,

  - sCS15KHZoneT: INTEGER (0..139)

  - sCS15KHZoneThirtySecondT: INTEGER (0.. 4479)

  - sCS480KHZoneSixteenthT: INTEGER (0.. 35839)

PDCCH-ConfigCommon

pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19  CHOICE {

sCS15KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneThirtySecondT                                                SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneThirtySecondT                                                SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS)

}                                                                                           OPTIONAL     -- Cond InitialBWP-Paging

]]

2.2.3	Issue 3 of RRC adaptation

Q15a: Please comment on whether the following parameters should be introduced for Rel-19 PEI configuration: - po-NumPerPEI-r19 - payloadSizeDCI-2-7-r19 - pei-FrameOffset-r19 - subgroupsNumPerPO-r19 - subgroupsNumForUEID-r19 Q15b: Please comment on whether the same value ranges as for legacy PEI should be used for the following parameters (if agreed to be introduced per Q15a above) for Rel-19 PEI configuration: - po-NumPerPEI-r19 - payloadSizeDCI-2-7-r19 - pei-FrameOffset-r19 - subgroupsNumPerPO-r19 - subgroupsNumForUEID-r19

On how/whether to define PEI-related parameters in Rel-19 paging adaptation, we think basically all parameters should be introduced to be configured independently from the ones for Rel-17. This is because, it should be up to NW configuration/implementation whether/how one common DCI2-7 deliver both or either PEI information to Rel-17 UEs and Rel-19 UEs. Regarding the value ranges for these parameters, we think the one for identifying radio frame where PEI is transmitted should be extended according to the extension of offset for Rel-19 PF, i.e., pei-FrameOffset-r19 can be up to 32. Considering the legacy PEI design that one PEI can indicate the presence of POs over 2 PF, since the maximum number of POs with a PF is extended for Rel-19, the number of POs which one PEI can indicate can also be considered. From our perspective, the increase of POs within a PEI might cause re-consideration of design of DCI and subgroups that should be involved with other WGs, so we prefer to keep the legacy values.

Proposal 3: (Issue 3 of RRC adaptation)

- The following parameters should be introduced for Rel-19 PEI configurations with the value range.

  - po-NumPerPEI-r19: same as the legacy

  - payloadSizeDCI-2-7-r19: same as the legacy

  - pei-FrameOffset-r19: INTEGER (0..32)

  - subgroupsNumPerPO-r19: same as the legacy

  - subgroupsNumForUEID-r19: same as the legacy

- PEI-Config-r17 ::=                        SEQUENCE {

- po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

- payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

- pei-FrameOffset-r17                       INTEGER (0..16),

- subgroupConfig-r17                        SubgroupConfig-r17,

- lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

- ...

- }

- SubgroupConfig-r17 ::=     SEQUENCE {

- subgroupsNumPerPO-r17      INTEGER (1.. maxNrofPagingSubgroups-r17),

- subgroupsNumForUEID-r17    INTEGER (1.. maxNrofPagingSubgroups-r17)                                        OPTIONAL,  -- Need S

- ...

- }

- Conclusion

In this contribution, we provided the following observations and proposals of adaptation of common signal and channel for network energy saving.

Proposal 1: (Issue 1 of RRC adaptation)

- Support option.iii for the maximum offset value for pagingAdaptatoinFirst PDCCH-MonitoringOccasion-OfPO-r19, e.g.,

  - sCS15KHZoneT: INTEGER (0..139)

  - sCS15KHZoneThirtySecondT: INTEGER (0.. 4479)

  - sCS480KHZoneSixteenthT: INTEGER (0.. 35839)

Proposal 2: (Issue 2 of RRC adaptation)

- Support option.iii for the maximum offset value for pagingAdaptatoinFirstPDCCH-MonitoringOccasion-OfPEI-r19, e.g.,

  - sCS15KHZoneT: INTEGER (0..139)

  - sCS15KHZoneThirtySecondT: INTEGER (0.. 4479)

  - sCS480KHZoneSixteenthT: INTEGER (0.. 35839)

Proposal 3: (Issue 3 of RRC adaptation)

- The following parameters should be introduced for Rel-19 PEI configurations with the value range.

  - po-NumPerPEI-r19: same as the legacy

  - payloadSizeDCI-2-7-r19: same as the legacy

  - pei-FrameOffset-r19: INTEGER (0..32)

  - subgroupsNumPerPO-r19: same as the legacy

  - subgroupsNumForUEID-r19: same as the legacy

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#129 meeting, Chairman’s notes

- 3GPP, RAN2#129bis meeting, Chairman’s notes

- 3GPP, RAN2#130 meeting, Chairman’s notes

- Report of [POST130][107][NES] (Ericsson), Ericsson