3GPP TSG-RAN WG2 Meeting #131                                    	R2-2505532

Bengaluru, India, 25th – 29th August 2025

Agenda item:	8.8.2

Source:	Qualcomm Incorporated

Title:	Discussion on beam hopping with multiple SMTC offsets

Document for:	Discussion and Decision

# Introduction

In RAN2#130 meeting, following agreements were made.

Agreements:

1.	the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

2.	To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

4.	No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

Working Assumption:

1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

In this document, we further discuss the RAN2 aspects of multiple SMTCs offsets beyond 4 and multiple SMTC periodicities.

# Discussion

Multiple SMTCs

In the last meeting, an LS [1] is sent to RAN4 on checking feasibility of 2 SMTC periodicities in the same frequency layer as follows:

Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases:

- RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements.

- RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

We think RAN2 can simply wait RAN4 to understand the complexity of multiple periodicities on the same frequency layer. However, more than 4 SMTCs can be considered, and we should define a mechanism for UEs not to select more than 4 SMTCs.

Specially there needs to be a way for UEs to select correct SMTCs/beams to measure (i.e., right SMTCs for the closest beams). There are some solutions discussed such as:

- SSB index based SMTC selection

- Location-based SMTC selection

In our view, how multi-beam operation (beam-based mobility) works properly in NTN has never been discussed due to large satellite beam footprint (e.g., > 50km). In the context of RACH-less HO, RAN2 sent LS to RAN1 asking question on multiple beams and RAN2 received response from RAN1 in [2] with following response. It could be understood that existing beam-level mobility could be sufficient and for measurements. Based on the location information, UE should be able to select four SMTCs for its serving beam’s neighbors.

2. To monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam selection is needed (e.g., performed by NW with selected beam(s) indicated, or performed by UE)?

RAN1 response: If single beam is indicated, UE will monitor the target cell PDCCH scheduling the first PUSCH based on the indicated beam. RAN1 will further discuss the case where multiple beams are indicated.

Figure 1 An Example of beam hopping with extended multiple SMTC offsets (i.e., 6 offsets)

In location-based SMTC selection, each SMTC can be associated with a reference location of the intended neighbor beams that need to be measured by the UE. Then UE will select the supported set of SMTCs that are closest to the UE’s location. We think UE should calculate the distance from its position to the reference location, and choose the 4 SMTCs with the shortest distances. The UE will perform measurements using existing procedure and RRM requirements based on 4 SMTCs.

The location-based SMTC can also be applicable to moving cell scenario. In Rel-18, we introduced the concept of moving reference location which can be time variant, and UE can predict the future reference location. Since the reference location in this case is for one time use to perform down selection of the 4 SMTCs out of 6 SMTCs, the use of reference location at time t is sufficient and UE probably would not need to track the reference location. Therefore, network would be able to update the reference location when ephemeris and epoch time changes without notifying UE.

Location-based SMTC enhancement is also supported in moving cell scenario. The existing movingReferenceLocation is reused for this purpose (i.e., network can update the movingReferenceLocation with epoch time without notifying UE).

Since RAN2 already adopted the location-based SMTC, we see no motivation to introduce another solution to solve the same problem in Rel-19. This solution works for both IDLE mode measurements (reference locations provided in SIB for each SMTC) and connected mode measurements (reference locations provided in MO for each SMTC). In future, for example, for the case of GNSS-less operation in Rel-20, it is worth exploring other solution.

Location-based SMTC is sufficient to solve issue of selecting four SMTCs both in connected mode and idle mode measurements and there is no need to introduce second solution to solve the same issue in Rel-19.

SMTC enhancements in connected mode

In case of connected mode, the same location-based solution is applicable for UE to select 4 SMTCs. However, it is important that network would need to know which of the 4 SMTCs are being used by UE, if there is issue of scheduling restrictions.

For intra-frequency measurements:

For intra-frequency measurements, there may not be any scheduling restrictions as UE may be able to perform measurements using multiple SMTCs as this is already possible today. We believe there is no need to report any feedback to network as it won’t matter to network which SMTC the UE is using.

For UEs capable of performing intra-frequency measurements without scheduling restriction, UE can be configured with more than 4 SMTCs and UE does not report to network which 4 SMTCs would be used. For example, there is no issue to configure 6 SMTCs for 6 intra-frequency neighbor cells corresponding to the same satellite.

If the UE has scheduling restriction for configuring more SMTCs, we think network can use existing tool to configure the 4 SMTCs. For such UEs, upon receiving the measurement object configuration in RRCReconfiguration message, we may also extend RRCReconfigurationComplete to indicate which SMTCs the UE has selected when more than 4 SMTC configuration is received via RRCReconfiguration message.

As in Rel-17, it is expected that network provides updated SMTC configuration via RRCReconfiguration message periodically.

If there is no scheduling restriction from configuration of more than 4 SMTCs based on UE’s capability, the UE does not report to network which SMTCs are used.

If there is scheduling restriction from configuration of more than 4 SMTCs based on UE’s capability, extend RRCReconfigurationComplete message to indicate which SMTCs the UE has selected.

Introduce a UE capability signaling to indicate whether the UE supports configuration of more than 4 SMTCs for intra-frequency measurements.

For inter-frequency measurements:

In our view, the discussion of beam hopping is limited to intra-frequency measurements scenarios, i.e., for energy saving, the satellite hops the beams but they all can operate in the same frequency. Multi-frequency beam hopping is an added complexity due to involvement of measurement gaps as no more than 2 concurrent MGs can be configured per UE given the UE supports parallelMeasurementGap-r17 and network may want efficient configuration choice.

With such consideration, RAN4 impact should also be investigated as in the LS [1], RAN2 only asked question for the intra-frequency neighbor cells.

Therefore, RAN2 first need to decide whether more than 4 SMTCs configuration is applicable for inter-frequency measurements. If yes, then a new UE capability to support more than 4 SMTCs configuration is applicable for inter-frequency measurements should be introduced and ask RAN4 for any issue/concern.

RAN2 first need to decide whether more than 4 SMTCs configuration is applicable for inter-frequency measurements.

If P6 is agreed, a new UE capability to support more than 4 SMTCs configuration for inter-frequency measurements is introduced and an LS is sent to RAN4 for any issue/concern.

Multiple periodicities

RAN4 has sent reply LS in [2], with following answer.

- For the SMTC enhancement on configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements: From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but that requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Note: RAN4 has not estimated the full impact during RAN4#115 meeting. There is a concern on the workload on to support those agreements in Rel-19 timeframe from RAN4 requirements perspective.

It is clear obviously this would be feasible but the critical point RAN2 needs to consider is whether such enhancement is critical and worth mandating change in current UE’s RRM requirements and more workload to other working groups and. In our view, more than 6 SMTCs configuration meets the purpose of beam hopping at different time instances. Therefore, it is not critical to introduce multiple periodicities to be used with more than 4 SMTCs per frequency layer.

Multiple SMTC periodicities are not introduced to be used in combination with more than 4 SMTCs per frequency layer.

# Conclusion

Following proposals are made.

Proposal 1	Location-based SMTC enhancement is also supported in moving cell scenario. The existing movingReferenceLocation is reused for this purpose (i.e., network can update the movingReferenceLocation with epoch time without notifying UE).

Proposal 2	Location-based SMTC is sufficient to solve issue of selecting four SMTCs both in connected mode and idle mode measurements and there is no need to introduce second solution to solve the same issue in Rel-19.

Proposal 3	If there is no scheduling restriction from configuration of more than 4 SMTCs based on UE’s capability, the UE does not report to network which SMTCs are used.

Proposal 4	If there is scheduling restriction from configuration of more than 4 SMTCs based on UE’s capability, extend RRCReconfigurationComplete message to indicate which SMTCs the UE has selected.

Proposal 5	Introduce a UE capability signaling to indicate whether the UE supports configuration of more than 4 SMTCs for intra-frequency measurements.

Proposal 6	RAN2 first need to decide whether more than 4 SMTCs configuration is applicable for inter-frequency measurements.

Proposal 7	If P6 is agreed, a new UE capability to support more than 4 SMTCs configuration for inter-frequency measurements is introduced and an LS is sent to RAN4 for any issue/concern.

Proposal 8	Multiple SMTC periodicities are not introduced to be used in combination with more than 4 SMTCs per frequency layer.

# References

[1] R2-2503068, “LS on SMTC enhancements” RAN2#129bis, Source RAN2, To RAN4.

[2] R2-230xxxx, “Reply LS on SMTC enhancements”, RAN2#131, Source RAN4, To RAN2.