3GPP TSG-RAN WG2 Meeting #131	R2-2505547

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.7.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Scheduling enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following DSR related open issues have been listed from the running CR discussions:

MAC: [MAC-1] Discuss whether a pending DSR should be cancelled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported. [MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE. [MAC-6] Whether any spec change is needed to capture the agreement on DSR cancellation in the DC case. Capability: (UE capability-01): Whether a UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18.

In this contribution, we discuss the above remaining issues and other minor issues from the running CRs identified after the comment collecting deadline.

## 2	Discussion on listed open issues

## 2.1	[Capability-1] Rel-19/Rel-18 DSR Capability dependency

On the open issue of Rel-19 DSR capability dependency on Rel-18 DSR:

Agreement => An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.

A UE supporting Rel-19 DSR should be able to be configured with a single reporting threshold per LCG. The reporting threshold could be configured to be equal to the triggering threshold. This means, based on network configuration, a UE supporting Rel-19 DSR should be able to build a DSR MAC CE which is exactly the same as the Rel-18 single entry DSR MAC CE, except from the LCID. Therefore, functionalities wise, we do not see why a UE supporting Rel-19 DSR would have difficulty supporting Rel-18 DSR as well.

Proposal 1 (UE Capability-01): a UE supporting multipleEntryDelayStatusReport-r19 shall also support delayStatusReport-r18.

## 2.2	[MAC-1] DSR cancellation if no delay critical data

Following the current cancellation rule, the DSR is cancelled when the MAC PDU includes all the PDCP SDUs associated with the DSR. Since the PDCP SDUs associated with the DSR now is different from Rel-18, it should be cancelled depending on the reporting data other than delay critical data. Thus, no change seems to be necessary.

Proposal 2 (MAC-1): DSR is cancelled when the MAC PDU includes all the PDCP SDUs associated with the DSR, i.e. the delay-reporting data for Rel-19 case, so no change is needed.

## 2.3	[MAC-2] BSR cancellation interaction with DSR

It was proposed to cancel BSR if the DSR already includes all the buffer status information. We agree it would be good to avoid such unnecessary double reporting. With current design of allowing independent configuration of the DSR triggering threshold and reporting threshold, the NW has the flexibility of configuring DSR to report buffer status of non-delay critical data as well which was covered by BSR in Rel-18. It would make sense to allow the NW to disable BSR for the LCH if DSR already serve the purpose or the UE should take care of cancellation of the BSR if DSR already includes the full buffer status to avoid unnecessary overhead with duplicated information.

Proposal 3 (MAC-2): To avoid double reporting with BSR and DSR, select one of the following options:

Option 1: The signalling design should allow to disable BSR and enable DSR only for an LCH/LCG;

Option 2: the UE cancels BSR if DSR already includes full buffer status for all the LCGs.

## 2.4	[MAC-6] DSR cancellation for DC

We understood the DC issue has been discussed in the previous meeting with the following agreements made:

DSR cancellation in DC configuration (MAC-03) - (MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR. - (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

Current wording in spec says when “a MAC PDU is transmitted” and “this MAC PDU includes all PDCP SDUs associated with the DSR”, for which there is no limitation of “a MAC PDU” to the same MAC entity, thus it seems no need to reopen the discussion.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Proposal 4 (MAC-6): no further enhancement for DSR is needed for DC case.

## 3	Discussion on other issues

## 3.1	DSR MAC CE format

Even if an LCG is configured for delay status reporting with multiple reporting thresholds, this is not sufficient to determine that the DSR MAC CE will always include multiple entries for the corresponding LCG. It could happen, for example, that the UE has only data to report in correspondence of one of the multiple reporting thresholds, in which case the multiple-entry DSR MAC CE may only include single-entry for the corresponding LCG.

Proposal 5: clarify that the multiple entry DSR MAC CE reports multiple entries for a LCG only if the LCG has been configured with multiple reporting thresholds and there is data to be reported associated with multiple of those reporting thresholds.

The following text proposal should be added to Delay Status Report MAC CE description in the MAC running CR:

In the Multiple Entry DSR MAC CE, as illustrated in Figure 6.1.3.72-2, the delay status associated with a reporting threshold, which includes the BT, the EXT, the Remaining Time and the Buffer Size fields, shall be reported in two consecutive octets. If an LCG is configured with more than one reporting threshold, and the delay status information associated with more than one reporting threshold is to be reported in the Multiple Entry DSR MAC CE, the delay status associated with different reporting thresholds in the LCG should be reported consecutively in ascending order based on the values of the reporting thresholds. The delay status associated with a reporting threshold may not be reported if the total amount of UL data associated with it is zero at the start of the MAC PDU assembly, according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively. The delay status for different LCGs should be included in the Multiple Entry DSR MAC CE in ascending order based on the field LCGi. Figure 6.1.3.72-1: Single Entry DSR MAC CE

## 3.2	SR triggering for DSR when not enough space in UL grant

In Section 5.4.9 of the current ongoing MAC running CR, the SR triggering is described as following:

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel. If there is at least one DSR pending, the MAC entity shall: 1> if UL-SCH resources are available for a new transmission: 2> if at least one LCG is configured with dsr-ReportingThresList and the UL-SCH resources can accommodate the Multiple Entry DSR MAC CE as specified in clause 6.1.3.72 plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Multiple Entry DSR MAC CE as specified in clause 6.1.3.72; 2> else if none of the LCG(s) is configured with dsr-ReportingThresList and the UL-SCH resources can accommodate the Single Entry DSR MAC CE as specified in clause 6.1.3.72 plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Single Entry DSR MAC CE as specified in clause 6.1.3.72; 2> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 3> trigger a Scheduling Request; 1> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 2> trigger a Scheduling Request. NOTE 1: The availability of UL-SCH resources for the transmission of a DSR MAC CE follows the same critieria specified in clause 5.4.5.

The highlighted text above is causing an SR to be triggered when the UL-SCH resources cannot accommodate a DSR and there is no pending SR already triggered by the DSR procedure for the same LCH. However, in our view, this is not aligned with the agreement made in RAN2#129, where the following proposal form R2-2500184 was discussed:

Proposal 4: RAN2 to discuss and perform the down selection among the below three options for limited PUSCH resource case: - Option 1, there is no spec impact, UE will transmit Rel-19 DSR in the following UL grant; - Option 2, a truncated DSR will be introduced for Rel-19 DSR; - Option 3, new procedure is introduced, such as fallback to Rel-18 DSR.

The agreement was to support option 1, i.e., do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. But option 1 and agreement also explicitly mentions “no spec impact”, which seems to suggest no SR should be triggered in case of limited PUSCH grant size.  It should also be noticed that the highlighted text may impact the behaviour of a Rel-18 UE supporting Rel-18 DSR, i.e., it may create backward compatibility issues.

Observation: current text of trigger SR if the UL grant is not big enough to include DSR is not aligned with previous agreement of no spec impact for such case nor with Rel-18 behaviour.

Proposal 6: remove the highlighted text of the current running MAC CR, stating that an SR should be triggered given that the UL-SCH resources cannot accommodate a DSR and there is no pending SR already triggered by the DSR procedure for the same LCH.

# 4	Conclusion

The following proposals have been proposed in this contribution:

Proposal 1 (UE Capability-01): a UE supporting multipleEntryDelayStatusReport-r19 shall also support delayStatusReport-r18.

Proposal 2 (MAC-1): DSR is cancelled when the MAC PDU includes all the PDCP SDUs associated with the DSR, i.e. the delay-reporting data for Rel-19 case, so no change is needed.

Proposal 3 (MAC-2): To avoid double reporting with BSR and DSR, select one of the following options:

Option 1: The signalling design should allow to disable BSR and enable DSR only for an LCH/LCG;

Option 2: the UE cancels BSR if DSR already includes full buffer status for all the LCGs.

Proposal 4 (MAC-6): no further enhancement for DSR is needed for DC case.

Proposal 5: clarify that the multiple entry DSR MAC CE reports multiple entries for a LCG only if the LCG has been configured with multiple reporting thresholds and there is data to be reported associated with multiple of those reporting thresholds.

Observation: current text of trigger SR if the UL grant is not big enough to include DSR is not aligned with previous agreement of no spec impact for such case nor with Rel-18 behaviour.

Proposal 6: remove the highlighted text of the current running MAC CR, stating that an SR should be triggered given that the UL-SCH resources cannot accommodate a DSR and there is no pending SR already triggered by the DSR procedure for the same LCH.