3GPP TSG-RAN WG2 Meeting #131	R2-2505562

Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining issues of on-demand SIB1 operation for NES

Agenda Item:	8.5.3

Document for:	Discussion and decision

# Introduction

In the last meeting, RAN2 continued the discussion on on-demand SIB1 operation and several further agreements have been achieved [1]:

Agreements on OD-SIB1

1.	Not to include access related information of NES into WUS configuration.

2.	Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

3.	TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

4.	Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

5.	If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

6.	Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

7.	Proposal (include ARFCN inside the list with the PCI to have a NES-CellId list associated to a OD-SIB1 config) is not proceeded.

8.	Rel-19 excluded cell list can be configured as an empty list. No new UE behavior is introduced (i.e. the UE supporting OD-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present).

9.	Same as the other 2 agreed cases of OD-SIB1 acquisition failure, capture the failure case of OD-SIB1 window expiry in TS 38.304. RAN2 confirm no other cases need to be considered and specified in TS 38.304.

In this contribution, we will discuss the remaining issues and the open issues listed in the email discussion for on-demand SIB1 operation.

# Discussion

## 2.1 The UE behaviour when Cell A is barred but broadcasts SIBxx

Due to the fact that the WUS config is provided by Cell A, if the Cell A bars a specific UE type (e.g., NTN or RedCap) from accessing or camping on it, that UE type would not be able to obtain the WUS config for neighbouring NES cells. In the current spec, if a UE bars the Cell, there is no need for it to acquire its OSI. Since these system information blocks are useless for a UE that is barred. But in the OD-SIB1 case, the WUS config in SIBxx provided by Cell A is useful for other NES Cells which might not be barred for this UE type. Therefore, we propose that a NES UEs could acquire Cell A’s SIB1 to know whether Cell A is an anchor cell or has the broadcast information (i.e., the SchedulingInfo) of the new SIBxx with WUS configurations. If yes, the NES UE could acquire Cell A’s SIBxx to be able to trigger OD-SIB1 in neighbouring NES Cells.

Proposal 1: Once a NES UE acquires Cell A’s SIB1 and knows that it provides SIBxx, it can acquire Cell A’s SIBxx to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

In the next section, we will discuss the UE behaviour when a parameter from UL WUS configuration is inconsistent with the same parameter in SIB1 in the NES cell.

## 2.2 Parameter inconsistency between OD-SIB1 and UL-WUS

In the RAN1#120 meeting, the parameter inconsistency issue has been discussed with no consensus and the topic was left for RAN2 to decide.

As observed in RAN1’s agreements about the UL WUS configuration, several parameters are shared with SIB1. RAN1 did not decide whether OD-SIB1 IEs can be different from the value of the corresponding IEs in the WUS configuration, and how to define the UE behaviour if the values are different.

### Copied from R1-2501380 ### FL Proposal 9-2-1 Huawei’s version: - If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. Otherwise it is up to gNB to decide that the corresponding IE is not configured in OD-SIB1. ETRI’s version: - For any parameter that exists in both the UL WUS configuration and SIB1, by default, the value in the UL WUS configuration is aligned with the value in OD-SIB1 unless an exceptional allowance is separately agreed. Rapporteur suggestion: - It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

From RAN2 perspective, the common parameters in WUS config and SIB1 should be aligned, since the WUS config is also generated by the NES Cell itself. However, there could be a case where the obtained WUS config parameters from Cell A and the OD-SIB1 parameters acquired from the NES Cell are different. This may occur since there might be a delay between updating SIB1 in the NES cell and sending an updated WUS config to neighbouring Cell A’s. In our understanding SIB1 will always be updated first, so the UE should use the parameters from OD-SIB1 if there is a mismatch. That means that the value in OD-SIB1 should override the value of the corresponding IE in the WUS configuration.

Proposal 2: If the value in an OD-SIB1 IE is different from the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration.

## 2.3 SUL based OD-SIB1 request issue

During the last meeting, whether to support OD-SIB1 request on SUL was discussed without conclusion.

5. Need to emphasize it is normal uplink? R2-2503391 On-demand SIB1 Samsung discussion Rel-19 Netw_Energy_NR_enh-Core Proposal 1a: SIB 1 request is supported for both SUL and NUL. Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration. [ZTE]: Probably no much impact on RAN2, but it was excluded in RAN1 decision. [Huawei]: Want to follow RAN1 discussion and decision. [Samsung]: Understand RAN1 actually has never discussed specific SUL aspect. [ZTE]: Understand RAN1 clearly excluded SUL. [IDC]: Support the proposal. [Xiaomi]: Share the view with Samsung. [LG]: Would like to have more time to check. Revisit it next meeting.

RAN1 has discussed this issue in the RAN1#120 meeting and achieved the following conclusion:

Conclusion - The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system. - RA-RNTI is not included in the UL-WUS configuration for FDD system. - Cell barring information is not included in the UL-WUS configuration for FDD system. - Cell selection information is not included in the UL-WUS configuration for FDD system. - Accessible UE types is not included in the UL-WUS configuration for FDD system. - CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration for FDD system. - InitialUplinkBWP is not included in the UL-WUS configuration for FDD system. - TotalNumberOfRA-Preambles is not included in the UL-WUS configuration for FDD system. - SIB1-RequestResourceRedCap is not included in the UL-WUS configuration for FDD system. - SupplementaryUL is not included in the UL-WUS configuration

The SUL is utilized to enhance the UL coverage by using a lower frequency. In the NES scenario, when the UE triggers the OD-SIB1 transmission, SUL would be beneficial for UEs at cell edge. However, in certain scenarios, the energy-saving benefits of the NES Cell are reduced when OD-SIB1 is triggered by some UEs with poor channel conditions, due to transmitting the OD-SIB1 request on the SUL. It is also worth noting that after OD-SIB1 is received, a UE can still perform RACH on SUL as in legacy (i.e., UE can use SUL after SIB1 is received). Therefore, the UE can still have the benefits of SUL on the NES cell after obtaining SIB1.

Based on RAN1’s conclusion, the OD-SIB1 request can only be transmitted on NUL. Taking into account the advantages and disadvantages of this agreement, RAN2 can follow the conclusion.

Proposal 3: Following RAN1’s conclusion, OD-SIB1 request can only be transmitted on NUL.

## 2.4 The definition of ra-SearchSpace in SIB1-RequestConfig

RAN1 has agreed to an updated consolidated Rel-19 higher layers parameters list Post RAN1#121 in [3], which was provided to RAN2. In the list RAN1 suggests that ra-SearchSpace is defined as a type SearchSpace, citing the following RAN1 agreements:

Agreement (RAN1#118b)

Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell.

Agreement (RAN1#120bis)

The parameter ra-SearchSpace in the UL-WUS configuration is of type SearchSpace.

The initial implementation of ra-SearchSpace in SIB1-RequestConfig was defined as of type SearchSpaceId. The issue with this implementation is that in the PDCCH-Config where SIBxx is sent, the search space is for cell A.

Observation 1: The search space provided in SIBxx should be the configuration of the NES cell.

If SIBxx includes the search space ID for the NES cell, the search space configuration of the NES cell also needs to be provided in SIBxx, it cannot be a SearchSpaceId from Cell A. Therefore, we either need to provide a full list of search spaces for the NES cell, or a single search space, as defined in the RAN1 parameter list. In our view one is enough because the configuration is only for RACH purposes.

Proposal 4: The search space for the NES cell in SIBxx is of type SearchSpace, i.e., a full search space configuration.

## 2.5 The implementation of si-BroadcastStatus agreements

RAN2 has discussed the issue of how to ensure that the RRC connected UEs have the latest SIB1 and its implication to si-BroadcastStatus of other SI with the following agreements:

Agreement (RAN2#129bis)

NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

Agreement (RAN2#130)

Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

It has been currently captured in 5.2.2.3.5:

2>	a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one.

There is a left open issue whether to capture a similar statement in the field description of si-BroadcastStatus. Firstly, we think it is beneficial to capture that it applies only to cells with OD-SIB1 enabled, e.g. add to the procedure that it is "in a cell providing OD-SIB1”.

Proposal 5: Clarify in the normative text that a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one if a cell is providing OD-SIB1.

Since the normative text already captures the UE behaviour, it is optional to include it in si-BroadcastStatus. In our view it can be captured in both places for clarity of the si-BroadcastStatus behaviour.

## 2.6 UE behaviour on how to obtain SIB1 when the NES Cell sends the SI update message

In the [POST130] [107][NES] email discussion, the issue of how to obtain SIB1 when a NES cell sends the SI update message was identified. The issue comes from a previous agreement at the RAN2#126 meeting:

Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Since the NES UE has camped on the NES Cell, it has the latest SIB1 information, including searchspace and CORESET. When the SI change notification is received, the UE can use the searchspace and CORESET information to receive SIB1 and check which SI has been changed.

Observation 2: The searchspace and CORESET information is provided in SIB1 of the NES cell.

However, since the NES Cell will not broadcast SIB1 periodically, there is an issue whether the UE can receive the SIB1 directly after SI change notification or whether the UE needs to trigger OD-SIB1 request. The issue lies in the UE behaviour regarding how to obtain SIB1 when the NES Cell sends the SI update message.

In our view, since the SI change is triggered by the NW, it has the responsibility to send the updated SIB1 to indicate which SI has been changed. Triggering OD-SIB1 signalling from each UE should be avoided as it would cause increased energy consumption both on UE and NW side. Hence, the following previous agreement could work for this issue:

After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

The main remaining question is whether the UE can use the searchspace and CORESET received in the last SIB1 or WUS configuration to receive the updated SIB1. If there is no change in the searchspace or CORESET, the UE can utilize this information to receive SIB1. However, if the search space or CORESET has been altered, it is unclear how SIB1 should be received.

- Option 1: The NW sends CD-SSB when the search space or CORESET has been modified. The UE can use the searchspace and CORESET information in pdcch-configSIB1 to receive SIB1. The new UE behaviour needs to be specified for the special case of searchspace and CORESET changes.

- Option 2: It is up to NW implementation to address the issue. For example, the NW releases the OD-SIB1 operation and returns to the normal state.

Proposal 6: RAN2 to discuss the behaviour when receiving an SI change notification if the searchspace and CORESET have been changed.

# Conclusion

In this contribution, we discussed the remaining issues of on on-demand SIB1 operation, and made the following observations and proposals:

Proposal 1: Once a NES UE acquires Cell A’s SIB1 and knows that it provides SIBxx, it can acquire Cell A’s SIBxx to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

Proposal 2: If the value in an OD-SIB1 IE is different from the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration.

Proposal 3: Following RAN1’s conclusion, OD-SIB1 request can only be transmitted on NUL.

Observation 1: The search space provided in SIBxx should be the configuration of the NES cell.

Proposal 4: The search space for the NES cell in SIBxx is of type SearchSpace, i.e., a full search space configuration.

Proposal 5: Clarify in the normative text that a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one if a cell is providing OD-SIB1.

Observation 2: The searchspace and CORESET information is provided in SIB1 of the NES cell.

Proposal 6: RAN2 to discuss the behaviour when receiving an SI change notification if the searchspace and CORESET have been changed.

# References

- R2-2504671 Report from session on R18 SL, R18/19 MOB and R19 NES

- Chair notes RAN1#120 eom0.doc

- R1-2503243 Consolidated Rel-19 higher layers parameters list Post RAN1#121