3GPP TSG-RAN WG2 Meeting #131                                     R2-2505572

Bangalore, India, Aug 25th – 29th, 2025

Source: 			OPPO

Title: 	Discussion on NTN MBS service area related open issues

Agenda item:		8.8.4

Document for: 	Discussion and Decision

- Introduction

Agreements:

- RAN2 assumes that the ISA can be valid for all the frequencies providing the MBS session.

- Do not introduce service area per neighbour cell in MCCH

- Do not introduce service area information specific for FSAI in SIB21.

- The new SIBxx containing MBS ISA(s) can be cell or area specific, which is up to NW implementation (No specs change).

In the last RAN2#130 meeting, agreements on the support of broadcast service in the NTN network have been achieved as follows:

In the post email discussion [1], some open issues have been raised by other companies. In this contribution, we aim to provide our views on them.

- Discussion

- Open issue#1: whether an explicit association between ISA and frequencies for frequency prioritization is required in AS layer

Since the Service Announcement carries the mapping relationship between FSA ID and ISA, the UE can know which frequency to be prioritized for inter-cell mobility from the Service Announcement information, there is no need to embed such information in AS layer.

Proposal 1: explicit association between ISA and frequencies for frequency prioritization is not required in AS layer.

- Open issue#2: whether RAN2 should use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX

As mentioned by the Rapporteur of the email discussion, the circular representation based on referenceLocation and distanceRadius exhibits a wider range than the Ellipsoid-PointWithUncertainCircle (range 3000 km vs. 1800km). In addition, if we want to adopt Ellipsoid-PointWithUncertainCircle into the definition of the ISA, the IE description needs to be changed also, since the range does not represent uncertainty about the positioning result anymore

Proposal 2: not to adopt the Ellipsoid Point with uncertainty circle from TS 37.355 to define the circle shape of the ISA in the new SIBxx

- Open issue#3: whether or not satellite movement should trigger MCCH update?

Due to the movement of satellites, the UE will need to perform cell reselection or handover towards the newly entered cell. As a result, the UE anyway needs to obtain related information from the network. If the UE camped cell is change, then for the convenience of MBS service continuity, it can be up to the UE implementation whether or not to update the MCCH information.

Proposal 3: it is up to the UE implementation whether or not to update the MCCH information.

Conclusion and proposals

In this paper, following proposals have been made by us:

Proposal 1: explicit association between ISA and frequencies for frequency prioritization is not required in AS layer.

Proposal 2: not to adopt the Ellipsoid Point with uncertainty circle from TS 37.355 to define the circle shape of the ISA in the new SIBxx

Proposal 3: it is up to the UE implementation whether or not to update the MCCH information.

Reference:

[1]: (5841) R2-25xxxx [draft] Open_issue_list_RRC_NTN, Ericsson