3GPP TSG-RAN2#131	R2-2505587

Bengaluru, India, August 25 – August 29, 2025

Agenda item:		8.7.6

Source:		Lenovo

Title:		 Remaining open issues on XR Rate Control

Document for:		Discussion and Decision

- Introduction

For XR traffic, it would be beneficial to enable the application to adapt the codec rate based on the network conditions to avoid dropping of packets over the air interface. Such discussion has been ongoing in the RAN Plenary with the following being agreed as the way forward in the last meeting, RAN #105 [1].

- Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

Additionally, the following agreements were made at the RAN2 #128and RAN2 #129 meetings, as follows:

Agreements from RAN2 #129 - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed. - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this Agreements from RAN2 #129bis - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID. Agreements from RAN2 #130 - (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. - We rule out explicit signalling of PDU session ID + QFI - We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping - (RRC-1) The granularity of bit rate query prohibit timer is QoS flow. - FFS The value of the prohibit timer is the same for all flows - (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow. - (MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit. - (MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate. - No optimization is needed to address DC case for XR rate control MAC CE - We capture in stage-2 that XR rate indication is for application and not for MAC entity. - RAN2 will not specify DL rate control and DL rate control query.

This contribution looks at how the XR rate control information may be signaled in the downlink to support source rate adaption to uplink congestion. The remaining open issues are discussed in this contribution.

- Discussion

For XR applications that have very strict requirements in terms of data rate and latencies, it is beneficial for the application to be able to adapt the source rates based on the congestion and network conditions on the uplink. During the CR reviews, some open issues have been identified regarding the design and specification of XR Rate Control procedure.

During the RAN2#129bis meeting, RAN2 had agreed that UL rate indication will be on a per QoS flow basis unless any issues are identified by RAN3. And it was FFS how this flow ID is indicated in the MAC CE. In our view, it is sufficient to go with the DRB ID and the QFI ID to be able to differentiate the different flows across PDU sessions. Furthermore, since the DRB ID is already indicated in the legacy RBR MAC CE, this is a simpler approach.

(MAC-3) The UL rate recommendation on a per QoS flow is indicated using explicit DRB ID and explicit QFI ID.

In the last meeting, it was also agreed that only a single table will be designed for the bitrate recommendation with 8 bits, exponential distribution and no multiplier. Accordingly, RAN2 also needs to discuss the format of the Rate Control MAC CE. In our view, the MAC CE needs to at least consist of the following fields:

- DRB ID

- QoS Flow ID (QFI)

- Bit Rate Recommendation

Additionally, we think that a multiplier field is not needed, i.e., we do not use the legacy multiplier values in the new MAC CE, since the new table will be specified without a multiplier and intended for the range of data rates provided by SA4. The number of bits to indicate QFI and DRB ID should be at least 6 and 5 respectively.

(MAC-4) The Rate Control MAC CE consists of the following fields: DRB ID, QoS Flow ID (QFI), Bit Rate Recommendation.

- The number of bits for DRB ID shall be at least 5 bits.

- The number of bits for QFI shall be at least 6 bits.

- The number of bits for Bit Rate Recommendation shall be 8 bits.

(MAC-4) The Rate Control MAC CE does not include a bit rate multiplier field.

For multiplexing, we think that the UL Rate Control Query MAC CE can be multiplexed with the same priority as the legacy RBR Query MAC CE as already implemented in the current CR. We don’t see a good motivation to change such behaviour.

(MAC-4) The Rate Control MAC CE can be multiplexed with the same priority as the legacy RBR Query MAC CE.

- Regarding the open issue on whether the UL Rate Control MAC Control Element (MAC CE) shall only be transmitted when the available UL-SCH resources can accommodate all pending queries (i.e., all entries in the MAC entity’s bit rate query list), our view is that such a straightforward transmission condition is sufficient.

- We do not identify a technical need to introduce a “truncated” UL Rate Control MAC CE format, as the MAC CE is not considered to be latency-critical. In scenarios where the available UL-SCH resources are insufficient, the UE may defer transmission of the MAC CE until an UL grant with adequate UL-SCH resources is received.

- Proposal 5: (MAC-5) a UL Rate Control MAC CE is transmitted only if the available UL-SCH resources can accommodate all the pending queries.

- Conclusion

It is proposed to agree on the following proposals:

- Proposal 1: 	(MAC-3) The UL rate recommendation on a per QoS flow is indicated using explicit DRB ID and explicit QFI ID.

- Proposal 2:	(MAC-4) The Rate Control MAC CE consists of the following fields: DRB ID, QoS Flow ID (QFI), Bit Rate Recommendation.

- The number of bits for DRB ID shall be at least 5 bits.

- The number of bits for QFI shall be at least 6 bits.

- The number of bits for Bit Rate Recommendation shall be 8 bits.

- Proposal 3:	(MAC-4) The Rate Control MAC CE does not include a bit rate multiplier field.

- Proposal 4:	(MAC-4) The Rate Control MAC CE can be multiplexed with the same priority as the legacy RBR Query MAC CE.

- Proposal 5:	(MAC-5) a UL Rate Control MAC CE is transmitted only if the available UL-SCH resources can accommodate all the pending queries.

- References

[1] RP-242390: Way Forward on WID Scope Revision, Nokia

[2] R3-250805, Reply LS on XR UL Rate Control, RAN3