3GPP TSG-RAN WG2 #131	R2-2505596

Bengaluru, India, 25 – 29 August 2025

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on RRM measurement relaxation and offloading

Document for:	Discussion and decision

Agenda Item:	8.4.3

- Introduction

In RAN2#130 meeting [1], the following open issues were discussed with no conclusion:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-7 | FFS on exit condition for serving cell RRM relaxation Editor’s NOTE: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-8 | FFS on whether/how to reduce the threshold number for LP-WUS/WUR Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and entry/exit condition of using LP-WUS, [and potential pre-condition between RRM relaxation/offloading criteria and entry/exit condition of using LP-WUS]. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-10 | FFS on low mobility criteria Editor’s NOTE: FFS on “low mobility” criteria. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

In this contribution, we will further discuss the above issues for RRM measurement relaxation and offloading.

- Discussion

  - Exit condition for serving cell RRM relaxation (RRC-7)

There are two FFS points on the exit condition for serving cell RRM relaxation as follows:

- FFS on whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed

- FFS on whether exit condition for serving cell RRM relaxation include MR and/or LR-based measurements

For the first FFS, we think it is preferrable to define separate exit condition from ‘not fulfilling the entry condition’ to avoid potential ping-pong issue.

For the second FFS, LR-based measurement should be suitable for the exit condition for serving cell RRM relaxation. During the serving cell RRM relaxation period, MR-based measurement quality is relaxed with longer measurement cycle. Thus, it is not reasonable to consider MR-based measurement for exit condition of the serving cell RRM relaxation.

Proposal 1: Separate exit condition from ‘not fulfilling the entry condition’ is defined to avoid potential ping-pong issue. The exit condition is based on LR measurement.

  - whether/how to reduce the threshold number for LP-WUS (RRC-8)

In our understanding, there are two FFS points on the reduction of the threshold number for LP-WUS as follows:

- FFS on relationship between thresholds of Rel-19 neighboring cell RRM relaxation and Rel-16/Rel-17 neighboring cell RRM relaxation

- FFS on relationship between thresholds of Rel-19 relaxation (including serving cell and neighboring cell)/offloading and LP-WUS monitoring

For the first FFS, it is straightforward that threshold of Rel-19 neighboring cell RRM relaxation is higher than that of Rel-16/17 neighboring cell RRM relaxation because Rel-19 neighboring cell RRM relaxation intends to relax the RRM measurement more than Rel-16/17.

Besides, it was agreed that the entry and exit conditions of Rel-19 serving cell RRM measurement relaxation and Rel-19 neighboring cell RRM measurement relaxation are identical at RAN2#129b [2]. If the threshold of Rel-19 neighboring cell RRM measurement relaxation is lower than that of Rel-16/17 neighboring cell RRM relaxation, Rel-19 serving cell RRM measurement relaxation is triggered before Rel-16/17 neighboring cell RRM relaxation. It is not reasonable.

Hence, the threshold of Rel-19 neighboring cell RRM relaxation is higher than that of Rel-16/17 neighboring cell RRM relaxation.

Proposal 2: threshold of Rel-19 neighboring cell RRM relaxation is higher than that of Rel-16/17 neighboring cell RRM relaxation.

For the second FFS, thresholds of Rel-19 relaxation and offloading for LP-WUS monitoring need to be separately discussed.

Regarding the relationship between Rel-19 relaxation and LP-WUS monitoring, Rel-19 RRM relaxation should work together with LP-WUS monitoring because energy saving gain of LP-WUS monitoring only is small based on the outcome of Rel-18 study. Thus, the threshold of LP-WUS monitoring should be higher than or equal to that of Rel-19 RRM relaxation.

About the relationship between Rel-19 RRM offloading and LP-WUS monitoring, LP-WUS monitoring should work together with Rel-19 RRM offloading as well as Rel-19 RRM relaxation. At RAN2#130 meeting, it was agreed that threshold of Rel-19 RRM offloading is higher than that of Rel-19 RRM relaxation. By taking the relationship between Rel-19 RRM offloading and Rel-19 RRM relaxation into account, the threshold of LP-WUS monitoring is lower than or equal to that of Rel-19 RRM offloading.

Proposal 3: threshold of LP-WUS monitoring should be higher than or equal to that of Rel-19 RRM relaxation.

Proposal 4: threshold of LP-WUS monitoring should be lower than or equal to that of Rel-19 RRM offloading.

  - Low mobility criteria (RRC-10)

Two cases can be considered separately for whether UE low mobility criterion is considered for RRM relaxation/offloading. One is RRM offloading case and the other is RRM relaxation case including both serving cell and neighboring cell measurements.

For RRM offloading case, UE with high mobility would move to cell edge after it transits to RRM offloading state based on measurement at cell center. In this scenario, for cell reselection, the UE needs to wake its MR up by satisfying the offloading exit condition and to do neighboring cell measurement by the MR. It means relatively long latency is required for sell reselection, e.g., 400 ms is assumed in Rel-18 SI for wake-up time of MR.

For RRM relaxation case, similarly, handover latency would increase for UE with high mobility due to the RRM relaxation if UE low mobility criterion is not considered.

Therefore, UE low mobility criterion should be considered for RRM relaxation/offloading. The existing UE low mobility criterion defined in section 5.2.4.9.1 of TS38.304 can be reused for Rel-19 RRM relaxation/offloading in our understanding.

Proposal 5: UE low mobility criterion is considered for RRM relaxation/offloading.

If UE low mobility criterion is considered for RRM relxation/offloading, structure of RRC parameter, lowMobilityEvaluation, needs to be discussed. In the current running CR of TS 38.331 [3], the parameters for cell edge evaluation, cellEdgeEvaluation, are introduced for LP-SS/SSB measurements with LR/MR, respectively. It is straightforward to follow this manner for lowMobilityEvaluation as thresholds can be different for receiver type and measurement signal.

Proposal 6: If UE low mobility criterion is considered for RRM relaxation/offloading, the parameters for cell edge evaluation, lowMobilityEvaluation, are introduced for LP-SS/SSB measurements with LR/MR, respectively.

- Conclusion

In this contribution, we discuss the remaining issues for RRM measurement relaxation and offloading. Based on the discussion, we have the following proposals.

Proposal 1: Separate exit condition from ‘not fulfilling the entry condition’ is defined to avoid potential ping-pong issue. The exit condition is based on LR measurement.

Proposal 2: threshold of Rel-19 neighboring cell RRM relaxation is higher than that of Rel-16/17 neighboring cell RRM relaxation.

Proposal 3: threshold of LP-WUS monitoring should be higher than or equal to that of Rel-19 RRM relaxation.

Proposal 4: threshold of LP-WUS monitoring should be lower than or equal to that of Rel-19 RRM offloading.

Proposal 5: UE low mobility criterion is considered for RRM relaxation/offloading.

Proposal 6: If UE low mobility criterion is considered for RRM relaxation/offloading, the parameters for cell edge evaluation, lowMobilityEvaluation, are introduced for LP-SS/SSB measurements with LR/MR, respectively.

- References

[1]	R2-2504672, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others, RAN2 Vice Chairman (CATT)

[2]	R2-2502982, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others, RAN2 Vice Chairman (CATT)

[3]	R2-2505392, Introduction of LP-WUS/WUR in RRC, vivo (Rapporteur)