3GPP TSG-RAN WG2 Meeting #131	R2-2505615

India, Bengaluru, August 25th – 29th, 2025

Agenda Item:	8.2.3

Source:	Panasonic

Title: 	Msg2 monitoring window for A-IoT random access

Document for:	Discussion

# Introduction

Agreements made in RAN2#130 meeting in relevance to this document are listed below, including the agreements for the possible options as agreed for Msg2 boundary condition, to determine contention failure of A-IoT devices.

- Agreements on RA - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. Agreements on NACK reception: - After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify.

This document discusses an open issue for A-IoT random access procedure, related to the requirements and considerations for Msg2 monitoring window or boundary condition.

# Discussion

It was discussed in previous meeting about different options possible for Msg2 boundary condition and among multiple options, only options B and C were seen suitable. But further, the best approach could not be concluded among option B or C, due to lack of pros and cons comparison between the one from the other.

Though option C could be seen as more flexible approach, and further by defining configurability of the K value, reader can then set value of K=1 to obtain equivalent to option B, to dynamically control the Msg2 boundary which may result in a flexible solution. But the feasibility and the benefits of option C and to introduce the configurability of the value K should be further evaluated based on the following aspects,

- Impact on contention resolution probability.

- Deployment or implementation complexity.

Observation 1: Option C can be beneficial if used with dynamic configurability of the K value to specify a flexible solution, which also implies that solution equivalent to Option B can be implemented by setting K = 1.

Observation 2: Higher flexibility for option C increases complexity for A-IoT operations to specify dynamic configuration options for the K value.

How is contention probability impacted by combining multiple trigger messages?

Paging message triggered for CBRA procedure provides access occasions for one or more devices to send a paging response message along with a 16bit random number transmitted with the message. Then the contention among multiple devices triggered during CBRA procedure is dependent on the number of access occasions (the Q factor) and the code space created from the size of random number generation (which is 16bits). Also from the current design, the total number of access occasions (or Q) are provided in terms of number of frequency (FDMed) and time (TDMed) domain occasions, triggered by one or more Access trigger messages per paging round.

For option B the Q factor can be expressed using the number of occasions possible to be triggered by each Access trigger message.

i.e., Q factor = number of FDMed occasions * number of TDMed occasions

TDMed occasions can be X = 1 or 2, for each Access trigger message.

For the case of option C in comparison, the Q factor reduces by the value K because the code space available from the size of the random number generation (16bits) is reduced by this same factor, since only single Msg2 will be used to resolve contention for the responses sent on multiple set of K access trigger messages.

i.e., Q factor = number of FDMed occasions * number of TDMed occasions * K

Further in option C to realise similar contention resolution probability equivalent to option B, the FDMed occasions may have to be increased or the number of access trigger messages per paging round should be increased to provide better access opportunities for all A-IoT devices participating in the group.

Observation 3a: For Option C the contention resolution probability may be optimised by increasing the number of FDMed (or TDMed) occasions per access trigger message. Or by increasing the total number of access occasions per paging round to provide better access opportunity to the group of A-IoT devices.

Observation 3b: In contrast, for Option B a similar contention resolution probability can be realised using smaller number of FDMed or TDMed occasions or even lesser number of access trigger messages per paging round, as compared to Option C executed with a given group of A-IoT devices.

Observation 4: Option C may not offer any significant improvements to reduce overhead with R2D messages sent from the reader (i.e. Msg2 or Access trigger messages), to achieve/ realise a required contention resolution probability among group of A-IoT devices.

As a further analysis, it may be possible in option C to make the contention resolution similar to option B, by introducing time domain indexing in Msg2 to indicate the corresponding TDMed occasion on which the random ID was detected. By this approach the reusability of each random ID value (in the range of the random ID code space) is now increased to just within one access trigger message (equivalent to option B) irrespective of the number of K used.

But it was decided in the last meeting that the TDMed occasion index is not feasible to be indicated to A-IoT devices in CBRA procedure, the agreement can be seen below.

3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it.

Observation 5: TDM occasion index can solve the contention resolution probability problem highlighted, but the feasibility to introduce such index is not agreed/ not clear.

How is implementation complexity impacted?

For Option C based implementation, devices must support an additional counter to track the number of Access trigger messages configured or indicated by K value. Then this approach increases the device side complexity as compared to Option B which is much simpler to implement or specify.

Observation 6: Without much observed improvements with option C, may be in terms of reducing overhead with R2D messages sent by the reader, to increase the complexity at the device side as to support additional counters may not be the best approach.

Proposal 1: Support simpler solution as in option B which does not involve any other complexities both at the reader side (e.g, dynamic configuration requirements of K value in case of option C) and at the device side (e.g., no additional counters are necessary).

Proposal 1a: It is sufficient to support option B approach (Msg2 boundary upon next R2D trigger message or the subsequent paging message), to attain necessary contention resolution probability among any group size of A-IoT devices, by controlling the number of FDMed/ TDMed occasions per access trigger message, and the total number of access trigger messages per paging round.

# Conclusions

This document provides pros and cons of the method discussed in option C in comparison to option B, for the Msg2 monitoring window discussions:

Benefits of option C:

Observation 1: Option C can be beneficial if used with dynamic configurability of the K value to specify a flexible solution, which also implies that solution equivalent to Option B can be implemented by setting K = 1.

Disadvantages of option C (or benefits of option B):

Observation 2: Higher flexibility for option C increases complexity for A-IoT operations to specify dynamic configuration options for the K value.

Observation 3a: For Option C the contention resolution probability may be optimised by increasing the number of FDMed (or TDMed) occasions per access trigger message. Or by increasing the total number of access occasions per paging round to provide better access opportunity to the group of A-IoT devices.

Observation 3b: In contrast, for Option B a similar contention resolution probability can be realised using smaller number of FDMed or TDMed occasions or even lesser number of access trigger messages per paging round, as compared to Option C executed with a given group of A-IoT devices.

Observation 4: Option C may not offer any significant improvements to reduce overhead with R2D messages sent from the reader (i.e. Msg2 or Access trigger messages), to achieve/ realise a required contention resolution probability among group of A-IoT devices.

Observation 5: TDM occasion index can solve the contention resolution probability problem highlighted, but the feasibility to introduce such index is not agreed/ not clear.

Observation 6: Without much observed improvements with option C, may be in terms of reducing overhead with R2D messages sent by the reader, to increase the complexity at the device side as to support additional counters may not be the best approach.

Proposals for discussion in RAN2:

Proposal 1: Support simpler solution as in option B which does not involve any other complexities both at the reader side (e.g, dynamic configuration requirements of K value in case of option C) and at the device side (e.g., no additional counters are necessary).

Proposal 1a: It is sufficient to support option B approach (Msg2 boundary upon next R2D trigger message or the subsequent paging message), to attain necessary contention resolution probability among any group size of A-IoT devices, by controlling the number of FDMed/ TDMed occasions per access trigger message, and the total number of access trigger messages per paging round.