3GPP TSG-RAN WG2 Meeting #131	R2-2505628

Bengaluru, India, Aug 25th –29th, 2025

Agenda Item:	8.2.3

Source:	Huawei, HiSilicon

Title:	A-IoT random access procedure

Document for: Discussion and Decision

1	Introduction

In this contribution, the details of A-IoT random access procedure are further discussed.

2	Discussion

2.1	Msg2 boundary and frequency index (2-4, 2-5)

In RAN2#130 meeting, there were the following agreements [1] on Msg2 boundary.

Agreements on RA 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it.

Figure 2.1-1	Registers maintained on the device side

Option B leads to scheduling restriction on the reader side, all the MSG2 re-transmission can only be done before next R2D trigger message. Option C can provide flexibility for the reader. To avoid unacceptable complexity for the device, K can be a fixed value. Assuming that K is 4, 2-bit counter is needed to maintain the number of received R2D trigger messages. Msg1 related registers can be reused to implement this 2-bits counter without requiring additional registers; for example, the storage for AO selection counter can be released after the device determines its Msg1 transmission, and it can be used for this 2-bit counter without requiring additional registers.

Observation 1:	If we assume K=4,2-bit counter can reuse the Msg1 related registers (TDM) without requiring additional registers.

Proposal 1:	(Issue 2-4) Option C with K fixed as 4 is supported, which allows reader flexibility without additional device complexity.

On whether to introduce 3-bit frequency index in Msg2, similarly as for the counter for K above, the 3-bit register for frequency index can reuse the Msg1 related registers without requiring additional registers. The 3-bit frequency index can reduce the collisions of Msg1 between different devices that select the same random ID but different Msg1 frequency resources. On the other hand, it introduces some overhead for Msg2.

Observation 2:	The 3-bit register for frequency index, used after Msg1 transmission, can reuse the Msg1 related registers (TDM) without requiring additional registers.

Proposal 2:	(Issue 2-5) RAN2 to discuss whether to introduce 3-bit frequency index for each echoed random ID in Msg2.

2.2	Re-access determination

For re-access, if the previous procedure was determined as failed for a specific Transaction ID, e.g., CBRA procedure has not been considered as successful, or NACK Feedback message was received, the device will perform the re-access. However, one case is missing, i.e., the selected device in this paging round has not sent Msg1 (e.g., due to missing of some Access Trigger messages) and receives the subsequent paging. In this case, the device also needs to perform the re-access.

Proposal 3:	RAN2 confirms that if the selected device has not sent Msg1 (e.g., due to missing of some Access Trigger messages), upon receiving the subsequent paging message, it should also perform re-access (as captured in the MAC running CR).

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	If we assume K=4,2-bit counter can reuse the Msg1 related registers (TDM) without requiring additional registers.

Observation 2:	The 3-bit register for frequency index, used after Msg1 transmission, can reuse the Msg1 related registers (TDM) without requiring additional registers.

Msg2 boundary and frequency index (2-4, 2-5)

Proposal 1:	(Issue 2-4) Option C with K fixed as 4 is supported, which allows reader flexibility without additional device complexity.

Proposal 2:	(Issue 2-5) RAN2 to discuss whether to introduce 3-bit frequency index for each echoed random ID in Msg2.

Re-access determination

Proposal 3:	RAN2 confirms that if the selected device has not sent Msg1 (e.g., due to missing of some Access Trigger messages), upon receiving the subsequent paging message, it should also perform re-access (as captured in the MAC running CR).

# 4	References

- R2_130_Chair_minutes.