3GPP TSG-RAN WG2 Meeting #131	R2-2505630

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.4.4

Source:	Huawei, HiSilicon

Title:	Further discussion on LP-WUS for RRC_CONNECTED mode

Document for:	Discussion and Decision

- Introduction

In this contribution, we further discuss higher layer design for CONNECTED mode.

- Discussion

  - LP-WUS configuration and enabling/disabling

Regarding the issue of how the NW provides the configuration to the UE, it can rely on the existing RRC measurement report. For example, the NW configures the UE to report measurement results based on some events and periodicity, then the NW can know whether it is suitable for the UE to start using LP-WUS functionality, e.g. the NW finds that the measurement result is good enough, or the change of the measurement result is small enough, the NW can configure the UE to use LP-WUS. However, the NW only knows the measurement results from MR. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference for LR.

Observation 1: The NW only knows the measurement results from MR based on existing RRC measurement report. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference for LR.

In this case, the UE can inform the NW to stop using LP-WUS since the UE knows the link quality of LR, and also indicate whether the LP-WUS can be used again if it finds that the LR link quality is good enough. For example, the UE is monitoring LP-WUS and measures LP-SS/SSB by LR, once it finds that the LR link quality is not good enough, the UE switches to MR and sends assistance information to the NW to disable the LP-WUS. How the UE determines the LR link quality is not good enough and sends the assistance information, it can be up to UE implementation without configuring additional conditions/thresholds.

Proposal 1: (MAC-X3) UE can send assistance information to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

Proposal 2: (MAC-X3) When to send the assistance information is up to UE implementation, without additional configuration to the UE.

After the NW provides the LP-WUS configuration to the UE by RRC signaling, the UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality. Separate activation/deactivation signaling, e.g., L1/L2 signaling is not needed, since it won’t change dynamically.

Proposal 3: The UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality after receiving LP-WUS configuration by RRC signaling. Separate activation/deactivation signaling is not supported.

We understand enabling LP-WUS functionality does not mean UE monitors LP-WUS. Once the LP-WUS functionality is enabled, UE can monitor LP-WUS (i.e., outside C-DRX active time), or UE does not monitor LP-WUS but monitors PDCCH (i.e., during C-DRX active time). Thus, one issue that needs discussion is whether the UE starts LP-WUS monitoring once it receives the LP-WUS configuration.

Observation 2: Enabling LP-WUS functionality does not imply UE monitors LP-WUS. Once the LP-WUS functionality is enabled, UE can monitor LP-WUS (i.e., outside C-DRX active time), or UE does not monitor LP-WUS but monitors PDCCH (i.e., during C-DRX active time).

One option is to start LP-WUS monitoring immediately after receiving the LP-WUS configuration. It seems like LP-WUS configuration can trigger the UE enter LP-WUS monitoring status. However, if the UE applies LP-WUS monitoring ahead of the NW, ongoing services from the NW may be affected which causes data loss for the UE. On another hand, if the LP-WUS functionality takes effect on the NW before the UE, UE may miss the LP-WUS which leads to LR/MR state mismatch between the UE and the NW. Thus, it is beneficial that the UE and NW can align the starting point for monitoring LP-WUS, e.g. the UE and the NW start LP-WUS monitoring after a time period after the UE sends RRC reconfiguration complete message.

Another option is to keep the UE stay in C-DRX Active Time (i.e., using MR) after it receives the LP-WUS configuration. As the UE receives RRC reconfiguration message and needs to receive PDCCH scheduling PUSCH for sending RRC reconfiguration complete message, the UE can naturally stay in C-DRX Active Time. And there is no interruption for ongoing service if any. After the data transmission is finished and C-DRX Active Time ends, the UE goes back to LP-WUS monitoring according to the agreed UE’s behavior.

Proposal 4: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring immediately after it receives the LP-WUS configuration, alignment of the starting point for LP-WUS monitoring needs to be further studied;

- Alt2: UE stays in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE’s behavior.

In RAN2#126 meeting, we had the following agreement:

- RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

It is unclear whether the same principle can be applied to Option 1-2. Since Option 1-2 can technically provide better performance for power saving and latency compared with DCP, there is no strong motivation to configure both legacy DCP and Option 1-2.

Proposal 5: RAN2 to discuss whether legacy DCP and Option 1-2 can be configured simultaneously for the same UE.

  - The impact to the BWP switching mechanism

RAN1 is still discussing the collision case that includes BWP switching. One option is that BWP switching can be avoided when UE monitors LP-WUS. Only timer-based BWP switching (instead of DCI/RRC based BWP switching) can happen during LP-WUS monitoring, which is a fallback operation to the default/initial DL BWP for power saving purpose. However, the power consumption of LP-WUS monitoring in a non-default/non-initial BWP would be smaller than PDCCH monitoring in a default/initial BWP. Therefore, the bwp-InactivityTimer can be stopped when UE monitors LP-WUS. In this case, the UE needs to (re)start the bwp-InactivityTimer when UE receives the LP-WUS for PDCCH monitoring.

Proposal 6: (MAC-X1) The bwp-InactivityTimer can be stopped when UE monitors LP-WUS, in this case, the UE needs to (re)start the bwp-InactivityTimer when it receives the LP-WUS for PDCCH monitoring. Otherwise, there is no impact on bwp-InactivityTimer.

- Conclusion

In this paper, we discuss higher layer design for CONNECTED mode, and have the following proposals.

Configuration for LP-WUS

Proposal 1: (MAC-X3) UE can send assistance information to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

Proposal 2: (MAC-X3) When to send the assistance information is up to UE implementation, without additional configuration to the UE.

Proposal 3: The UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality after receiving LP-WUS configuration by RRC signaling. Separate activation/deactivation signaling is not supported.

Proposal 4: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring immediately after it receives the LP-WUS configuration, alignment of the starting point for LP-WUS monitoring needs to be further studied;

- Alt2: UE stays in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE’s behavior.

Proposal 5: RAN2 to discuss whether legacy DCP and Option 1-2 can be configured simultaneously for the same UE.

The impact to the BWP switching mechanism

Proposal 6: (MAC-X1) The bwp-InactivityTimer can be stopped when UE monitors LP-WUS, in this case, the UE needs to (re)start the bwp-InactivityTimer when it receives the LP-WUS for PDCCH monitoring. Otherwise, there is no impact on bwp-InactivityTimer.

- References

R2-2501332 Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD	Vice Chairman (CATT) report.

R1-2504888 Reply LS on LP-WUS in RRC_CONNECTED.