3GPP TSG-RAN WG2 Meeting #131	R2-2505635

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Msg3 indication on support of Msg4 PDSCH repetition

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN1 has agreed to support for Msg4 PDSCH repetition for DL coverage enhancement. In the RAN1 LS R1-2504936, RAN2 is expected to design Msg3 indication to inform network the support of Msg4 PDSCH repetition. On activation/deactivation of Msg4 PDSCH repetition, below agreements were reached in RAN1.

Agreement For the activation/deactivation of Msg4 PDSCH repetition: - Alt 1: UE specific PDSCH with Msg4 repetition activation indicated via DCI Format 1_0: - Signaling uses re-interpretation of 1 MSB in MCS field in DCI. - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH. - Note: RAN1 considers there is no difference between capability and request - FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs. - The aggregation factor is configured in SIB1, with possible value 2 or 4 - When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled.

In this document, we provide our view on how to indicate the information related to Msg4 PDSCH repetition capability to network.

# 2	Discussion

Fisrt of all, for the Msg3 indication on UE capabilities, RAN2 had a similar discussion on PUCCH repetitions for Msg4 HARQ-Ack during Rel-18 WI. In short, RAN2 has extended the LCID space for additional 64 codepoints which can be used for indication of a given feature or feature combination via Msg3. In current specification, the LCID-extension is used for feature combination among the features of 48/64 bits CCCH, PUCCH repetition for Msg4 HARQ-ACK, RedCap UE and eRedCap UE. As illustrated in below TS38.321 table 6.2.1-2c, within the 64 extend-LCID codepoints, 8 was used while still there are 56 codepoints reserved for future usage.

To support the Msg3 indication on support of Msg4 PDSCH repetition, it seems simply to use the same mechanism by using the reserved LCID codepoints via Msg3.

Proposal 1: Use the LCID codepoint within the Rel-18 extension space in Msg3 to indicate the UE’s capability to support Msg4 PDSCH repetition.

TS38.321 Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 to 63 | (216 + 328) to (216 + 383) | Reserved |

The support of Msg4 PDSCH repetition is an addition on top of legacy UE capabilities which can be indicated via LCID in Msg3. Therefore, the design of the LCID for Msg4 PDSCH repetition should consider the cobmination with legacy features, including the CCCH size, RepCap UE, eRedCap UE and the PUCCH repetiton of Msg4 HARQ-Ack.

Proposal 2: The LCIDs used to indicate Msg4 PDSCH repetition capability shall be defined to account for combinations with legacy UE capabilities signaled via Msg3 LCIDs, such as CCCH size, RepCap UE, eRedCap UE, and PUCCH repetition for Msg4 HARQ-ACK.

In total, 12 LCIDs should be used to indicate the Msg4 PDSCH repetition capability and its combination with other UE capabilities. We have provided a TP for TS38.321 in Annex for further discussion.

Furthermore, it worth to discuss whether explicit indication is needed to control UE behavior to report capability for Msg4 PDSCH repetition. It has been noticed that RAN1 has agreed that the Msg4 PDSCH aggregation factor is configured in SIB1, with possible value 2 or 4. When the aggregation factor is configured in SIB1, the PDSCH Msg4 repetition is enabled. Based on above, there is no need to have explicit network indication to enable/disable Msg4 PDSCH repetition besides the needed signalling for PDSCH repetition aggregation factor in SIB.

Proposal 3: No explicit network indication to enable/disable Msg4 PDSCH repetition besides the needed signalling for Msg4 aggregation factor in SIB1.

Additionally, there is an FFS in the RAN1 LS whether to specify condition(s) under which the UE reports its capability or request. As summarized in R1-2503692, RAN1 discussed whether the network should configure a specific RSRP threshold, such that the UE would report its Msg4 PDSCH repetition capability only when the measured RSRP falls below this threshold. However, no consensus was reached on this proposal. In our understanding, there would not gain much from UE having RSRP threshold configured – if network configures Msg4 PDSCH repetitions it is with a purpose to enhance the Msg4 coverage, and hence we would generally just like to know if UE supports the feature or not. Since the Msg4 PDSCH repetition feature was introduced and is being discussed in RAN1, from a RAN2 perspective, it is appropriate to await further progress from RAN1 before proceeding.

Proposal 4: An RSRP threshold shall not be introduced as a condition for reporting the Msg4 PDSCH repetition capability. RAN2 may revisit this conclusion if further progress is made in RAN1.

Proposal 5: RAN2 to discuss the TP for TS38.321 in Annex.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Use the LCID codepoint within the Rel-18 extension space in Msg3 to indicate the UE’s capability to support Msg4 PDSCH repetition.

Proposal 2: The LCIDs used to indicate Msg4 PDSCH repetition capability shall be defined to account for combinations with legacy UE capabilities signaled via Msg3 LCIDs, such as CCCH size, RepCap UE, eRedCap UE, and PUCCH repetition for Msg4 HARQ-ACK.

Proposal 3: No explicit network indication to enable/disable Msg4 PDSCH repetition besides the needed signalling for Msg4 aggregation factor in SIB1.

Proposal 4: An RSRP threshold shall not be introduced as a condition for reporting the Msg4 PDSCH repetition capability. RAN2 may revisit this conclusion if further progress is made in RAN1.

Proposal 5: RAN2 to discuss the TP for TS38.321 in Annex.

# Text Proposal for TS38.321

TS38.321 Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 | (216 + 328) | CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE |
| 9 | (216 + 329) | CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE |
| 10 | (216 + 330) | CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE |
| 11 | (216 + 331) | CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE |
| 12 | (216 + 332) | CCCH of size 48 bits for Msg4 PDSCH repetition of an eRedCap UE |
| 13 | (216 + 333) | CCCH of size 64 bits for Msg4 PDSCH repetition of an eRedCap UE |
| 14 | (216 + 334) | CCCH of size 48 bits for Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 15 | (216 + 335) | CCCH of size 64 bits for Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 16 | (216 + 336) | CCCH of size 48 bits for Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 17 | (216 + 337) | CCCH of size 64 bits for Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 18 | (216 + 338) | CCCH of size 48 bits for Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 19 | (216 + 339) | CCCH of size 64 bits for Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 20 to 63 | (216 + 340) to (216 + 383) | Reserved |