3GPP TSG-RAN WG2 Meeting #131		         R2-2505644

Bengaluru, India, 25th - 29thAugust 2025

Agenda Item:	8.5.3

Source: 	ITRI, Acer Incorporated

Title:  	Discussion on the remaining open issues for OD-SIB1

Document for:	Discussion

Introduction

In this contribution, we would like to discuss and provide our views on the remaining on-demand SIB1 issues.

Discussion

During the last RAN2 meeting, it was agreed that “Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.”

Following this, in the long email discussion “[POST130][107][NES] Running RRC CR”, the following sentence was added to section 5.2.2.3.5: “2> a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one.” The remaining issue is whether this UE behavior should also be captured in the file description of si-BroadcastStatus.

Since the si-BroadcastStatus field’s primary purpose is to indicate whether a System Information (SI) message is currently being broadcast. Embedding UE behavior (e.g., interpreting stored SIB1 as latest) into this field’s description creates several issues:

- Specification Inconsistency – The UE behavior related to OD-SIB1 is a specific function. The field description of si-BroadcastStatus is a foundational element that should apply broadly. Placing specific, conditional behavior within a general field description can lead to inconsistencies and make it challenging to maintain the standard as new features are added or modified.

- Unnecessary Redundancy – The statement relates to UE procedures in RRC_CONNECTED, which is already defined in the 5.2.2.3.5 section. Including the same information in the field description of si-BroadcastStatus is redundant.

Therefore, we think it may not be necessary to include the OD-SIB1 connected mode UE behaviour in the file description of si-BroadcastStatus.

Proposal 1:

The file description of si-BroadcastStatus does not need to include the OD-SIB1 connected mode UE behavior.

In the proposed RRC CR for NES, it requires UE supporting OD-SIB1 shall ensure having a valid version of SIBxx, as following:

“The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE is capable of slice-based cell reselection and the UE receives NSAG information for cell reselection from upper layer), SIB17 or SIB17bis (if the UE is using TRS resources for power saving in RRC_IDLE and RRC_INACTIVE), SIB19 (if UE is accessing NR via NTN access), SIB22 (for ATG access), and SIB23 (if UE is capable of NR sidelink positioning and is configured by upper layers to receive or transmit SL-PRS). The UE supporting OD-SIB1 shall ensure having a valid version of SIBxx.”

At the RAN2 #129 meeting, it was agreed that the UE shall follow the legacy validity principle for stored SIBs. According to the current validity principle, the validity of a stored SIB depends on whether the UE receives an SI change indication from the paging DCI.

For example, if the UE receives an SI change indication from the paging DCI during modification period #1, it considers the stored SIB valid only until the boundary of modification period #1, as illustrated in Figure 1. The UE will then initiate the SI acquisition procedure in the subsequent modification period (i.e., modification period #2).

Figure 1

Conversely, if the UE does not receive an SI change indication from the paging DCI within modification period #1, it considers the stored SIB valid until the boundary of modification period #2, as shown in Figure 2. In this case, the UE expects to monitor for the SI change indication in the next modification period (i.e., modification period #2).

Figure 2

In current spec, the behavior of a connected UE receving SI change indication will be:

“UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.”

According to the specification, a UE in connected mode shall monitor for SI change indications in any paging occasion. In other words, the current specification does not restrict when, within a modification period, the UE should monitor for an SI change indication. Considering that Radio Link Failure (RLF) can occur unpredictably, it is possible that an RLF occurs before the UE has monitored for an SI change indication during the current modification period. Consequently, the UE may not be able to determine whether the stored SIBxx is still valid when it selects a NES cell for connection reestablishment after the current modification period.

For instance, if RLF occurs during modification period #1 and the UE has not monitored for an SI change indication within that period, then upon selecting a NES cell after modification period #1, as illustrated in Figure 3, the UE is unable to verify the validity of the stored SIBxx.

Figure 3

If the UE cannot verify whether the stored SIBxx is valid, the safest approach is to consider the SIBxx invalid. However, this may increase the latency of the connection reestablishment procedure. This is because, if SIBxx is deemed invalid, the UE cannot acquire OD-SIB1 and may need to select another cell.

An alternative approach is to require the UE to monitor for an SI change indication before the occurrence of RLF—for example, during T310—if the UE has not yet monitored for an SI change indication in the current modification period. This approach ensures that the UE is able to verify the validity of the stored SIBxx when selecting a cell for the connection reestablishment procedure.

Proposal 2:

A connected mode NES capable UE should monitor for SI change indication during T310 if the UE has not yet monitored for SI change indication within the modification period.

Conclusion

Proposal 1: The file description of si-BroadcastStatus does not need to include the OD-SIB1 connected mode UE behavior.

Proposal 2: A connected mode NES capable UE should monitor for SI change indication during T310 if the UE has not yet monitored for SI change indication within the modification period.