3GPP TSG-RAN2#131	R2-2505654

Bengaluru, India, August 25-29th, 2025

Agenda Item:	8.3.2.3

Source: 	Sony

Title:	UE side Data collection for measurement/ event prediction

Document for:	Discussion and Decision

# Introduction

Agreements for AIML MOB regarding data collection:

In RAN2#129bis:

2 For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details 3 Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP. 4 As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

In RAN2#130:

For the collection of data for the training of a network-sided model for RRM measurement prediction: a) UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) b) The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. g) The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling) h) The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). i) The UE releases the collected data upon transitioning to IDLE/INACTIVE For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

In this contribution we provide our views on UE side data collection for measurement and event prediction.

# Discussion

UE performs measurements based on measurement object configuration, evaluate different events, and report measurement results. Measurement model captured in 38.300 and shown in figure below shows different steps involved during measurements and evaluation of an event:

Figure 1: Measurement model as described in 38.300.

UE performs measurements on different beams and both L1 and L3 filtering is applied to these measurement samples and a trigger is evaluated based on these consolidated cell level results. The number of good beams is also reported along with measurement results. UE is configured to perform measurements based on either SSB or CSI-RS. Further, a list of cell specific offsets, a list of exclude-listed cells and a list of allow-listed cells are provided to the UE.

For measurements or event prediction, in our understanding, UE will collect measurement samples for a UE side model and based on these samples will predict measurements or event. UE may collect these measurement samples while it was in connected mode. But in some implementations, measurement samples from idle/inactive mode may be used as well.

Observation 1: UE will use measurement samples predominantly while in RRC_Connected mode. But some implementations might use measurements collected in RRC_Idle/Inactive state.

If observation 1 is agreeable, then there are certain parameters which are configured differently when UE is performing measurements in idle/inactive mode and connected mode. For example, absThreshSS-BlocksConsolidation, nrofSS-BlocksToAverage, offsetMO, allowed and exclude cell list are configured in connected mode. Similarly, some of these parameters are indicated in SIB2 for cell reselection in idle/inactive mode. In addition to above parameters, UE may be configured with relaxed measurements in idle/inactive mode.

Idle mode cell coverage:

UE may be configured to stay for longer in a cell before it moves out or start performing measurements and try to save as much of power as possible. So, the thresholds at which UE starts performing measurements and at which the UE perform cell reselection, are normally on the higher side in order to save UE power consumption.

Connected mode cell coverage:

Handover is carried out when UE is in RRC_Connected mode and active user data transmission taking place. So, network sets rather conservative thresholds for handover compared to cell reselection. Handover takes place when both source and target cells are providing good coverage. This is not needed in idle mode and UE may stay in idle/inactive camped cell for longer. Also, idle/inactive measurements are performed on SSB whereas connected mode measurements may be configured either based on SSB or CSI-RS. One such idle/inactive and connected mode cell coverage example is shown in figure below with red lines representing coverage of the same cell in connected mode for handover purposes:

Figure 2: different coverage boundaries of same cell for idle and connected mode

Model training, Model inference, federated learning, model monitoring, model parameters update, model selection switching, and selection depends on the quality of input data. If UE does not distinguish measurement samples collected when it was in idle or connected mode or which set of parameters did it use to collect measurement samples, AI/ML model for mobility may not work well. So far, RAN2 has agreed measurement samples being collected at regular intervals like MDT measurements. Some of these samples might be based on actual measurements for mobility. Any inference based on these samples will have an effect on the model performance.

We think that event prediction will be impacted by cell specific offset, number of beams being taken into account, hysteresis parameters. measurement samples during idle/inactive state should be allowed for accuracy of measurements and allow different implementations. But network should be aware if samples from idle/inactive mode were used.

Proposal: UE is allowed to collect measurement samples in idle/inactive state as well. If UE has used measurements from idle/inactive state for UE side model, then network should be aware.

# Summary

In this contribution, we discuss measurement event prediction and propose RAN2 to agree:

Proposal: UE is allowed to collect measurement samples in idle/inactive state as well. If UE has used measurements from idle/inactive state for UE side model, then network should be aware.

# References

- 3GPP TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-242393: Revised SID on AIML for mobility in NR